3GPP TSG-RAN WG2 Meeting #130	R2-2504116

St Julian's, Malta, 19th – 23rd May, 2025

Agenda item:	8.16

Source: 	Huawei, HiSilicon

Title: 	Discussion on the support of NavIC L1 SPS

Document for:	Discussion and Decision

1. Introduction

In this paper, we discuss on the potential issues on the support of NavIC L1 SPS [1] in NR and LTE system.

2. Discussion

With the previous RANP decision, we are going to support NavIC L1 SPS signal in NR and LTE in R19 [2]. The main task of standardization in 3GPP will be to deliver the contents of the navigation message for the NavIC L1 SPS signal in the 3GPP system, either by dedicated signalling with LPP, or by broadcast signalling by posSIB.

For the support of content delivery by LPP, since the spec is 37 series inter-RAT spec between E-UTRA and NR, introduction of the support of NavIC L1 SPS in the LPP naturally introduces the support for E-UTRA and NR simultaneously. Similarly for broadcast, the information elements within the posSIB are also defined in the LPP and LPP already defined a framework well enough that the new GNSS signal can be easily added to the posSIB framework without the need to add new posSIBs.

Hence, for the support of the new NavIC signal, all we need to do is to examine the LPP spec for the NavIC specific information elements that are not currently supported for L1 SPS from the ICD document. For the description of the NavIC L1, the following description has been given in [2].

India has now introduced second generation of NavIC satellite services. Compared to the first-generation satellite series, the second-generation satellite series includes L1 navigation band and incorporates indigenously developed rubidium atomic lock. The inclusion of L1 navigation band improves the positioning, navigation, and timing services for civilian users, and facilitates better interoperability with other GNSS services. The NavIC Signal-in-Space ICD (ISRO-IRNSS-ICD-SPS-L1-1.0) for SPS in L1 Frequency was released to the public in August 2023 for providing the essential information on the NavIC signal-in-space, to facilitate research & development and aid the commercial use of the NavIC signals for navigation-based applications. NavIC L1 signal is optimized to enhance overall system performance on NavIC SPS which included introduction of a new ionospheric model, update to Ephemeris representation, update to transformation of TOWC among others.

While creating a new clock/navigation model and ionospheric model, the grid model and differential corrections for the enhancement of NAVIC L1 SPS mainly reuses the previous introduce field navic-DifferentialCorrections and navic-GridModel. In the previous release, we have introduced similar field for BDS in R12. While by examining the two fields, we find that the content of the fields in different release are more or less the same. For the field of differential corrections, both GNSS include reference time and signal type list; for grid model, both fields include reference time and grid ionospheric information. Thus, we think that we could create a generic IE for regional enhancement corrections that the IE can also be used in the future when another regional correction is introduced.

Based on the discussion above we propose the following:

Proposal: Consider to create a common IE for differential corrections and grid model for GNSS regional enhancements.

3. Conclusion

In this paper, we discuss the support of L1 SPS in LPP, we propose the following

Proposal: Consider to create a common IE for differential corrections and grid model for GNSS regional enhancements.

4. References

[1]	NavIC SPS L1 ICD, https://www.isro.gov.in/media_isro/pdf/SateliteNavigation/NavIC_SPS_ICD_L1_final.pdf

[2] RP-242414, Introduction of A-GNSS support for NavIC L1 SPS in NR & LTE