3GPP TSG-RAN WG2 Meeting #130	R2-2504123

St Julian’s, Malta, May 19 – 23, 2025

Agenda Item:	7.0.2.22

Source:	Ericsson

Title:	Clarification of pdcch-RACH-AffectedBandsList-r18 and related capabilities

Document for:	Discussion, Decision

# 1	Introduction

An LS from RAN4 (R4-2504940) has provided further input on how to interpret pdcch-RACH-AffectedBandsList-r18. This contribution discusses the updates for pdcch-RACH-AffectedBandsList-r18 and related capabilities that also require update.

# 2	Discussion

## 2.1	Updates for pdcch-RACH-AffectedBandsList-r18

According to the feedback from RAN4, interruption/no interruption must be indicated for each serving cell and whether interruption will occur or not depends on the target band. It is also noted that both pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 are clarified to be for the case of PDCCH ordered RACH from which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells – this same clarification is missing in pdcch-RACH-AffectedBandsList-r18.

Hence, it is not clearly stated in the field description of pdcch-RACH-AffectedBandsList-r18 that:

- the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission;

- the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission;

- PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells.

Those aspects should be clarified in the field description of pdcch-RACH-AffectedBandsList-r18 in 38.306. A possible update is provided below:

pdcch-RACH-AffectedBandsList-r18

Indicates whether UE may cause interruption may occur on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission towards target bands where the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells.

Each "source-target" pair indicates the band pair between the entry in the list corresponds to a target band for RACH transmission and band under UE's current band combination.

The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on.

A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18.

pdcch-RACH-AffectedBandsList-r18 field description is updated to clarify that:

- the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission;

- the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission;

- PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells.

Moreover, pdcch-RACH-AffectedBandsList is a misleading name since this is a list of target bands rather than affected bands, so the field name should be updated in 38.331 from pdcch-RACH-AffectedBandsList to e.g. pdcch-RACH-TargetBandsList.

pdcch-RACH-AffectedBandsList field name is updated to pdcch-RACH-TargetBandsList.

## 2.2	Updates for pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18

The fields pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 both have similar description that generated the discussion for pdcch-RACH-AffectedBandsList i.e. “Each "source-target" pair indicates the band pair between the target band for RACH transmission and band under UE's current band combination.”. Hence, similar updates as done for pdcch-RACH-AffectedBandsList should also be done for pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18.

pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 field descriptions are updated to clarify that:

- the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission;

- the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission;

In addition, both field description state “If absent, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP.” But it is not clear how to interpret this information related to the case where the UE sets one entry in the list to the value notSuppoted. In our understanding, the current description about absence is related to the case where the entire field is absent, while setting an entry to notSupported means that UE does not support the feature for the target band. This should also be clarified.

pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 field descriptions are updated to clarify that when an entry in the list is set to notSupported, the UE does not support the feature for that target band. If the entire field is missing, the UE does not support the feature in any target band.

Lastly, if the proposal above can be clarified, it also implies that the UE shall set consistently the bits in pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 e.g. it would be ambiguous to report pdcch-RACH-SwitchingTimeList-r18 set to 1 ms for a certain band while setting pdcch-RACH-PrepTimeList-r18 notSupported for the same band.

The capability values for pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 must be set consistently.

A possible way to implement the changes described in this section is provided below.

pdcch-RACH-PrepTimeList-r18 Indicates the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells. If this field is absent, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in any target band. Each "source-target" pair indicates the band pair between the entry in the list corresponds to a target band for RACH transmission and band under UE's current band combination. If an entry is set to notSupported, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in that target band. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 and pdcch-RACH-SwitchingTimeList-r18. The UE must set the capability values consistently with pdcch-RACH-SwitchingTimeList-r18.

pdcch-RACH-SwitchingTimeList-r18 Indicates the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells, if absent, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP. Each "source-target" pair indicates the band pair between the entry in the list corresponds to a target band for RACH transmission and band under UE's current band combination. If an entry is set to notSupported, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in that target band. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 and pdcch-RACH-PrepTimeList-r18. The UE must set the capability values consistently with pdcch-RACH-PrepTimeList-r18.

The proposed changes has been also provided in the CR in Annex. Therefore, we propose:

RAN2 to agree on the CR provided in Annex.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	pdcch-RACH-AffectedBandsList-r18 field description is updated to clarify that:

	the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission;

	the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission;

	PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells.

Proposal 2	pdcch-RACH-AffectedBandsList field name is updated to pdcch-RACH-TargetBandsList.

Proposal 3	pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 field descriptions are updated to clarify that:

	the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission;

	the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission;

Proposal 4	pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 field descriptions are updated to clarify that when an entry in the list is set to notSupported, the UE does not support the feature for that target band. If the entire field is missing, the UE does not support the feature in any target band.

Proposal 5	The capability values for pdcch-RACH-PrepTimeList-r18 and pdcch-RACH-SwitchingTimeList-r18 must be set consistently.

Proposal 6	RAN2 to agree on the CR provided in Annex.

# Annex (CR to TS 38.306)

3GPP TSG-WG2 Meeting #130	R2-250xxxx

# St Julian’s, Malta, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of pdcch-RACH-AffectedBandsList-r18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An LS from RAN4 (R4-2504940) has provided further input on how to interpret pdcch-RACH-AffectedBandsList-r18. According to the feedback from RAN4, interruption/no interruption must be indicated for each serving cell and whether interruption will occur or not depends on the target band. It is not clearly stated in the field description of pdcch-RACH-AffectedBandsList-r18 that: - the feature set entry where the UE includes this capability corresponds to the serving cell that may be impacted during PDCCH-ordered RACH transmission; - the feature set entry includes a list where each entry in the list corresponds to a target band for RACH transmission. Those aspects should be clarified in the field description of pdcch-RACH-AffectedBandsList-r18. Impact analysis Impacted 5G architecture options: Standalone, NR-DC Impacted functionality: pdcch-RACH-AffectedBandsList Inter-operability: If the network implements the CR and the UE does not, the network may interpret differently than the UE report where interruption may occur for PDCCH-ordered RACH transmission. If the UE implements the CR and the network does not, the network may interpret differently than the UE report where interruption may occur for PDCCH-ordered RACH transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify how pdcch-RACH-AffectedBandsList should be interpreted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will remain unclear on how to interpret the UE capability for pdcch-RACH-AffectedBandsList. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

| pdcch-MonitoringMixed-r16 Indicates support of Rel-15 monitoring capability and pdcch-Monitoring-r16 on different serving cells. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| pdcch-MonitoringMixed-r18 Indicates whether the UE support Rel-15 monitoring capability and pdcch-Monitoring-r16 monitoring capability on different serving cells. The UE supporting this feature shall also indicate support of pdcch-Monitoring-r16 for (7,3) or (4,3) span based PDCCH monitoring. The UE supporting this feature shall also indicate support of pdcch-MonitoringSpan2-2-r18 for (2, 2) span based PDCCH monitoring with additional restriction(s). When a UE reports both pdcch-MonitoringMixed-r16 and this capability, the value reported in this capability is used if the configured span pattern of any serving cell satisfies pdcch-MonitoringSpan2-2-r18. | FS | No | N/A | N/A |
| pdcch-MonitoringSpan2-2-r18 Indicates support of (2, 2) span-based PDCCH monitoring with the additional restriction that there is at least one OFDM symbol gap between two PDCCH monitoring occasions. When a UE reports both pdcch-Monitoring-r16 and this capability, the union of supported span patterns in pdcch-Monitoring-r16 and this capability establishes the multiple combinations (X,Y) used to determine per-span BD/CCE limit as described in Clause 10 of TS 38.213 [11]. | FS | No | N/A | N/A |
| pdcch-RACH-AffectedBandsList-r18 Indicates whether interruption may occur on DL slot(s) on this serving cell due to PDCCH-ordered RACH transmission towards target bands where the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells. Each entry in the list corresponds to a target band for RACH transmission. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. | FS | No | N/A | N/A |
| pdcch-RACH-PrepTimeList-r18 Indicates the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells. If this field is absent, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in any target band. Each entry in the list corresponds to a the target band for RACH transmission. If an entry is set to notSupported, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in that target band. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 and pdcch-RACH-SwitchingTimeList-r18. The UE must set the capability values consistently with pdcch-RACH-SwitchingTimeList-r18. | FS | No | N/A | N/A |
| pdcch-RACH-SwitchingTimeList-r18 Indicates the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells, if absent, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP. Each entry in the list corresponds to a target band for RACH transmission and band under UE's current band combination. If an entry is set to notSupported, the UE does not support PDCCH ordered RACH if the PRACH bandwidth is outside of any configured UL BWP in that target band. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 and pdcch-RACH-PrepTimeList-r18. The UE must set the capability values consistently with pdcch-RACH-PrepTimeList-r18. | FS | No | N/A | N/A |

END OF CHANGE