3GPP TSG-RAN WG2 Meeting #130	R2-2504138

St Julians, Malta, 19 – 23 May 2025

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN impacts for IoT-NTN SF Operations

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following are the agreements in RAN2 on the SF operation for IoT-NTN in RAN2-128 and RAN2-129.

RAN2-128 Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable.

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

RAN2-129 Agreements: 1. RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change 2. The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) 3. Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). 4. A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

RAN2-129bis Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In this paper we analyse the remaining open issues for SF operation related to mode transition and access /paging procedure impacts in SF operation.

# 2	Discussion

## 2.1 Remaining Issues related to SF Operation indication

Issues related to Transition from Normal mode to SF mode

RAN2 agreed to introduce indication in system information for transition from normal mode to SF mode for NAS usage with further details marked FFS. When the SF mode transition from normal to SF the system information needs to be updated for legacy access barring bits to barred if needed and also to introduce the ‘SF-operation-mode’ in SIB1. This system information modification, however, requires system information notification via paging.  The SF capable UE can also know the transition based on the same. No additional indication is needed in this regard.

Proposal 1: RAN2 to confirm that existing system information change notification can be reused to indicate the transition from normal to SF mode.

Whether additional information on the remaining time in normal mode or the absolute time of transition is required was discussed in RAN2. In our view this information may be beneficial by UE applications if it intend to complete data transmission within the normal mode duration. However, such enhancements are not essential, and the UE can also decide on the pending application data based on actual events also. Alternatively, the UE can also estimate this information internally based on its earlier mode transition for the given satellite.

Observation 1: The remaining time information for normal to SF mode may be useful for applications on delaying some transactions based on this information. The actual usage scenario where this information is beneficial is not clear.

In case if the benefits of transition time information for applications is established, we propose to re-use the same parameter used for SF mode to normal for such information. It is natural to re-use the parameter that has similar meaning on the remaining time of current mode operation here.

Proposal 2: The transition time information introduced for SF mode to Normal is also re-used for Normal- SF mode transition.

Cell reselection changes related to SF mode operation

For SF operation SA2 is discussing multiple options to provide assistance information towards UE from the MME on board that allows the UE to know when and which satellites can be considered for resuming the NAS operations or MO data transmissions started in the current satellites.  This information is only related to UE operations for avoiding any pending actions in satellites which may not have context for the UE. However, the new registrations and new MO can still continue in other satellites.  Moreover, SA2 has already clarified that the NAS Satellite list does not restrict the UE access to other satellites in anyway.  Hence, we propose to confirm that access layer procedures are not impacted by the NAS provided satellite information or wait timer information. If any AS impacts are foreseen, we expect LS from SA2 on the same and RAN2 can act accordingly based on the LS.

Proposal 3: RAN2 to confirm that cell selection /reselection is not impacted by the satellite list provided from NAS. RAN2 to decide on the changes if any based on SA2 LS indicating RAN2 impacts based on the NAS provided information.

When SF mode parameter is configured in the IoT-NTN cell, it is up to UE implementation to stay in the current cell or start reselection for neighbouring cell (either TN or normal cell) based on the priority of the application data. This UE behaviour needs not be captured in RAN2 specification. In the similar lines introduction of parameters in neighbour-cell configuration on whether neigbour cell is operating in SF mode or not is not needed.

Proposal 4: Cell reselection when SF mode is active in current cell is up to UE implementation. No specification changes needed to enhance the cell reselection procedure for SF mode operation.

## 2.2 Access Control and Paging Changes for SF operation

When NTN node is serving new area, it may need to handle the new data transmissions from the coverage area and also complete the data transmission in downlink that was received and stored from CN at the time of earlier connectivity. In such cases the access control and data transmission across these two variants of users need to be differentiated. Mainly to avoid the delivering the stored data becoming invalid or time-out at UE.  In the same way for the uplink transmissions depending on the storage available at NTN node to receive and buffer the packets the access control and scheduling of transmissions needs to be handled. This can be achieved by explicit indication of support for MO or MT traffic along with SF operation parameter in system information.

Proposal 5:  RAN2 to consider inclusion of additional information for access control for MO or MT based on storage status in SF operation.

When the mobile originated IoT data transmission that requires application level ack are sent to eNB operating in store and forward mode the UE may expect the corresponding downlink packets after connecting to the next eNB. In this case UE monitors the next gNB for receiving ACK and new MT packets if any stored in the eNB. The data packet available at this eNB may contain ACK for earlier MO traffic and MT traffic toward the UE. In some other cases, the next eNB may not have any downlink ACK for earlier transmission depending on its CN connection time. In different scenarios based on the network awareness of CN connectivity time and the actual stored downlink data to be delivered towards UE, it is possible to improve the paging monitoring towards UE. With additional indication on whether ENB have stored ACK for MO traffic can also be used by UE application to avoid early retransmissions of MO data.

Proposal 6:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

In addition to paging enhancements, it is also beneficial for the UE battery life to consider that a satellite may not have more MT data for the UE. The typical case is that if the feeder link is not available, the satellite has no more MT data for the UE if the satellite buffer is drained since the satellite will not receive further MT data from CN. Until the feeder link recovery, the UE will not benefit from continuing paging monitoring on the current cell, nor will it benefit from reselecting to and monitoring paging from other cells of the same satellite. Thus, RAN2 can consider whether Idle mode procedures can be enhanced when there is no more MT data to be delivered in S&F operation.

Proposal 7: RAN2 to consider indication of MT data unavailability in storage for paging monitoring enhancements for SF capable UE.

# 3	Conclusion

In this paper we analyse the radio signalling procedure impacts for store and forward operation. Following proposals are

made based on the analysis.

Issues for normal mode to SF mode transition

Proposal 1: RAN2 to confirm that existing system information change notification can be reused to indicate the transition from normal to SF mode.

Observation 1: The remaining time information for normal to SF mode may be useful for applications on delaying some transactions based on this information. The actual usage scenario where this information is beneficial is not clear.

Proposal 2: The transition time information introduced for SF mode to Normal is also re-used for Normal- SF mode transition.

Issues for cell reselection

Proposal 3: RAN2 to confirm that cell selection /reselection is not impacted by the satellite list provided from NAS. RAN2 to decide on the changes if any based on SA2 LS indicating RAN2 impacts based on the NAS provided information.

Proposal 4: Cell reselection when SF mode is active in current cell is up to UE implementation. No specification changes needed to enhance the cell reselection procedure for SF mode operation.

Access control and Paging Enhancements for SF operation

Proposal 5:  RAN2 to consider inclusion of additional information for access control for MO or MT based on storage status in SF operation.

Proposal 6:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

Proposal 7: RAN2 to consider indication of MT data unavailability in storage for paging monitoring enhancements for SF capable UE.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements