3GPP TSG-RAN WG2 Meeting #130	R2-2504139

St Julians, Malta, 19 – 23 May 2025

Agenda item:	7.0.2.23

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN2 Impacts for NR NB-IoT-NTN

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2 has received an LS [1] from RAN4 about the introduction of inband operation for NB-IoT NTN and the potential RAN2 impact. RAN2 sent LS to RAN4 confirming the feasibility of RAN4 assumptions in [2]. In this discussion paper we further discuss the RAN2 impacts related to RAN4 assumptions.

# 2	Discussion

In Rel-17 the IoT-NTN operation was only supported for standalone mode. There was no specification changes introduced about this restriction in RAN2 specifications. For Rel-18 RAN4 has analysed the possibility of NB-IoT operation with NR inband for NTN operation in the above LS and confirmed that it is possible and the UE may need to support guard-band for NR inband operation to convey additional information related to raster offset.

In our view following information can be considered for stage-2 specification changes to reflect the RAN4 assumptions

- For IoT-NTN operation In-band operation is not supported.

- For IoT-NTN deployments only same operating mode for anchor and non-anchor carriers are supported (excluding in-band).

Proposal 1: RAN2 to consider NOTE for Table5.5a-1 to capture supported operating modes for IoT-NTN and also the allowed combination for multi-carrier operation

RAN4 LS also indicates the Inband operation with NR for NB-IoT-NTN is supported for Rel-18 UE.  It is not clear whether Rel-17 IoT-NTN UE consider camping to cell operating in guard-band mode. Hence some further clarification needed on whether additional capability is needed to differentiate the support for additional mode over the standalone mode operation.

Proposal 2: RAN2 to discuss whether UE capability without signalling is needed to indicate the UE support for new deployment mode.

# 3	Conclusion

In this discussion we analyse the RAN2 impacts for RAN4 assumption on support for NR-Inband operation for NB-IoT-NTN and make following proposals.

Proposal 1: RAN2 to consider NOTE for Table5.5a-1 to capture supported operating modes for IoT-NTN and also the combinations for multi-carrier operation.

Proposal 2: RAN2 to discuss whether UE capability without signalling is needed to indicate the UE support for new deployment mode.

# Reference

[1]	R4-2503035, LS on NR NB-IoT in-band operation, Ericsson, RAN4#114, February 2025.

[2]	R2-2503064, Reply LS on NR NB-IoT in-band operation, Ericsson, RAN2#129bis, April 2025.