3GPP TSG-RAN2 Meeting #130	R2-2504177

Malta, 19th-23rd May, 2025

Agenda item:	8.2.2

Title:	AIoT Paging: Handling a New Service Request

Source:	Philips International B.V.

Document for:	Discussion

# Introduction

RAN#106 approved WI for Ambient IoT in [1]. One of the objectives for RAN2 is listed as follows:

- Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

In RAN2#129 [2], the following agreements were made:

Agreements: 1. Parallel service requests by the same reader is not supported. 2. The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). 3. The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID 4. 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. 5. RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it.

To allow further discussion of the FFD items, RAN2#129 allocated an email discussion [3]:

[POST129][035][AIoT] Paging (Qualcomm) Intended outcome: Discuss and address the remaining paging FFSs considering the agreements this meeting: FFS device behaviour if multiple requests are received in parallel (if needed), FFS how reader will generate “transaction ID”, FFS the size of transaction ID. Deadline: Long

The report from this email discussion makes a number of proposals which were discussed during RAN2#129bis. Some proposals relating to the expected behaviour of the device when it receives a new service request while processing an earlier one are quoted below:

Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/8/4) Proposal 2: Discuss and decide how a device determines a procedure is ‘ongoing’ or not. Proposal 3: If option (a) is to be agreed in P1: Discuss and decide how a device determines whether a R2D message from Reader is associated with the ongoing procedure or not. Proposal 4: Discuss and decide whether the device needs to distinguish and behave differently between when the same service request is received again from the same reader vs a different reader (yes/no/ffs = 14/12/2). Proposal 5: Discuss and decide whether RAN2 needs to consider the case where the device has already successfully responded to the same service request but now received the same request from a different reader e.g. for location use case (yes/no/unclear = 9/16/3).

During RAN2#129bis [4[, an offline discussion [5] was held to determine whether to choose from proposal 1:

Option (a): Ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure

or:

Option (b): Terminate the ongoing procedure and respond to the latest request

No conclusion was reached and the decision was deferred to the present meeting. In this contribution we revisit the issue.

# Discussion

A key issue is that there is no clear mechanism by which a device can determine that a service request has completed. If it is addressed by a new service request, that is, it receives a new service request with a valid address, is it safe for the device to assume that the current service request has been terminated and to follow the new service request?

## Problems with determining that a service request has terminated

One aspect of a service request that emerged in Wuhan is that the command stage of a combined inventory + command service request is performed after the inventory stage has been completed.

Furthermore, in Athens, it was agreed that the service type of the service request i.e., inventory only, or inventory + command, is not indicated in the paging message. This means that a device cannot assume its role in a service request has completed once it has successfully taken part in an inventory stage: it must wait for an unspecified period in case it subsequently receives a command. Even after responding to the command, it cannot assume its role is over since another command may be sent by the reader.

The offline discussion in Wuhan [4] considered an explicit release message to indicate to a device that the ongoing service request procedure has finished but this was considered problematic for a number of reasons, one of which was that a device might not actually receive it.

Switching off the CW signal powering the device and, effectively, forcing a power-on reset at the device relies on the network having sufficient control over the CW signal (and an appropriate power-on reset implementation at the device). It is also something of a blunt instrumentation in that all devices in an area would be affected.

Conversely, one could also note that, in areas of overlapping coverage, it might not even be possible to remove all sources of CW energy from a device, meaning that it might not actually be possible to force a power-on reset.

A service timer was considered a potentially complex addition to a simple device and it is not clear what would be an appropriate value for a given service procedure.

All of this means that there is no clear mechanism that allows a device to determine that a service request has terminated. In this context, use of option (a) may result in lock-up because, if the device believes that a previous procedure is still running, it will ignore all new service requests and, without a clear means of resetting it, the device becomes unresponsive.

Observation 1: There is no clear mechanism that allows a device to determine that a service request has terminated meaning that use of option (a) has the potential to cause device lock-up

## Problems with responding to all new service requests

The difficulty with option (b) is that any paging message with a different service request ID that happens to address a device will capture the device. In environments with unco-ordinated readers operating simultaneously with areas of overlapping coverage, this could mean devices being captured and recaptured by successive paging messages from different readers, leading to unexpected and generally sub-optimal device behaviour.

Observation 2: Use of option (b) in environments with unco-ordinated readers operating simultaneously with areas of overlapping coverage, could lead to unexpected and generally sub-optimal device behaviour

## Device behaviour in controlled environments

From the above discussion, with the current state of the art, care must be taken to ensure that the device is not placed in ambiguous situations that could lead to unexpected and potentially undesirable behaviour.

### A controlled environment

In certain environments, like factories or warehouses, it might be possible to arrange the network installation so that the whole plant or, at least, well-defined areas can be covered by a single base station. If all readers in a coverage zone are associated with the same base station, their operation can be co-ordinated by the base station so that, at most, only one ongoing service request across all readers covering the same coverage zone.

Observation 3: In a controlled environment, there should only be one ongoing service request across all readers covering the same coverage zone

We can propose that the agreement from RAN2#129 that parallel service requests by the same reader are not supported be extended to say that parallel service requests by readers with overlapping coverage areas are not supported.

Proposal 1: RAN2 agrees that parallel service requests by readers with overlapping coverage areas are not supported.

A necessary further assumption is that any other readers that are simultaneously active with a local reader are sufficiently far away for their transmissions not to be detected by devices in the local area. Figure 2.3.1-1 provides an exemplary illustration.

Figure 2.3.1-1 – Co-existence assumptions for D1T1 AIoT

Some support for this assumption could be drawn from assumptions on co-existence modelling for the D1T1 case detailed in TR 38.769. According to section 6.6.2, “Co-existence evaluation assumptions,” readers are assumed to be placed on a square lattice with separation of 20 m between adjacent readers. By comparison, for active readers a minimum distance of 60 m is used as a base line.

One should use caution here, however, because assumptions used for coexistence modelling do not constitute a specification for deployment, nor do the base line distances assumed offer a guarantee of performance in a real world deployment. Nevertheless, the notion that, for coexistence in a controlled environment, there should be sone distance between active readers is consistent with the coexistence modelling assumptions in TR 38.769.

Observation 4: For coexistence in a controlled environment, there should be sone distance between active readers

In such controlled environments, the first indication for a device that the current service request has indeed been terminated may the arrival of a new service request from the same or a related reader belonging to a multi-reader group.

Observation 5: In controlled environments, the first indication for a device that the current service request has indeed been terminated may be the arrival of a new service request from the same or a related reader belonging to a multi-reader group

Some examples of this are described in the following sections.

### New service request from the same reader

Given the observations made in the previous section, when a device receives a new service request from the same reader, the most appropriate behaviour is for it to abandon the current service request, at whatever stage it has reached, and respond as required to the new service request, i.e., follow option (b).

Proposal 2: When a device operating in a controlled environment receives a new service request from the same reader, it may abandon the current service request and respond as required to the new service request

### New service request from a different reader

This can be extended easily to the multi-reader scenario when all readers are managed by the same base station. In this case, a new service request issued by a new reader implies that any ongoing service request is terminated. Therefore, the device’s expected behaviour should be the same.

Proposal 3: When a device receives a new service request from a different reader belonging to the same base station, it may abandon the current service request and respond as required to the new service request

### Same service request from a different reader

Proposals 4 and 5 of the email discussion ask about the expected device behaviour when the same service request is received from the same reader v. a different reader.

One reason that a base station might send the same service request via a different reader is to solicit a response from devices that are part of the group but are inaccessible to the first reader.

Observation 6: A base station might send the same service request via a different reader is to solicit a response from devices that are part of the group but are inaccessible to the first reader

In this case, it is an ongoing service request and there is no reason for a device to treat this any differently regardless of the fact that it comes from a different reader. Thus, if it has already responded to the service request, it need not respond to the second reader; if it has not responded to the first reader, it may respond to the second reader.

Proposal 4: A device receiving the same service request via a different reader shall treat it as an ongoing service request and respond as necessary

### Option (a) or option (b) in controlled environments?

From the above discussion, for operation in controlled environments as defined in the preceding sections, it is safe for a device that receives a new service request to assume that the current service request has terminated and for it to process the new service request. It is therefore proposed that, for this release, RAN2 should adopt option (b).

Proposal 5: For this release, a device that receives a new service may assume that the current service request has terminated and may process the new service request. Thus, for Rel-19, RAN2 should adopt option (b).

## Operation in uncontrolled environments

For Rel-20 A-IoT, extensions including Topology 2, use of device types 2b and 2c, DO-A operation and operation outdoors are under consideration. It may be the case that the assumptions for controlled environments discussed above do not hold for some Rel-20 deployments.

For Rel-20, then, it may be appropriate to allow support for option (a). This might require a reliable mechanism allowing a device to determine that a service request has terminated, e.g., a new message or message element. One might also consider a procedure timeout timer as one component of such a mechanism, serving at least as a backstop against signalling or other failures.

While the details of this discussion are out of scope for now, RAN2 should nevertheless allow for future enhancements to the signalling and device handling defined in Rel-19 to allow for operational robustness in future, uncontrolled environments that cannot meet coexistence assumptions. This may include support for Option (a).

Proposal 6: RAN2 should allow for future enhancements to the signalling and device handling defined in Rel-19  to allow for operational robustness in future, uncontrolled environments, including support of option (a)

# Conclusion

In this paper, we make the following observations:

Observation 1: There is no clear mechanism that allows a device to determine that a service request has terminated meaning that use of option (a) has the potential to cause device lock-up

Observation 2: Use of option (b) in environments with unco-ordinated readers operating simultaneously with areas of overlapping coverage, could lead to unexpected and generally sub-optimal device behaviour

Observation 3: In a controlled environment, there should only be one ongoing service request across all readers covering the same coverage zone

Observation 4: For coexistence in a controlled environment, there should be sone distance between active readers

Observation 5: In controlled environments, the first indication for a device that the current service request has indeed been terminated may be the arrival of a new service request from the same or a related reader belonging to a multi-reader group

Observation 6: A base station might send the same service request via a different reader is to solicit a response from devices that are part of the group but are inaccessible to the first reader

From these observations, we derive the following proposals:

Proposal 1: RAN2 agrees that parallel service requests by readers with overlapping coverage areas are not supported.

Proposal 2: When a device operating in a controlled environment receives a new service request from the same reader, it may abandon the current service request and respond as required to the new service request

Proposal 3: When a device receives a new service request from a different reader belonging to the same base station, it may abandon the current service request and respond as required to the new service request

Proposal 4: A device receiving the same service request via a different reader shall treat it as an ongoing service request and respond as necessary.

Proposal 5: For this release, a device that receives a new service may assume that the current service request has terminated and may process the new service request. Thus, for Rel-19, RAN2 should adopt option (b).

Proposal 6: RAN2 should allow for future enhancements to the signalling and device handling defined in Rel-19  to allow for operational robustness in future, uncontrolled environments, including support of option (a)

# References

[1]	RP-243326, Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN#106, Dec 2024

[2]	R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17 - 21 February, 2025

[3] 	R2-2502211	Email discussion report: [POST129][035][AIoT] Paging	Qualcomm Incorporated

[4] 	RAN2#129bis draft ChairNotes, 25-04-10_18-00

[5[	R2-2503155	Offline discussion report: [AT129bis][010][AIoT] Paging	Qualcomm Incorporated