3GPP RAN WG2 Meeting #130	R2-2504178

St Julian’s, Malta, May 19th – 23rd 2025

Agenda Item:	8.8.2

Source:	InterDigital

Title:	Downlink coverage enhancement for NTN

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to study DL coverage impacts and modifications to support dynamic power sharing among satellite beams. Enhancements can be considered at both the link level (e.g., improving the link margin of selected physical channels) and system level (e.g., supporting dynamic and flexible power sharing between beams). Detailed objectives include the following [1]:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

This contribution discusses Msg4 PDSCH repetition feature for DL-CE.

# Discussion

In RAN1#120bis meeting, the following agreement was made:

Msg-4 PDSCH Repetition:

1.	Msg-4 PDSCH Repetition: Enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: 
Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 
Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. 
Option 3: The enabling/disabling is indicated by SIB1 configuration

For Options 2 and 3, they won’t require any detailed discussion in RAN2.

But if RAN1 go for Option 1, then there is some RAN2 impact and this clause clarifies the potential RAN2 impact of Option 1.

For Option 1, the network needs to identify which UE requires the Msg4 PDSCH repetition. Thus, either Msg1 or Msg3 needs to signal that the UE expects the PDSCH repletion for Msg4 reception. There are number of features relying on the RACH partitioning to signal the capabilities and so it’s better to go for the Msg3 signalling option to signal the fact that the UE requires (and supports) the Msg4 PDSCH repetition.

Observation 1: For Option 1, Msg3 based signalling to request Msg4 PDSCH repletion would make sense and a special LCID can be defined for that signalling.

In addition, for Option 1, the UE should request the Msg4 PDSCH repetition only when necessary (i.e. the radio condition is not good). Thus, the network should provide an RSRP threshold so that a UE can request the Msg4 PDSCH repetition only when the measured RSRP value is less than the threshold.

Observation 2: For Option 1, an RSRP threshold should be provided by gNB so that only UEs in bad radio condition can request the Msg4 PDSCH repetition.

According to the above observations, we propose;

Proposal 1: For UE specific PDSCH with Msg4 repetition activation, a special LCID value is defined to request the Msg4 PDSCH repetition by Msg3.

Proposal 2: For UE specific PDSCH with Msg4 repetition activation, an RSRP threshold is defined in SIB1 and a UE can request the Msg4 PDSCH repetition only when the measured RSRP is less than the RSRP threshold.

# Conclusion

In this contribution the following observations and proposals are made regarding DL coverage enhancements:

Proposal 1: For UE specific PDSCH with Msg4 repetition activation, a special LCID value is defined to request the Msg4 PDSCH repetition by Msg3.

Proposal 2: For UE specific PDSCH with Msg4 repetition activation, an RSRP threshold is defined in SIB1 and a UE can request the Msg4 PDSCH repetition only when the measured RSRP is less than the RSRP threshold.

# References

RP-243300 – Non-Terrestrial Networks (NTN) for NR Phase 3 – Thales, CATT

R2-2500002 RAN2#128 Meeting Report, MCC