3GPP RAN WG2 Meeting #130 	R2-2504180

# St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.6.4

Source:	InterDigital, Inc.

Title:	Conditional LTM.

Document for:	Discussion

# Introduction

The Rel-19 WI on mobility enhancements contains the following measurement related objectives.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In RAN2#127bis, the following progress was made:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In RAN2#128, the following progress was made:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In RAN2#129 the following progress was made:

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In RAN2#129bis the following progress was made:

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

# Discussion

In previous meetings,  only conditional LTM to trigger the cell switch case has been discussed.

It was agreed to support explicit triggering of early UL sync (using PDCCH order for TA acquisition) and early DL sync (Rel-18 Early candidate TCI State activation/deactivation).

For event triggered reporting, the following scenarios are supported:

  - Select the candidate beam/cell to trigger early synchronization.

  - Select the target beam/cell and trigger LTM cell switch procedure.

For the conditional case, similar scenarios should also be supported. The conditional LTM trigger could be a trigger to execute LTM cell switch itself, or it could be a trigger for triggering RA for TA acquisition.

Proposal 1: A condition may be configured to trigger RA on a target cell for TA acquisition.

One of the main advantages of LTM in R18 is the reduction of interruption due to support of RACH-less, and it has been agreed to support RACH-less conditional LTM, and at least PDCCH ordered early TA acquisition.

For the case of PDCCH ordered RACH for TA acquisition, one scenario that we assume to be supported is the TA is provided by the source cell in a MAC CE, similar to R18 (either the existing cell switch MAC CE, with new indicator, or a new MAC CE). The main difference compared to R18 is that the resulting RACH-less LTM cell switch does not take place immediately, but rather only after the condition is met.

While this can provide some improvement compared to explicitly triggered cell switch, there is still some dependency on signalling from the source to enable the cell switch (e.g. an event triggered MAC CE to report measurements, and a MAC CE to provide TA, before the conditional LTM can occur)

The advantage of enabling conditional early UL sync would be that the TA value can be obtained early (ahead of cell switching) reducing the amount of time needed during cell switch itself. It also allows the RACH to be performed without relying on sending event triggered report to the source and receiving PDCCH order – the early sync and the cell switch could therefore be performed completely based on UE conditions and not rely on being enabled by the source just before cell switch.

For this approach, the conditional trigger for performing RACH would need to be set such that the condition is met earlier than the condition for performing cell switch. If conditional uplink sync is supported, then the UE will trigger a RACH to the target when the condition is met. In this case, it would make sense to obtain the TA by RAR.

Proposal 2: For conditional trigger used for performing RACH for UL sync, RAR containing TA is received from the target.

It was agreed in the previous meeting that “When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.”

While it might be interpreted that the same execution condition (same C-LTM Configuration) can be used to perform RACH or RACH-less depending on whether there is a valid TA, another interpretation could be that a conditional LTM configuration is only be valid while there is a valid TA.

It seems more straightforward to configure whether RACH-less is used for a specific event or not by RRC. That is, for a RACH-less conditional LTM, it is only to be executed if the condition is met, and a valid TA is stored. It would then be up to the network to configure a corresponding RACH-based conditional LTM, if necessary.

Proposal 3: RACH-less conditional LTM is executed only if the condition is met AND the UE has a valid TA.

Proposal 4: It is up to the network to configure a separate RACH-based conditional LTM, if necessary, for the same candidate (i.e. no dual RACH-less/RACH-based LTM for the same conditional event).

Reserving a CG in advance implies longer term resource reservation than compared to explicit RACH-less LTM. It could be beneficial if there is some indication shortly before the cell switch occurs, so that the CG can be reserved at the target.

One option is that the CG is reserved for the duration of TA validity time – so, e.g. the UE first reports an event triggered MAC CE report that the candidate cell measurement is above a threshold (event LTM4). Then the source issues a PDCCH order for the UE to send a PRACH to the target. After this the source sends a MAC CE containing the TA, and this enables RACH-less conditional LTM, with the CG being available for the duration of TA validity.

In case of conditional early UL sync, the TA value may come from a RAR on the target, but the same principle could apply.

Proposal 5: CG can be considered as valid for the duration of TA validity.

However, for the case of UE based TA measurement, the target may not know when the conditional LTM will occur. In this case the CG resource may need to be reserved for a long time unless there is some pre-notification. Similar to the DG case, a dedicated SR might be needed first in order to activate the CG. Otherwise, the UE may need to report completion of the UE based TA measurement to the source, so that the source can “enable” conditional LTM for a time duration (E.g. using a TA validity timer), to limit how long the target needs to reserve the CG resources.

Proposal 6: Discuss which of the following CG validity options for UE based TA measurement need to be supported:

- CG can always be considered as valid, if configured.

- CG can be considered as valid after dedicated SR transmission to target (at least for the case of UE based TA measurement)

- UE reports TA measurement completion to the source, to enable the CG.

# Conclusion

In this paper we make some suggestions on conditional LTM as follows:

Proposal 1: A condition may be configured to trigger RA on a target cell for TA acquisition.

Proposal 2: For conditional trigger used for performing RACH for UL sync, RAR containing TA is received from the target.

Proposal 3: RACH-less conditional LTM is executed only if the condition is met AND the UE has a valid TA.

Proposal 4: It is up to the network to configure a separate RACH-based conditional LTM, if necessary, for the same candidate (i.e. no dual RACH-less/RACH-based LTM for the same conditional event).

Proposal 5: CG can be considered as valid for the duration of TA validity.

Proposal 6: Discuss which of the following CG validity options for UE based TA measurement need to be supported:

- CG can always be considered as valid, if configured.

- CG can be considered as valid after dedicated SR transmission to target (at least for the case of UE based TA measurement)

- UE reports TA measurement completion to the source, to enable the CG.

# References

- RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”