3GPP TSG-RAN WG2 Meeting #130											        R2-2504202

Malta, 19-23 May 2025

Agenda item:			8.9.2

Title:						Further considerations on S&F operations

Source:					Continental Automotive

Document for:		Discussion and Decision

# Introduction

The aim of this 3GPP Rel-19 WI is to enhance IoT-NTN with the following objectives [1]:

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

o	Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

	At least specify necessary enhancements e.g. related to S1 protocol, especially to address
the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

During RAN2#128 [2] the following agreements were made:

Agreements:

1.	Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

2.	It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

3.	“S&F operation” indication is provided in SIB1

4.	RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN).

5.	RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation

6.	RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite

7.	RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

During RAN2#129 [3] and RAN2#129bis [4] the following agreements were made:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In this paper, we share further considerations on S&F operations.

# 2. Discussion

## On MME-configured satellite list: cell (re-)selection enhancement

In NGSO scenarios with earth-moving cells, feeder link is only intermittently available. User data needs to be stored at satellite until feeder link is available again and data can be forwarded to ground segment (e.g., packet data gateway).

Citation from TS 23.401, 4.13.9.1 [3] […] - When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. […]

As noted in the excerpt of [5] above, the MME-configured S&F monitoring list can be used by the UE as a means for reducing the probability that the UE will not be able to complete the NAS procedure.

However, some satellites operating in S&F mode may be (or become) capacity constraint. Hence, the network may want to control whether and which UEs can transmit data during S&F operations, which will impact MO data delivery.

To manage situations of constraint storage capacity and enhance quality of service at the network level, the network should provide further guidance to the UE whether and which service data to transmit when an MME-configured satellite is operating in S&F mode. For example, the network could differentiate between delay tolerant and non-delay tolerant services per logical channel and configure S&F-capable UEs to transmit data during S&F operations accordingly.

Proposal 1: To manage situations of constraint storage capacity and enhance quality of service, the network configures to UE which services are allowed to transmit when operating in S&F mode.

## On transition time

In RAN2#129 [4] and RAN2#129bis [5] agreements regarding transition time indications from S&F operation mode to normal mode and vice versa were made. For the normal mode to S&F mode transition, an FFS was captured to discuss whether this indication can be linked to other existing information. Further, companies discussed whether the time indication t-Service could be re-used.

Citation from TS 36.331, 6.3.1 [6] t-Service Time information on when an NTN cell is going to stop serving the area it is currently covering, as specified in TS 36.304 [4]. This field applies for service link switches in NTN quasi-Earth fixed cells and feeder link switches for both NTN quasi-Earth fixed and earth-moving cells.

As t-Service is a time indication that is broadcast and applies to the entire NTN cell, all UEs receive and may use the same time indication when accessing the network. This may result in congestion and potentially high network load. In case of colliding access attempts, UEs will retry connecting, which consumes additional energy. Hence, t-Service should not be re-used.

Proposal 2: t-Service should not be re-used as transition time indication from normal mode to S&F mode.

To avoid and/or mitigate the issue of colliding access attempts, particularly when operating in capacity constraint S&F mode, the network should configure different wait timers to the UEs considering the MME-configured satellite list. Further, the network can consider configuring cell- or UE-specific wait timers. For example, upon receiving the configured wait timer from the network, the UE moves to idle mode and refrains from cell search, measurements, and/or cell re-selection procedures until wait timer expired.

Proposal 3: The network should configure different wait timers to the UEs considering the MME-configured satellite list for the transition from normal mode to S&F mode.

# Conclusions

The following proposals have been made in this contribution:

Proposal 1: To manage situations of constraint storage capacity and enhance quality of service, the network configures to UE which services are allowed to transmit in UL when operating in S&F mode.

Proposal 2: t-Service should not be re-used as transition time indication from normal mode to S&F mode.

Proposal 3: The network should configure different wait timers to the UEs considering the MME-configured satellite list for the transition from normal mode to S&F mode.

# References

- RP-240776, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 (revision of RP-234077).

- R2-2410913, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#128, Orlando, Nov 18th – 22nd, 2024.

- 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access" (V19.2.0)

- R2-2501333, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#129, Athens, Greece, Feb. 17th-21st, 2025.

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#129bis, Wuhan, China, April 7th-11th, 2025.

- 3GPP TS 36.331: “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification” (V18.5.0)