3GPP TSG-RAN WG2 Meeting #130	R2-2504203

St. Julian’s, Malta,  May 19th – 23rd , 2025

Agenda item:	6.1.3.1

Source:	Samsung

Title:	Corrections on measurement with (cond)EventD1/D2/T1

WID/SID:	NR_NTN_solutions, NR_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In Rel-17/18, eventD1/D2 are introduced to support location-based measurement, condEventD1/D2/T1 are introduced to support location/time-based CHO. In Rel-17 NTN CHO, condEvent/D1/D2/T1 is always configured with an RRM event. In Rel-18 NTN CHO, condEvent/D1/D2/T1 can be configured without RRM event. In this paper, we discuss some potential issues in the current measurement procedure and propose potential solutions for correction. The corresponding CRs are provided in R2-2504204, R2-2504205, R2-2504206.

# Discussion

In the current procedure of performing measurements, UE in RRC_CONNECTED shall derive cell measurement results based on measurements of SSB or CSI-RS as yellow highlighted below and details specified in 5.5.3.3. For all cell measurement results, layer 3 filtering is applied as green highlighted below and details specified in 5.5.3.3, except for CBR, Rx-Tx time difference, and altitude measurements.

### 5.5.3 Performing measurements #### 5.5.3.1 General An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the altitude measurements. … 2> if the reportType for the associated reportConfig is periodical, eventTriggered; or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the MCG VarMeasConfig and is indicated in the condExecutionCond or in the condExecutionCondPSCell associated to a condReconfigId in the MCG VarConditionalReconfig (for CHO, CPA, MN-initiated inter-SN CPC, or subsequent CPAC in NR-DC); or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCond associated to a condReconfigId in the SCG VarConditionalReconfig (for intra-SN CPC or subsequent CPAC); or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the condExecutionCondSCG associated to a condReconfigId in the MCG VarConditionalReconfig (for SN-initiated inter-SN CPC or subsequent CPAC in NR-DC); or 2> if the reportType for the associated reportConfig is condTriggerConfig, the measId is within the SCG VarMeasConfig and is indicated in the triggerConditionSN associated to a condReconfigurationId in VarConditionalReconfiguration as specified in TS 36.331 [10] (for SN-initiated inter-SN CPC in EN-DC): 3> if a measurement gap configuration is setup, or 3> if the UE does not require measurement gaps to perform the concerned measurements: 4> if s-MeasureConfig is not configured, or 4> if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or 4> if s-MeasureConfig is set to csi-RSRP and the NR SpCell RSRP based on CSI-RS, after layer 3 filtering, is lower than csi-RSRP: 5> if the measObject is associated to NR and the rsType is set to csi-rs: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 filtered beam measurements only based on CSI-RS for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on CSI-RS for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3; 5> if the measObject is associated to NR and the rsType is set to ssb: 6> if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured: 7> derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a; 6> derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

For eventD1/D2 distance measurement and condEventT1 time measurement, layer 3 filtering is also not applicable since distance/time measurement is not cell measurement based on SSB/CSI-RS and is somewhat out of scope of measurement procedure here. Thus, it should be clarified as an exceptional case, same as the other exceptional cases already captured.

Observation 1: For eventD1/D2 distance measurement and condEventT1 time measurement, layer 3 filtering is also not applicable, thus, should be clarified as an exceptional case.

Proposal 1: Clarify in clause 5.5.3.1 that UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive distance measurements and time measurements.

As blue highlighted above, trigger quantity is configured for cell measurements (in MeasTriggerQuantity in ReportConfigNR for event-triggered report and CHO), and UE derives cell measurement results based on SSB/CSI-RS measurements for 1) the trigger quantity and 2) each measurement quantity indicated in reportQuantityCell. However, trigger quantity is not defined for eventD1/D2 or condEventD1/D2/T1, and reportQuantityCell is not applicable for CHO. It becomes not clear what is trigger quantity or measurement quantiy for event-triggered measurement with eventD1/D2 and for CHO with condEventD1/D2/T1. It is not clear for UE how to perform the procedure blue highlighted above. One possible solution is adding ”if configured” for trigger quantity and reportQuantityCell. However, in this way trigger/measurement quantity are not still not clear to UE. A clean solution is to clarify UE does not derive cell measurement results based on SSB or CSI-RS measurements.

Observation 2: Trigger quantity and measurement quantity are not defined for eventD1/D2 or condEventD1/D2/T1. It is not clear for UE how to derive cell measurement results for measurement configuration with eventD1/D2 or with condEventD1/D2/T1.

Proposal 2: Clarify that UE doesn’t derive cell measurement results based on SSB or CSI-RS measurements for distance measurements and time measurements, considering trigger quantity or measurement quantity cannot be configured in these cases.

In the current procedure of CHO evaluation, as grey highlighted below, it is not clear how UE determines the applicable cell for CHO with condEventD1/D2/T1 only. The reason is that it is not clear whether/how UE performs cell measurement when no RRM event is configured, so how to determine the cell which has a physical cell identity matching the value in CHO configuration seems not clear. One interpretation can be UE consider the cell(s) detected on the associated measObject (similar to applicable cell for reportCGI in clause 5.5.4.1), and how to configure measObject for cell measurement on SSB/CSI-RS is up to NW implementation.

##### 5.3.5.13.4 Conditional reconfiguration evaluation The UE shall: 1> for each condReconfigId within the VarConditionalReconfig: 2> if the RRCReconfiguration within condRRCReconfig includes the masterCellGroup including the reconfigurationWithSync: 3> if the associated condExecutionCondPSCell is configured: 4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell; and 4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the secondaryCellGroup within the nr-SCG within the received condRRCReconfig to be applicable cell; 3> else: 4> consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync within the masterCellGroup in the received condRRCReconfig to be applicable cell;

Observation 3: For CHO with condEventD1/D2/T1 only, it is not clear whether/how UE performs cell measurement based on SSB/CSI-RS when no RRM event is configured and how to determine applicable cell.

Proposal 3: To determine applicable cell for CHO with only condEventD1/D2/T1, clarify in clause 5.3.5.13.4 that UE considers the cell detected on the associated measObject which has a physical cell identity matching the value in CHO configuration.

The same issue exists for event-triggered measurement with eventD1/D2, that is, it is not clear how UE determines applicable cell to be included in the measurement report, as yellow highlighted below. The issue was discussed before but nothing was formally captured.

5.5.4.1. Measurement report triggering 4> for measurement events other than eventA1, eventA2, eventD1, eventD2, eventX2, eventH1 or eventH2: 5> if useAllowedCellList is set to true: 6> consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId; 5> else: 6> consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId; … 5.5.5.1. Measurement reporting 1> if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report: 2> if the reportType is set to eventTriggered or periodical: 3> if the measurement report concerns the candidate L2 U2N Relay UE: … 3> else: 4> set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following: 5> if the reportType is set to eventTriggered and eventId is not set to eventD1 or eventD2 or eventH1 or eventH2: 6> include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId; 5> else: 6> include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

One interpretation based on discussion in RAN2#126 is that it is up to UE implementation to determine the applicable cell(s) for event-triggered measurement with eventD1/D2. If this is common understanding, a note can be captured in spec. for future reference.

Observation 4: For event-triggered measurement with eventD1/D2, it is not clear how UE determines applicable cell to be included in the measurement report.

Proposal 4: Capture a note that it is up to UE implementation to determine the applicable cell(s) for event-triggered measurement with eventD1/D2.

# Conclusion

In this contribution we discussed the following proposals.

Observation 1: For eventD1/D2 distance measurement and condEventT1 time measurement, layer 3 filtering is also not applicable, thus, should be clarified as an exceptional case.

Proposal 1: Clarify in clause 5.5.3.1 that UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive distance measurements and time measurements.

Observation 2: Trigger quantity and measurement quantity are not defined for eventD1/D2 or condEventD1/D2/T1. It is not clear for UE how to derive cell measurement results for measurement configuration with eventD1/D2 or with condEventD1/D2/T1.

Proposal 2: Clarify that UE doesn’t derive cell measurement results based on SSB or CSI-RS measurements for distance measurements and time measurements, considering trigger quantity or measurement quantity cannot be configured in these cases.

Observation 3: For CHO with condEventD1/D2/T1 only, it is not clear whether/how UE performs cell measurement based on SSB/CSI-RS when no RRM event is configured and how to determine applicable cell.

Proposal 3: To determine applicable cell for CHO with only condEventD1/D2/T1, clarify in clause 5.3.5.13.4 that UE considers the cell detected on the associated measObject which has a physical cell identity matching the value in CHO configuration.

Observation 4: For event-triggered measurement with eventD1/D2, it is not clear how UE determines applicable cell to be included in the measurement report.

Proposal 4: Capture a note that it is up to UE implementation to determine the applicable cell(s) for event-triggered measurement with eventD1/D2.

# Reference

[1] R2-2504204 Corrections on measurement with (cond)EventD1/T1 Rel-17 CR

[2] R2-2504205 Corrections on measurement with (cond)EventD1/T1 Rel-18 CR mirrored

[3] R2-2504206 Corrections on measurement with condEventD1/D2/T1 Rel-18 CR