3GPP TSG-RAN2 Meeting #130-RAN2#130	R2-2504213

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2084 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PHR for STx2P multi-panel scheme |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Rel-18 STx2P multi-panel scheme, if twoPHRmode is not configured for the MAC entity transmitting the PHR MAC CE, only one PH, one Pcmax, and one MPE can be reported for a serving cell, and the report value is different for two cases: 1) there is at least one real PUSCH transmission; 2) there is no real PUSCH transmission. However, the latter case is missing in the current procedure for reporting Pcmax and MPE. It is supposed to be captured according to the following agreement in RAN2#127bis, but was missing in CR R2-2410528. R2-2409221 Report of [AT127bis][201][MIMOevo] PHR related aspects Samusng P1 and P2 are agreed, and we consider TP in the Appendix A of R2-2409221 agreeable. CR will be discussed in the next meeting.. |
| Summary of change: |  | In clause 5.4.6 for Pcmax and MPE reporting, add for the case there is no real PUSCH transmission at the slot where the PHR MAC CE is transmitted. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: PHR for STx2P single-DCI multi-panel scheme Inter-operability: If the UE is implemented according to this CR but the network is not, the network may not be able to receive the PHR for STx2P correctly. If the network is implemented according to this CR but the UE is not, the network may not be able to receive the PHR for STx2P correctly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For Rel-18 STx2P multi-panel scheme, Pcmax and MPE cannot be correctly reported in PHR procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.4.6	Power Headroom Reporting

The Power Headroom reporting procedure is used to provide the serving gNB with the following information:

-	Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell;

-	Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC, NE-DC, and NGEN-DC cases);

-	Type 3 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated Serving Cell;

-	MPE P-MPR: the power backoff to meet the MPE FR2 requirements for a Serving Cell operating on FR2;

-	DPC: the adjustment to maximum output power for a given power class for a Serving Cell operating on FR1;

-	DPCBC: the adjustment to maximum output power for a given power class for a Band Combination operating on FR1.

RRC controls Power Headroom reporting by configuring the following parameters:

-	dpc-Reporting-FR1;

-	phr-AssumedPUSCH-Reporting;

-	phr-PeriodicTimer;

-	phr-ProhibitTimer;

-	phr-Tx-PowerFactorChange;

-	phr-Type2OtherCell;

-	phr-ModeOtherCG;

-	multiplePHR;

-	mpe-Reporting-FR2;

-	mpe-ProhibitTimer;

-	mpe-Threshold;

-	numberOfN;

-	mpe-ResourcePoolToAddModList;

-	twoPHRMode.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

NOTE 1:	The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink of which firstActiveDownlinkBWP-Id is not set to dormant BWP;

-	activation of an SCG;

-	addition of the PSCell except if the SCG is deactivated (i.e. PSCell is newly added or changed);

-	phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

-	Upon switching of activated BWP from dormant BWP to non-dormant DL BWP of an SCell of any MAC entity with configured uplink;

-	if dpc-Reporting-FR1 is configured, ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance, as specified in TS 38.101-1 [14] and TS 38.101-3 [16]).

-	if mpe-Reporting-FR2 is configured, and mpe-ProhibitTimer is not running:

-	the measured P-MPR applied to meet FR2 MPE requirements as specified in TS 38.101-2 [15] is equal to or larger than mpe-Threshold for at least one activated FR2 Serving Cell since the last transmission of a PHR in this MAC entity; or

-	the measured P-MPR applied to meet FR2 MPE requirements as specified in TS 38.101-2 [15] has changed more than phr-Tx-PowerFactorChange dB for at least one activated FR2 Serving Cell since the last transmission of a PHR due to the measured P-MPR applied to meet MPE requirements being equal to or larger than mpe-Threshold in this MAC entity.

in which case the PHR is referred below to as 'MPE P-MPR report'.

NOTE 2:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,f,c/PH when a PHR is triggered by other triggering conditions.

NOTE 3:	If a HARQ process is configured with cg-RetransmissionTimer and if the PHR is already included in a MAC PDU for transmission on configured grant by this HARQ process, but not yet transmitted by lower layers, it is up to UE implementation how to handle the PHR content.

If the MAC entity has UL resources allocated for a new transmission the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start phr-PeriodicTimer.

1>	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in clause 5.4.3.1:

2>	if multiplePHR with value true is configured:

3>	for each activated Serving Cell with configured uplink associated with any MAC entity of which the active DL BWP is not dormant BWP; and

3>	for each activated Serving Cell with configured uplink associated with E-UTRA MAC entity:

4>	if this MAC entity is configured with twoPHRMode:

5>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN and the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	if the UE supports mTRP-PUSCH-PHR-Type1-Reporting-r17:

7>	obtain two values of the Type 1 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell.

6>	else:

7>	obtain two values of the Type 1 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell.

5>	else if this Serving Cell is configured with multiple TRP PUSCH repetition (i.e., not configured with multipanelSchemeSDM or multipanelSchemeSFN) and the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	obtain two values of the Type 1 or the value of Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell.

5>	else:

6>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell.

4>	else (i.e. this MAC entity is not configured with twoPHRMode):

5>	if this Serving Cell is configured with multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN and if the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	if there is at least one real PUSCH transmission at the slot where the PHR MAC CE is transmitted:

7>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

8>	if the first TCI-State or TCI-UL-State is applied for a real PUSCH transmission:

9>	obtain the value of the Type 1 power headroom of the real PUSCH transmission associated with the first TCI-State or TCI-UL-State for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

8>	else:

9>	obtain the value of the Type 1 power headroom of the real PUSCH transmission associated with the second TCI-State or TCI-UL-State for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

7>	else if this Serving Cell is configured with multiple TRP PUSCH repetition:

8>	obtain the value of the Type 1 power headroom of the first real transmission of the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

6>	else if there is no real PUSCH transmission at the slot where the PHR MAC CE is transmitted:

7>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

8>	obtain the value of the Type 1 power headroom of the reference PUSCH transmission associated with the first TCI-State or TCI-UL-State for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

7>	else if this Serving Cell is configured with multiple TRP PUSCH repetition:

8>	if the UE supports mTRP-PUSCH-PHR-Type1-Reporting-r17:

9>	obtain the value of the Type 1 power headroom of the reference PUSCH transmission associated with the SRS-ResourceSet with a lower SRS-resourceSetID for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

8>	else:

9>	obtain the value of the Type 1 power headroom of the reference PUSCH transmission associated with the SRS-ResourceSet with a lower SRS-resourceSetID or the value of the Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213[6] for NR Serving Cell.

5>	else:

6>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell.

4>	if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

5>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

5>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

6>	if dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 or dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 is set to enabled in the active BWP of this Serving Cell:

7>	obtain the value for the corresponding PCMAX,f,c field for assumed PUSCH from the physical layer if available, as specified in clause 7.7 of TS 38.213 [6].

6>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

6>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

7>	obtain the value for the corresponding MPE field from the physical layer.

4>	else (i.e. if this MAC entity is not configured with phr-AssumedPUSCH-Reporting):

5>	if this MAC entity is configured with twoPHRMode and any Serving Cell belonging to this MAC entity is configured with multipanelSchemeSDM or multipanelSchemeSFN; and

5>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN and the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	obtain two values for the corresponding PCMAX,f,c,k fields from the physical layer.

6>	if mpe-Reporting-FR2 is configured for the MAC entity this Serving Cell belongs to and this Serving Cell operates on FR2:

7>	obtain two values for the corresponding MPEk fields from the physical layer.

5>	else if this MAC entity is not configured with twoPHRmode, or if this MAC entity is configured with twoPHRMode and any Serving Cell belonging to this MAC entity is configured with multiple TRP PUSCH repetition; and

5>	if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN and the MAC entity this Serving Cell belongs to is configured with twoPHRMode:

6>	if the first TCI-State or TCI-UL-State is applied for a real PUSCH transmission at the slot where the PHR MAC CE is transmitted:

7>	obtain the value for the PCMAX,f,c field for the real PUSCH transmission associated to the first TCI-State or TCI-UL-State from the physical layer.

7>	if mpe-Reporting-FR2 is configured for the MAC entity this Serving Cell belongs to and this Serving Cell operates on FR2:

8>	obtain the value for the corresponding MPE field for the real PUSCH transmission associated to the first TCI-State or TCI-UL-State from the physical layer.

6>	else if the second TCI-State or TCI-UL-State is applied for a real PUSCH transmission at the slot where the PHR MAC CE is transmitted:

7>	obtain the value for the PCMAX,f,c field for the real PUSCH transmission associated to the second TCI-State or TCI-UL-State from the physical layer.

7>	if mpe-Reporting-FR2 is configured for the MAC entity this Serving Cell belongs to and this Serving Cell operates on FR2:

8>	obtain the value for the corresponding MPE field for the real PUSCH transmission associated to the second TCI-State or TCI-UL-State from the physical layer.

6>	else if there is no real PUSCH transmission at the slot where the PHR MAC CE is transmitted:

7>	obtain the value for the PCMAX,f,c field for the reference PUSCH transmission associated to the first TCI-State or TCI-UL-State from the physical layer.

7>	if mpe-Reporting-FR2 is configured for the MAC entity this Serving Cell belongs to and this Serving Cell operates on FR2:

8>	obtain the value for the corresponding MPE field for the reference PUSCH transmission associated to the first TCI-State or TCI-UL-State from the physical layer.

5>	else:

6>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

6>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

7>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

7>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

8>	obtain the value for the corresponding MPE field from the physical layer.

7>	if mpe-Reporting-FR2-r17 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

8>	obtain the value for the corresponding MPEi field from the physical layer;

8>	obtain the value for the corresponding Resourcei field from the physical layer.

7>	if dpc-Reporting-FR1 is configured and ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting is triggered and this Serving Cell operates on FR1 and this Serving Cell is associated to this MAC entity:

8>	obtain the value for the corresponding DPC field(s) from the physical layer.

3>	if phr-Type2OtherCell with value true is configured:

4>	if the other MAC entity is E-UTRA MAC entity:

5>	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity);

5>	if phr-ModeOtherCG is set to real by upper layers:

6>	obtain the value for the corresponding PCMAX,f,c field for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity) from the physical layer.

3>	if this MAC entity is configured with mpe-Reporting-FR2-r17:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple entry PHR as defined in clause 6.1.3.49 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and any Serving Cell belonging to this MAC entity is configured with multipanelSchemeSDM or multipanelSchemeSFN:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE as defined in clause 6.1.3.82 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and any Serving Cell belonging to this MAC entity is configured with multiple TRP PUSCH repetition:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Multiple Entry PHR for multiple TRP MAC CE as defined in clause 6.1.3.51 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR with assumed PUSCH MAC CE as defined in clause 6.1.3.79 based on the values reported by the physical layer.

3>	else:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR MAC CE as defined in clause 6.1.3.9 based on the values reported by the physical layer.

2>	else (i.e. Single Entry PHR format is used):

3>	if this MAC entity is configured with twoPHRMode for multiple TRP PUSCH repetition or multipanelSchemeSDM or multipanelSchemeSFN:

4>	obtain two values of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell.

3>	else:

4>	obtain the value of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell.

3>	if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

4>	if dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 or dynamicTransformPrecoderFieldPresenceDCI-0-2-r18 is set to enabled in the active BWP of this Serving Cell:

5>	obtain the value for the corresponding PCMAX,f,c field for assumed PUSCH from the physical layer, if available, as specified in clause 7.7 of TS 38.213 [6].

3>	if this MAC entity is configured with twoPHRMode and if this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

4>	obtain two values for the corresponding PCMAX,f,c,k fields from the physical layer.

4>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

5>	obtain two values for the corresponding MPEk fields from the physical layer.

3>	else:

4>	obtain the value for the corresponding PCMAX,f,c field from the physical layer;

4>	if mpe-Reporting-FR2 is configured and this Serving Cell operates on FR2:

5>	obtain the value for the corresponding MPE field from the physical layer.

4>	if mpe-Reporting-FR2-r17 is configured and this Serving Cell operates on FR2 and this Serving Cell is associated to this MAC entity:

5>	obtain the value for the corresponding MPEi field from the physical layer;

5>	obtain the value for the corresponding Resourcei field from the physical layer.

4>	if dpc-Reporting-FR1 is configured and this Serving Cell operates on FR1:

5>	obtain the value for the corresponding DPC field from the physical layer.

3>	if this MAC entity is configured with mpe-Reporting-FR2-r17:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Single entry PHR as defined in clause 6.1.3.48 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and this Serving Cell is configured with multipanelSchemeSDM or multipanelSchemeSFN:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Single Entry PHR for multiple TRP STx2P MAC CE as defined in clause 6.1.3.81 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with twoPHRMode and this Serving Cell is configured with multiple TRP PUSCH repetition:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Single Entry PHR for multiple TRP MAC CE as defined in clause 6.1.3.50 based on the values reported by the physical layer.

3>	else if this MAC entity is configured with phr-AssumedPUSCH-Reporting:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR with assumed PUSCH MAC CE as defined in clause 6.1.3.78 based on the values reported by the physical layer.

3>	else:

4>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR MAC CE as defined in clause 6.1.3.8 based on the values reported by the physical layer.

2>	if this PHR report is an MPE P-MPR report:

3>	start or restart the mpe-ProhibitTimer;

3>	cancel triggered MPE P-MPR reporting for Serving Cells included in the PHR MAC CE.

2>	start or restart phr-PeriodicTimer;

2>	start or restart phr-ProhibitTimer;

2>	cancel all triggered PHR(s).

All triggered PHRs shall be cancelled when there is an ongoing SDT procedure as in clause 5.27 and the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the PHR MAC CE plus its subheader.