3GPP TSG RAN WG2 Meeting #130	 R2-2504214

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.2.2

Source:	Futurewei

Title:	Discussion on LCM for UE-Side Model for Beam Management Use Case

Document for:	Discussion

# Introduction

During RAN2 meeting #129bis [10], discussions were focused on the following aspects of LCM for UE-Side Model.

- Details related to Option B: Inference related parameters for applicability determination/reporting

- Parameters included within Option B

- Applicability reporting, including cause of non-applicability, inference configuration handling upon non-applicability, prohibit timer for applicability reporting,

- Data collection, including details on data collection request

The group reached the follow agreements.

Agreements on option B

- RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list).

- Potential aspects to consider if RAN2 revisit:

- To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig.

- To take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec.

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

Agreements on data collection configuration

- The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

    - An indication on start/stop of data collection

    - Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In this contribution, we will further discuss LCM for UE-side models focusing on the details, including clarifications and revisions of the LCM procedures.

# Procedures for UE-side LCM

## Background Information

In RAN2 meeting #127 [5] (Maastricht), the procedures for UE-side LCM were discussed, based on the result of post-meeting 126 email discussion. The following agreements were achieved [5].

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.

- FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities.

- FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

  - FFS via UAI or RRCReconfigurationComplete, etc

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities can be activated at the same time.

  - FFS what is the granularity of functionality

- We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In meeting #127bis (Hefei), further agreements have been made for the LCM.

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step 3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.

  - FFS how this is signaled (e.g. explicitly/implicitly).

  - Consider different scenarios, whether it is regarding an active functionality.

- Data collection initiation and configuration for data collection is under network control.

  - FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.

Can come back to this discussion later.

In meeting #128 (Orlando), the following agreements about LCM has been made.

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

Also during RAN2 meeting #128 (Orlando, RAN1 meeting #119), RAN1 sent the reply LS to RAN2 on applicable functionality reporting for beam management UE-sided model [8]. The Option A and Option B of applicability report mentioned in the following agreements are from RAN1 agreements in the LS, as quoted below.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

In meeting #129 (Athens), the following agreements about LCM has been made.

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause.

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- The provided periodic CSI configuration should be consistent with reported UE capabilities

- FFS option B

- Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

  - Semi-persistent reporting, activated by MAC CE/DCI

  - Aperiodic CSI reporting, activated by DCI

In meeting #129bis (Wuhan), the topic was further discussed and the agreements reached can be found in the Introduction section (Section 1).

In the following sections, the discussions will be based on the above agreements.

## NW-side additional conditions

In Step 1 and Step 2 of the agreed LCM procedure, NW configures the UE to report supported functionalities and UE reports them. It is our understanding that the main purpose of Step 3 is for the NW and the UE to exchange necessary information so that the UE can decide on the applicable functionalities. For example, the NW provides NW-side additional conditions so that the UE can choose the right applicable functionality. Therefore, we think this step (i.e., the NW providing additional conditions to the UE) should be mandatory, unless the model was trained specifically for this NW so no additional conditions are needed.

Proposal 1: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities.

Note that with this proposal, the following (underlined) FFS after Step 3 can be removed (we don’t need to deal with this situation).

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

## The use of associated IDs

In the LS RAN2 sent to RAN1, RAN2 asked the following question related to additional conditions.

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

In RAN1’s response to RAN2 [8], RAN1 provided the following answer to RAN2’s question.

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Note that the concept of associated ID was initially designed to reflect the configuration for data collection, and then was extended to a broader concept to ensure the consistency of NW-side additional condition across training and inference for UE-sided model. In both RAN1 and RAN2, there isn’t a definition for associated ID. What is it and what is it for? Will it be used for other purposes too? We think it is necessary to clearly define the concept of associated ID and its usage.

Observation 1: The exact definition and usage of Associated ID was not clear and needs to be clarified.

## Identifying parameter sets for Option B

During RAN2 meeting #128 (Orlando, RAN1 meeting #119) [7], RAN1 sent the reply LS to RAN2 on applicable functionality reporting for beam management UE-sided model [8]. The reply LS provides two options for applicability reporting, Option A and Option B, as quoted below.

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

A good illustration of signaling flows of Option A and Option B can be found in Figure 1 of [11], which is also quoted below.

Figure 1 Signaling flow for Option A) and Option B)

As we can see from the agreement and the figure, in the case of Option B, the UE sends the initial applicability report in RRCReconfigurationComplete in Step 4, upon receiving a set or multiple sets of inference related parameters from the NW in Step 3 (for applicability report only). In addition, we have agreed that UAI can be sent to update applicability of the set(s) of the inference parameters. To differentiate these potential multiple sets of parameters, and to know which set of parameters the UAI is going to update, the set of parameter needs an identifier or an index.

Proposal 2: Define an identifier or an index for identifying inference parameters.

## UE internal capability/capacity and CSI Processing Unit

Besides AI/ML-based UE capability reporting (which associates with AI/ML functionality), another important aspect the UE needs to report to the NW is the UE’s internal capability or capacity for model inference, for example, its computational power and memory. Note this is different from AI/ML-based UE capability reporting; it is used for the NW to decide whether the UE will be able to handle the task of certain model inference.

During RAN1 meeting #120, the topic of CSI processing unit (CPU) was discussed in Agenda Item 9.1.3 (CSI Prediction) where various companies expressed different opinions regarding CSI processing criteria for UE-sided AI/ML-based CSI prediction. The central point of discussion revolved around how the computational resources should be handled for the new AI/ML-based functionality in relation to legacy CSI reporting.

Based on the discussion, we observed that separating the CPU counting for legacy and AI/ML-based CSI reporting provides a more accurate, flexible, and manageable approach to handling the computational demands of this new technology. It aligns with the potential for dedicated AI/ML processing hardware, facilitates better resource management and prioritization, enables more informative UE capability reporting, and supports efficient sharing of AI/ML resources within the UE.

We further observed that supporting the sharing of AI/ML-based computing resources among different AI/ML features and functionalities is a more efficient, flexible, and scalable approach. It optimizes resource utilization, potentially reduces power consumption, simplifies UE capability reporting, and aligns with industry trends in AI hardware design. Counting the resources of this shared pool separately from legacy CPU usage provides a clearer understanding of the processing demands of AI/ML-based operations, facilitating better network management and configuration.

For this upcoming meeting (RAN1 #120bis), the topic of CPU was arranged to be discussed in all CSI-related features of AI/ML use cases/sub use cases, as RAN1 Chair indicated in his email to the group, as below.

- For any processing unit related discussions on normative work, handle under agenda items 9.1.1, 9.1.2, and 9.1.3.

- For any processing unit related discussions headed for TR, handle under agenda item 9.1.4

With this, I think we can even merge 9.1.4.1 and 9.1.4.2 as follows:

9.1.4.1 CSI compression and any other aspects on AI/ML model/data

Including any remaining discussions on “Processing Unit”.

The above emphasizes the importance of UE capability reporting, in particular, the computational power of the UE.

This is also covered in the RAN2 agreements above, after Step 3, as quoted below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

The “UE-side additional conditions (internally known by UE)” in this agreement can include the UE’s capability or capacity for model inference. we believe this information should also be made known to the NW (not only to the UE itself) for various reasons. This information may include UE’s computational power (as captured in CPU for AI/ML-based features), memory, storage, model inference time, battery level etc. that are applicable for AI/ML functionality operations. Also included should be the timeline of each resources that is being used to help the NW to better schedule the inference of different functionalities/models.

With this information, the NW can have a better understanding of the resources available at the UE side and activate/deactivate AI/ML-based functionality at the right time/situation. For example, if the UE’s current computational power or memory is not able to support the inference of a certain functionality, the NW will not instruct the UE to activate the functionality.

To capture this aspect, we propose a revised agreement for the text after Step 3 and Step 4.

Proposal 3: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - UE-side additional conditions can include UE’s capabilities and capacities that are related to AI/ML functionality operations, such as CPU for AI/ML-based features, memory, storage, model inference time, battery level and etc.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

## Activating multiple applicable functionalities at the same time

After Step 5, the applicable functionality may be activated by the received inference configuration in Step 5. In this stage, one of the topics listed for further discussion is whether multiple applicable functionalities can be activated at the same time.

  - FFS if multiple applicable functionalities can be activated at the same time.

However, we think the question itself needs to be clarified first. When we talk about multiple functionalities, there could be two different things.

- In the first case, when these multiple applicable functionalities are for the same use case/sub-use case, they are likely developed for different applicable situations. In this case we need some mechanisms to differentiate different applicable functionalities (e.g., functionality ID).

- In the second case, when these multiple applicable functionalities are not for the same use case/sub-use case, for example, one for beam management and the other for CSI prediction, then there is no reason to activate them at the same time, for the same use case/sub use case. Note that it should be always allowed to activate different functionalities for different use cases or sub use cases.

Based on the above discussion, we think the FFS belongs to the first case. We hence propose the following revision to the agreement. After this clarification, whether they can be activated at the same time can be further studied.

Proposal 4: Revise the procedures for UE-side LCM as below.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities for the same use case/sub use case can be activated at the same time.

  - FFS what is the granularity of functionality

# Conclusions

In this contribution, we discussed our views on LCM for UE-side models for the beam management use cases. Based on the discussions in the previous sections, our proposals are as follows.

Observation 1: The exact definition and usage of Associated ID was not clear and needs to be clarified.

Proposal 1: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities.

Proposal 2: Define an identifier or an index for identifying inference parameters.

Proposal 3: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - UE-side additional conditions can include UE’s capabilities and capacities that are related to AI/ML functionality operations, such as CPU for AI/ML-based features, memory, storage, model inference time, battery level and etc.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

Proposal 4: Revise the procedures for UE-side LCM as below.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities for the same use case/sub use case can be activated at the same time.

  - FFS what is the granularity of functionality

# References

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024

Chair’s notes, R2_127bis_ChairNotes_ 24-10-18_15-45-_eom_clean, RAN2 meeting #127bis, Hefei, Oct 2024

Chair’s notes, R2_128_ChairNotes_24-11-22_17-00_eom, RAN2 meeting #128, Orlando, Nov 2024

R2-2500011 (R1-2410898), Reply LS on applicable functionality reporting for beam management UE-sided model, TSG RAN WG1, Orlando, Nov 2024

Chair’s notes, R2_129_ChairNotes_25-02-21_final.docx, RAN2 meeting #129, Athens, Feb 2025

Chair’s notes, R2_129b_ChairNotes_25-04-11_16-45 final, RAN2 meeting #129bis, Wuhan, Apr 2025

R2-2502108, Further Discussion on LCM for UE-side Model for AI-BM, Mediatek, Wuhan, Apr 2025.