3GPP TSG-RAN WG2 Meeting #130						   R2-2504239

St Julian's, Malta, 19 - 23 May, 2025

Title: 		Discussion on training data collection for UE-sided model for BM and CSI prediction

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

- Introduction

In [1], open issue RRC-3 is for UE data collection request, and the email rapporteur suggested to further discuss this open issue. This paper provides our views on this open issue. In addition, we also would like to discuss the following issues:

(1) potential RAN2 impacts due to CSI prediction (UE-sided model). We think the discussion of open issue RRC-3 may be common for AI/ML based beam management and AI/ML based CSI prediction, so we discuss both use cases together in section 2

(2) other issues, e.g. need of prohibit timer

- Discussion

Background of Open issue RRC-3

Open issue RRC-3: UE data collection request

Issue description: For UE-side data collection, the details of UAI signaling for the UE start/stop request and especially the following aspects are not yet clarified:

- how to refer in UAI to a preferred radio resource candidate configuration from a list of candidate configurations provided by NW

- where/what the NW provides as candidate configurations

- what the content of otherConfig for enabling UE data collection requests in UAI should be (e.g. just a flag, the list of candidate UE data collection configurations, etc.).

This issue refers to the RAN2#129bis agreements:

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

The issue is captured as an editor’s notes in the running CR, clause 5.7.4.3, 6.2.2, and 6.3.4.

Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

Background of the use case CSI prediction

In the WID RP-250792, we have a use case CSI prediction:

- CSI feedback enhancement, encompassing [RAN1/RAN2]:

  - CSI prediction (UE-sided model):

    - Functionality-based LCM leveraged from other use cases, when necessary and applicable,

    - Study, and if necessary, specify consistency of training/inference,

    - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

This UC is only about UE-sided model, and RAN1 has made some progress. Details can be found in Section 5. For data collection configuration for training, RAN1 mainly concluded on the following:

[RAN1#120bis agreement] For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported. [RAN1#120bis agreement] For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report - FFS on how to indicate without CSI report in CSI-ReportConfig [RAN1#120 agreement] For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead. - FFS on CSI report configuration

Discussion on Open issue RRC-3

General discussion on AI/ML based CSI prediction

RAN2#129 made the following agreements, and these agreements should be naturally applied to CSI prediction. There is no need to discuss them again in RAN2.

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

RAN2#129bis made the following agreements, and for now it is only about AI/ML based beam management. We think these agreements can be applied to CSI prediction as well.

- Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In summary, we have the following proposal:

Proposal 1: (RRC-3) The following RAN2#129bis meeting agreements are also applied for AI/ML based CSI prediction:

The UE can request measurement configuration for AI/ML based CSI prediction. The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Where/what the NW provides as candidate configurations, how to refer in UAI to a preferred configuration and the content of otherConfig

At RAN1#120bis, the following agreements were made for BM:

Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Firstly, RAN2#129bis agreed on two types of UE request, i.e. (1) an indication on start/stop of data collection, or (2) preferred configuration. For start/stop indications, we understand that they are indicated per UE, i.e. the UE can indicate whether data collection procedure has been started or not, or whether data collection procedure needs to be stopped or not. For the content of otherConfig, we think the following method could be considered:

- a flag for the 1st type of UE request

- a list of candidate UE data collection configurations for the 2nd type of UE request

The network can decide to send one of the above indications or both to UE, and then UE can send the relevant UE request to the network. For example, if only the flag is sent from the network to UE, the network can directly send training data collection to the UE, and then the UE may send start/stop indications to the network. If only the list is sent from the network to UE, the signalling details are discussed as below. If both flag and list are sent to the UE, it also works.

Secondly, for both BM and CSI prediction, we think the network can configure one or more CSI-ReportConfig as candidate configurations for the UE, and each has a CSI-ReportConfigId. Such configurations can be done via the IE OtherConfig. Then, the UE can indicate the preferred CSI-ReportConfigId(s) to network, and these ids should be from the previous list of candidate configurations sent from the network. Afterwards, the network can decide on the final configurations. It may happen that the network wants the UE to keep the list and at the same time the network sends the final configurations to the UE. Later, the UE can still use the list for reporting the preferred configuration, if the UE’s preference changes. This follows the behavior which is used for most items that are indicated via UE assistance information.

Proposal 2: (RRC-3) For both BM and CSI prediction, NW can configure a flag for the UE to send an indication on start/stop of data collection.

Proposal 3: (RRC-3) For both BM and CSI prediction, NW can configure one or more CSI-ReportConfig in OtherConfig as a list of candidate configurations.

Proposal 4: (RRC-3) For both BM and CSI prediction, from the list of candidate configurations, the UE  indicates the preferred CSI-ReportConfigId(s) to network.

Proposal 5: (RRC-3) For both BM and CSI prediction, the UE can indicate the updated preferred configuration in case it changes at a later stage.

Need of prohibit timer for UE side data collection request for BM and CSI prediction

In UE-side data collection part, we have agreement as following:

Agreement The UE can request measurement configuration for data collection of AI/ML based beam management.

We need to consider a scenario where the UE initiates a data collection request but does not receive a response from the network (i.e., the network fails to provide the necessary configuration for UE-side data collection), the UE may repeatedly send request messages, leading to excessive signaling overhead. To mitigate this issue, it is essential to implement mechanisms to limit such abnormal behavior and prevent the associated signaling overhead. This is related to the following FFS in the running CR:

Editor's Note: FFS other procedures, e.g. prohibit timer.

As a commonly used method to avoid the frequent request by UE, a prohibit timer can be configured. After UE sends a request for data collection configuration, prohibit timer should start, and while it is running, the UE should not send the request again, even if the NW does not send data collection configuration to the UE. This is also applicable for CSI prediction.

Proposal 6: (Other issues) For both BM and CSI prediction, a prohibit timer can be introduced to prevent the UE from sending a data collection request repeatedly.

- Conclusion

In this paper, we mainly discuss open issue RRC-3 for both BM and CSI prediction, and we also discuss the need of prohibit timer. The proposals are listed as below:

Proposal 1: (RRC-3) The following RAN2#129bis meeting agreements are also applied for AI/ML based CSI prediction:

The UE can request measurement configuration for AI/ML based CSI prediction. The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Proposal 2: (RRC-3) For both BM and CSI prediction, NW can configure a flag for the UE to send an indication on start/stop of data collection.

Proposal 3: (RRC-3) For both BM and CSI prediction, NW can configure one or more CSI-ReportConfig in OtherConfig as a list of candidate configurations.

Proposal 4: (RRC-3) For both BM and CSI prediction, from the list of candidate configurations, the UE  indicates the preferred CSI-ReportConfigId(s) to network.

Proposal 5: (RRC-3) For both BM and CSI prediction, the UE can indicate the updated preferred configuration in case it changes at a later stage.

Proposal 6: (Other issues) For both BM and CSI prediction, a prohibit timer can be introduced to prevent the UE from sending a data collection request repeatedly.

- References

[1]	RRC open issues AIML PHY_v15_Rapp

- Annex - RAN1 progress on CSI prediction

RAN1#120bis agreements

Agreement

Introduce a dedicated AI/ML PU for AI/ML features for UE,

- The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

Conclusion

For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported.

Agreement

For CSI prediction using UE-side model, for training data collection, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report

  - FFS on how to indicate without CSI report in CSI-ReportConfig

Agreement

For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring

Agreement

For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

Agreement

For the definition of SGCS,

Note: How to handle layer mapping mismatch, if any, is up to UE implementation.

Agreement

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

- Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window

  - Monitoring window can be configured by NW

    - FFS on signalling details

- FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances

- FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband)

- FFS on whether to report per layer, the first layer, averaged over layer

- FFS on how to quantize and report mechanism

Agreement

For CSI prediction using UE side model, for inference, consider following options for potential down selection

- Option 1: only dedicated AI/ML PU (OAPU) is occupied

- Option 2: only legacy CPU (OCPU) is occupied

- Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- FFS whether OAPU and OCPU are based on defined rule and/or reported by UE

- Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported.

The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

- Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Agreement

For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

- Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion

- Detailed parameter name is upto RAN2

RAN1#120 agreements

R1-2501528	Summary #1 of CSI prediction	Moderator (LG Electronics)

From Wednesday session

Agreement

For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused.

- For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported.

- For inference report,

  - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

R1-2501529	Summary #2 of CSI prediction	Moderator (LG Electronics)

R1-2501530	Summary #3 of CSI prediction	Moderator (LG Electronics)

From Thursday session

Agreement

For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support

- Based on intermediate KPI

  - Down select between SGCS and NMSE by RAN1#120bis

  - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

Agreement

For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on

- Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting

- Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities.

- Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates.

- Whether legacy framework for active CSI-RS resource and port counting can be reused

Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point.

R1-2501610	Summary #4 of CSI prediction	Moderator (LG Electronics)

Presented in Friday session

Agreement

For CSI prediction using UE-side model, for data collection for training,

- For resource configuration,

  - For CSI-RS resource type for CMR, periodic and semi-persistent are supported.

    - FFS on support of aperiodic CSI-RS resource

  - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI

  - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead.

- FFS on CSI report configuration

Final summary in R1-2501626.