3GPP TSG RAN WG2 Meeting #130                          	   R2-2504246

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.12.3

Source:	Qualcomm Incorporated

Title:	Discussion on UE initiated beam reporting

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

For UE-initiated/event-driven beam management, the following subject was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we discuss UL skipping with UE initiated beam reporting.

# Discussion

At RAN2 #129bis, the following agreement was made for UE initiated beam report Mode A and Mode B [2].

Agreement - Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. - FFS if any other MAC impact for UL skipping

The following UL skipping is currently supported for both dynamic grant and configured grant [3].

Based on the conditions for UL skipping, UL skipping may be triggered for UE initiated beam reporting Mode A using the legacy DCI for dynamic grant.

Observation 1. UL skipping may be triggered if a dynamic grant carries only UE initiated beam report.

Since the UCI (SR-like) requesting a dynamic grant for UE initiated beam report is handled by PHY layer, MAC layer doesn’t know if a dynamic grant indicated by a DCI is for UE initiated beam report or not or the MAC layer doesn’t know if PHY is wanting for a dynamic grant for transmit a UE initiated beam report.

Observation 2. MAC is not aware of whether a dynamic grant is for UE initiated beam report or whether PHY has UE initiated beam report to send.

MAC needs to know if any UE initiated beam report needs to be sent at PHY and then MA can decide if UL skipping can be applied or not.

Observation 3. UL skipping is not applied by MAC if PHY can indicate to MAC that PHY has UE initiated beam report to transmit.

Based on the observations, we propose the following proposal.

Proposal 1. To avoid UL skipping, PHY sends an indication to MAC when there is a UE initiated beam report to be sent.

# Conclusion

In this contribution, we discussed UL skipping with UE initiated beam report and concluded with the following observations and proposals.

Observation 1. UL skipping may be triggered if a dynamic grant carries only UE initiated beam report.

Observation 2. MAC is not aware of whether a dynamic grant is for UE initiated beam report or whether PHY has UE initiated beam report to send.

Observation 3. UL skipping is not applied by MAC if PHY can indicate to MAC that PHY has UE initiated beam report to transmit.

Proposal 1. To avoid UL skipping, PHY sends an indication to MAC when there is a UE initiated beam report to be sent.

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- “R2_129b R18 MIMO R19 MIMO LPWUS SBFD NR_Others (Erlin) 2025-04-11 eom”, RAN2 #129bis, Wuhan, China, Apr. 7th – 11th, 2025

- 3GPP TS 38.321 “Medium Access Control (MAC) protocol specification (Release 18)” V18.5.0 (2025-03)