3GPP TSG-RAN WG2 Meeting #130		R2-2504249

Malta, MT, 19th – 23rd May. 2025

Source:	China Telecom

Title:	Discussion on UE initiated beam reporting

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

MAC impact In Mode A of UE-initiated CSI reporting, the active time of a DRX operation includes the time after a new UCI for UE-initiated beam reporting is sent on first PUCCH. Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. FFS if any other MAC impact for UL skipping The UE continues to perform CSI measurements for the UEIBM procedure when the active BWP is the dormant BWP. If the BWP in an SCell is a dormant BWP, the UE should not report mode-A beam measurement results. The UE cannot perform mode-B beam reporting on this BWP. RAN2 understand the event evaluation and report triggering for UE-initiated beam report is captured by RAN1 spec. RRC impact enabledCurrentBeamReport-r19 is added as an optional need-R field. Reuse resourcesForChannelMeasurement in CSI-ReportConfig. Clarify in the field description that for UEI BM, the new beam to be measured is either CSI-RS (nzp-CSI-RS-ResourceSetList) or SSB (csi-SSB-ResourceSetList). ng-n1-n2-r19, cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 and cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 in the same way as corresponding legacy fields i.e.: - ng-n1-n2-r19 is defined in the same way as ng-n1-n2 in R15 typeI-multiPanel - cri-typeI-SinglePanel-ri-restriction-r19/cri-typeII-ri-restriction-r19 are defined in the same way as legacy RI restrictions - cri-typeI-SinglePanel-CBSR-r19/cri-typeII-CBSR-r19 are defined in the same way as n1-n2-codebookSubsetRestriction-r18. mrSelectedResources is defined as a SEQUENCE structure containing two fields with integer values from one to eight. delayOffsetCompensation can be located under CSI-AperiodicTriggerState and outside of CSI-AssociatedReportConfigInfo and that the parameter triggeringScheme is not needed. Define numberofSubbandsPO as a list (with size up to the number of subbands) where each element is an integer value within the maximum size of a BWP. The following is used the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank: - (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2;

According to last meeting’s progress as above, we will discuss the RAN2 aspects related to UEIBM.

- Discussion

Issue 1: UEI-BR is trigger when UE is not UL synchronized

A UE may enter the UL out-of-sync state due to the case such as TAT expiry, RACH failure, handover or cell reselection, BFR, etc. Among these, when TAT expiry, there is still significance in transmitting the UE initiated beam reporting. Therefore, when the UE enters the UL out-of-sync state due to TAT expiry and UE initiated beam reporting is triggered, the UE shall initiate a RACH procedure to re-establish UL synchronization and further acquire uplink transmission resource.

- Proposal 1: When the UE enters the UL out-of-sync state due to TAT expiry and UE initiated beam reporting is triggered, the UE shall initiate a RACH procedure to re-establish UL synchronization.

Issue 2: Collaborative design of UEI-BR and other mechanism

UE primarily performs operations such as cell measurements, beam measurements, and cell reselection within measurement gap. If UE performs beam measurements during measurement gap, the UE initiated beam reporting may represent earlier beam measurement feedback. In this case, it is preferable to perform UE initiated beam reporting. For the cases where UE performs other measurements within the measurement gap, it tends to not perform UE initiated beam reporting.

Proposal 2: If UE performs beam measurements during measurement gap, it is preferable to perform UE initiated beam reporting; otherwise, it tends to not perform UE initiated beam reporting.

- Conclusion

- Proposal 1: When the UE enters the UL out-of-sync state due to TAT expiry and UE initiated beam reporting is triggered, the UE shall initiate a RACH procedure to re-establish UL synchronization.

Proposal 2: If UE performs beam measurements during measurement gap, it is preferable to perform UE initiated beam reporting; otherwise, it tends to not perform UE initiated beam reporting.