3GPP TSG RAN WG2#130	R2-2504264

St Julian’s, Malta, 19th - 23rd May 2025

Agenda item:	8.4.2

Source:	Nokia

Title:	LP-WUS in IDLE and INACTIVE

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a new work item (RP-240801) was agreed for Low Power Wake Up Signal and Receivers designs. These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and receiver architectures.

In this contribution we focus on the following objective:

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# 2	Discussion

RAN2 discussed the following scenarios described in [2]:

RAN2 agreed the following:

RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

There is a common understanding that the UE may not support LP-WUS on all the bands the UE support. UE may be camping on the cell on the band where LP-WUS is not supported by the UE/NW, although cell supporting LP-WUS maybe available on the other band.  In this case LP-WUS UE power saving gains would be lost. Since it was agreed that any overload issues can be addressed by current mechanism in spec it would be beneficial to allow the UE to camp on the cells supporting LP-WUS more often to save battery.

Proposal 1: Cell (re)selection rules are enhanced so that LP-WUS UE prioritizes bands supporting LP-WUS

Additional aspect is that what would be the OFDM-based receiver behaviour when both SSB-based and LP-SS based thresholds (together with LP-SS itself) are configured for LP-WUS monitoring. In this case it would seem preferable that if UE is capable to do measurements based on the SSB (i.e. supports OFDM-based WUR), UE would apply the thresholds based on SSB. This would allow network to configure these thresholds in a manner that would reflect the expected receiver performance.

Proposal 2: If both LP-SS based and SSB based thresholds are configured for LP-WUS monitoring, OFDM-based WUR applies the SSB-based threshold (and metric) for evaluation.

Small data transmission (SDT) was specified in R17 and R18. SDT is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT can be initiated either by the UE in case of MO-SDT (Mobile Originated SDT) or by the network in case of MT-SDT (Mobile Terminated SDT). MO-SDT is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. SDT was specified also for power saving purposes like LP-WUS and therefore it can be assumed that it may be beneficial to implement both SDT and LP-WUS features in the same device.

Proposal 3: LP-WUS UE may support MO-SDT and/or MT-SDT

Proposal 4: The UE can initiate MO-SDT while UE is monitoring LP-WUS

Proposal 5: NW can initiate MT-SDT while UE is monitoring LP-WUS

In scope of the IDLE/INACTIVE mode operation, the WID objective seeks to specify LP-WUS triggered paging monitoring. In strict sense this would seem to imply that LP-WUS triggering should result UE to monitor the associated PO. In Rel-18 different approaches were discussed how to determine the PO which UE should monitor, upon LP-WUS triggering:

- ‘Dynamic PO’; where LP-WUS trigger the UE to monitor a time occasion where PDCCH and PDSCH are monitored would appear dynamically, some predefined time after the LP-WUS

- ‘Closest PO’; where the LP-WUS trigger the UE to monitor PO that would be closest to the LP-WUS

- ‘Normal PO’; where LP-WUS would trigger the UE to monitor the same IDLE/Inactive mode PO that UE would monitor normally without LP-WUS

In any case network won’t be aware if UE is monitoring LP-WUS or, as a fall-back, the PO(s). This means that network would need to account both possible situations when transmitting the paging. From network perspective, the alternative procedures, ‘dynamic PO’ and ‘closest PO’ would mean that network would need in minimum transmit the paging in two alternative POs during the paging cycle, one associated to the LP-WUS and other in the normal PO. This would increase the paging overhead, doubling it. Based on that the following is proposed.

Proposal 6: UE monitors the legacy PEI and/or PO after receiving LP-WUS indicating wake-up and no changes to the paging occasion to be monitored are supported.

As a part of the WID objectives the need to determine entry and exit conditions for LP-WUS monitoring was identified. These procedures are mostly related to RAN2 and the objective (overall) is to be led by RAN2:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

Now in scope of the LR based measurements, both entry/exit conditions would be relevant. For entry/exit condition, it could be expected that network can set a threshold for the UE in the system information, after which the LP-WUS should not be monitored and vice versa when LP-WUS should be monitored. Based on this the following is proposed.

The following baseline was agreed in the RAN2#126 meeting:

- Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Based on the above baseline the UE behaviour is not clear i.e. it is not clear when the UE monitors LP-WUS or legacy PO. Deterministic and testable UE behaviour should always be targeted and therefore we propose the following:

Proposal 7: It is specified when the UE supporting LP-WUS shall monitor LP-WUS and when legacy PO.

Proposal 8: The UE supporting LP-WUS shall start LP-WUS monitoring when LP-WUS monitoring entry condition (better than threshold) is fulfilled.

Proposal 9: The UE supporting LP-WUS shall stop LP-WUS monitoring when LP-WUS monitoring exit condition (worse than threshold) is fulfilled. In this case the UE shall monitor legacy PO instead of LP-WUS.

# 3	Conclusion

The following is proposed based on the discussion:

Proposal 1: Cell (re)selection rules are enhanced so that LP-WUS UE prioritizes bands supporting LP-WUS

Proposal 2: If both LP-SS based and SSB based thresholds are configured for LP-WUS monitoring, OFDM-based WUR applies the SSB-based threshold (and metric) for evaluation.

Proposal 3: LP-WUS UE may support MO-SDT and/or MT-SDT

Proposal 4: The UE can initiate MO-SDT while UE is monitoring LP-WUS

Proposal 5: NW can initiate MT-SDT while UE is monitoring LP-WUS

Proposal 6: UE monitors the legacy PEI and/or PO after receiving LP-WUS indicating wake-up and no changes to the paging occasion to be monitored are supported.

Proposal 7: It is specified when the UE supporting LP-WUS shall monitor LP-WUS and when legacy PO.

Proposal 8: The UE supporting LP-WUS shall start LP-WUS monitoring when LP-WUS monitoring entry condition (better than threshold) is fulfilled.

Proposal 9: The UE supporting LP-WUS shall stop LP-WUS monitoring when LP-WUS monitoring exit condition (worse than threshold) is fulfilled. In this case the UE shall monitor legacy PO instead of LP-WUS.

# 4	References

[1] RP-243266 Way forward on Rel-19 LP-WUS separate band issues RAN2 chair, Interdigital

[2] RP-242656 On LR and MR operating in different bands in release 19, Vodafone, Vivo, Huawei, HiSilicon