3GPP TSG-RAN WG2 Meeting #130	R2-2504274

St Julian’s, Malta, May. 19th – 23rd, 2025

Agenda Item:	 8.13.3

Source:	 Huawei, HiSilicon

Title:	 Control Plane Procedures for Multi-hop Relay

Document for:  Discussion and Decision

# 1	Introduction

In RAN2 #129bis meeting, we made good progress on the control plan procedures for multi-hop relay, and reached the following agreements on the baseline procedure, SRAP configuration, and paging. The FFSs and open issues are highlighted in yellow [1].

RAN2#129bis meeting agreements [1]:

Agreements on baseline Control Plane design: Control plane design considers only approach 1 (along with the fast forwarding enhancement for SRB0 if agreed), and approach 2 is not further pursued in Rel-19. FFSs for Fast forwarding enhancement for SRB0 : Continue to consider forwarding of SRB0 messages by relay UEs not in RRC_CONNECTED with respect to control plane approach 1. TPs showing the two approaches for fast forwarding of SRB0 (SRAP header and local ID assignment by RRC signalling) are invited for next meeting (co-sourcing strongly encouraged). Other approaches are not precluded (contribution-driven) but should be shown at a mature stage considering the time left. Strive to avoid additional RAN3 impact specific to fast forwarding. FFS if applicable to DL. FFS what level of gNB awareness of the path information would be needed. FFS if fast forwarding is optional/mandatory for UEs to support. Agreements on SRAP: Reuse the single-hop relay mechanism to support the Local ID allocation for multi-hop relay: • First relay UE reports the L2 ID of the remote UE to the gNB to request the local ID allocation, the uniqueness of the local ID within the cell is assumed to be guaranteed by the gNB by implementation. • The remote UE local ID is 8 bits. As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay, SRAP header is present over each hop except the PC5 hop between the remote UE and first relay UE. As in single-hop U2N Relay mechanism, R2 confirm, for the DL and UL SRB0 of remote UE in multi-hop U2N Relay: - At the link between remote UE and the first relay UE, reuse the specified PC5 RLC channel (i.e., SL-RLC0); - At the link between intermediate relay UEs or the link between intermediate relay and the last relay, or the link between the last relay and the network, the RLC channel is configured by the network via dedicated RRC message. SRAP configuration including remote UE ID and BEARER ID is configured by the network via dedicated RRC signalling as in R17. For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for at least the directly connected child UEs, based on the L2 IDs of the direct connected child UEs. For CR drafting, assume as a baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination. We will review next meeting if this explicit mapping functionality is needed. FFS whether to introduce reflective bearer mapping as an optional feature. From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information: • Local identity of the Remote UE • The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping From a relay UE perspective (including first/intermediate/last relay UEs), the legacy contents of the SRAP configuration are applied for at least the directly connected child UEs. FFS (depending on outcome of the reflective mapping) if they are also applied for indirectly connected child UEs. Agreements on paging: Support the child UE relying on the parent relay UE for paging monitoring. Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, while avoiding duplicated paging delivery to the remote UE due to double-monitoring by upstream UEs.

In this paper, we discuss the FFSs and open issues from last meetings. In addition, we also discuss other important issues not  being  discussed before. Considering that there are  only two meetings left to finalize this work item, we need to prioritize the baseline procedures to make the multi-hop relay work, and consider the enhancements only if time allows.

# 2	Discussion

## Latency Reduction of Connection Establishment

### Alternative Solutions for Reducing Connection Establishment Latency

In the email discussion [2] and in the submitted contribution [3], some companies think that for IDLE/INACTIVE relay UE, the latency of remote UE connection establishment could be a problem. Particularly in the case when  all the relay UEs participating in the multi-hop connection establishment are in RRC IDLE/INACTIVE state, the overall latency for connection establishment could be large as latency could increase as the number of hops increases. If following the legacy triggering mechanism where the IDLE/INACTIVE relay UE(s) triggers the RRC connection establishment when receiving the SRB0 message from its adjacent child UE,, the multi-hop remote UE needs to wait for the completion of the connection establishment procedures of all the IDLE/INACTIVE relay UEs, which is done in a recursive way, before it can receive the configuration for the SRB1.

Even though the latency in above scenario could be a problem, the scenario where the Remote UE triggers connection establishment while all the relay UEs are RRC IDLE/INACTIVE is unlikely to occur frequently in reality. A more common scenario could be that the last and intermediate relay UEs are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UE.

Observation 1: The scenario where all the relay UEs involved in the multi hop connection establishment are in the RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment is unlikely to occur frequently in reality. A more common scenario is that the last and intermediate relay UEs (including the first relay UE) are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs.

Furthermore, the latency problem in Approach 1 can be resolved with some simple solutions rather than having the solutions involving new SRAP Headers. Some of these potential solution is described below

#### 2.1.1.1 Relay UE indicates its RRC State in the Discovery Message

A relay UE can indicate its RRC states in the discovery message, so that the remote UE can prioritize to select an RRC CONNCETED relay UE if the service requirements of the remote UE are stringent. In SA2, it was already agreed that the hop count can be carried in the discovery message [4]. With the hop count information and then RRC state information made available to the remote UE, the remote UE can select the path on which the relay UE are in RRC_CONNECTED if it has stringent service requirement depending on its implementation. If the relay UE does not have stringent service requirements, it can choose a path through RRC_IDLE/RRC_INACTIVE relay UEs

The specification impacts for introducing this change involves including the RRC State information in the container SL-AccessInfo-L2U2N of the discovery message as shown below.

=================================START CHANGE=======================================

#### –	SL-AccessInfo-L2U2N

The IE SL-AccessInfo-L2U2N includes the radio information included in Discovery Message used for L2 U2N relay operation.

SL-AccessInfo-L2U2N information elements

-- ASN1START

-- TAG-SL-ACCESSINFO-L2U2N-START

NR-Sidelink-DiscoveryMessage DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CellAccessRelatedInfo,

SL-ServingCellInfo-r17,

SL-RelayIndicationMP-r18

FROM NR-RRC-Definitions;

SL-AccessInfo-L2U2N-r17 ::=             SEQUENCE {

cellAccessRelatedInfo-r17               CellAccessRelatedInfo,

sl-ServingCellInfo-r17                  SL-ServingCellInfo-r17,

...,

[[

sl-RelayIndication-r18                  SL-RelayIndicationMP-r18                  OPTIONAL

]],

[[

relayRRC-State-r19              		ENUMERATED {idle, inactive, connected}	  OPTIONAL

]]

}

END

-- TAG-SL-ACCESSINFO-L2U2N-STOP

-- ASN1STOP

=================================END CHANGE=======================================

The state information in the Discovery message will also be helpful for the for Remote UE in selecting the path in which multi hop Relay UE are in RRC_CONNECTED state for supporting the service continuity scenarios C and D.

#### 2.1.1.2 Limit the discovery procedure based RRC State and number of hops

Another simple potential solution is to impose a limit on the maximum hop number between the relay UEs and the network when RRC IDLE/INACTIVE relay UEs perform the multi-hop relay discovery operations.. For example, this limit could be setup for one hop lesser for RRC IDLE/INACTIVE relay UEs compared to the maximum number of hops supported when they are in RRC_CONNECTED state. As a result, the IDLE/INACTVIE relay UE can suspend the discovery procedure if the number of hops between the relay UE and the network exceed this threshold. This approach prevents a Remote UE from selecting an IDLE/INACTIVE relay UE that is already at the maximum supported hop count from the network, thereby helping to reduce latency during the RRC Connection Setup procedure

This restriction can be captured in for example in TS 38.300 specification in section 16.12.3 without impacting Stage 3 specification as below.

=================================START CHANGE=======================================

### 16.12.3	Relay Discovery

Model A and Model B discovery models as defined in TS 23.304 [48] are supported for U2N/U2U Relay discovery. The protocol stack used for discovery is illustrated in Figure 16.12.3-1.

Figure 16.12.3-1: Protocol Stack of Discovery Message for UE-to-Network/UE-to-UE Relay

The U2N Remote UE can perform Relay discovery message (i.e., as specified in TS 23.304 [48]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The network may broadcast or configure via dedicated RRC signalling a Uu RSRP threshold, which is used by the U2N Remote UE to determine if it can transmit Relay discovery messages to U2N Relay UE(s).

The U2N Relay UE can perform Relay discovery message (i.e., as specified in TS 23.304 [48]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The network may broadcast or configure via dedicated RRC signalling a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, which are used by the U2N Relay UE to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

In multi-hop U2N relay, one U2N Relay UE establishes a PC5 connection with each of its child UE(s). One intermediate U2N Relay UE or U2N Remote UE maintains a single PC5 connection with its parent UE.

In multi-hop U2N Relay UEs in RRC_CONNECTED can perform discover message (i.e., as specified in TS 23.304 [48]) transmission until the hop count reaches the Maximum Hop Count Limit of 3 where as U2N Relay UEs in RRC_IDLE/RRC_INACTIVE can perform discover message transmission only up to a hop count of 2 in this release.

The U2U Remote UE and U2U Relay UE can perform Relay discovery message transmission or DCR/DCA message with integrated discovery transmission and may monitor for Relay discovery message or DCR/DCA message with integrated discovery while in coverage (i.e. RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED) or out-of-coverage.

The network may provide the Relay discovery configuration using broadcast or dedicated signalling. In addition, the U2N/U2U Remote UE, L3 U2N Relay UE and U2U Relay UE may use pre-configuration for Relay discovery. If the candidate intermediate U2N Relay UE is out of coverage without a PC5 connection to a parent UE, it can forward discovery messages based on pre-configuration. If the candidate intermediate U2N Relay UE is in coverage without a PC5 connection to a parent UE or out of coverage with a PC5 connection to a parent UE, it can forward discovery message based on SIB12 or dedicated configuration.

The resource pool(s) used for NR sidelink communication can be used for Relay discovery or the network may configure resource pool(s) dedicated for Relay discovery. Resource pool(s) dedicated for Relay discovery can be configured simultaneously with resource pool(s) for NR sidelink communication in system information, dedicated signalling and/or pre-configuration. Whether dedicated resource pool(s) for Relay discovery are configured is based on network implementation. If resource pool(s) dedicated for Relay discovery are configured, only those resource pool(s) dedicated for Relay discovery shall be used for Relay discovery. If only resource pool(s) for NR sidelink communication are configured, all the configured resource pool(s) can be used for Relay discovery and NR sidelink communication. Only the resource pool for NR sidelink communication is used for the DCR/DCA message with integrated discovery.

For U2N Remote UE (including both in-coverage and out of coverage cases) that has been connected to the network via single/multip-hop U2N Relay UE(s), only resource allocation mode 2 is used for Relay discovery message transmission.

For in-coverage U2N Relay UE, and for both in-coverage and out of coverage U2N Remote UEs, NR sidelink resource allocation principles are applied for Relay discovery message transmission.

For U2U Remote UE and U2U Relay UE, NR sidelink resource allocation principles, both mode 1 and mode 2, can be applied for Relay discovery message transmission.

The sidelink power control for the transmission of Relay discovery messages is same as for NR sidelink communication.

No ciphering or integrity protection in PDCP layer is applied for the Relay discovery messages.

The U2N/U2U Remote UE and U2N/U2U Relay UE can determine from SIB12 whether the gNB supports Relay discovery, or Non-Relay discovery, or both.

=================================END CHANGE=======================================

#### 2.1.1.3 Authorised Relay UEs remain in RRC_CONNECTED for longer duration

For emergency or disaster recovery scenario certain Relay UE(s) can be authorised to remain in RRC Connected State for longer duration after establishing the connection with the network, mainly to serve other intermediate and remote UEs until the recovery operation is ongoing. This authorisation information can be provided to the UE at the NAS layer and gNB can receive the authorization for these relay UEs from the CN when the connection is established in NGAP : INITIAL CONTEXT SETUP REQUEST message.

After the emergency or disaster recovery operation is completed the Relay UEs can request the RRC Connection Release from the network through UE Assistance Information message which will trigger the RRC Connection release for these UEs.

In summary this solution would require the following specification impacts

- Authorisation for Relay UEs to remain in RRC_CONNECTED state to be included in TS 23.304 and TS 38.413

- Indication in UE Assistance Information message to the network to initiate the release RRC Connection when there are no connected UEs for certain time.

Considering the benefits and minimal specification impact of the three candidate solutions described above—each aiming to reduce latency compared to the Fast/Parallel RRC Connection Setup Procedure or the RRC-based solution involving allocation of a local ID—it would be worthwhile to evaluate these simpler alternatives before deciding to adopt the more complex Fast/Parallel RRC Connection Setup or the local ID-based approach. Therefore, we propose the following

Proposal 1: Considering the benefits and minimal specification impact of the three candidate solutions, one of the following candidate solutions can be adopted to reduce the latency associated with the connection establishment procedure of a remote UE:

- Solution 1 - The Relay UE includes its RRC state in the discovery message, allowing the remote UE to prioritize selection of an RRC_CONNECTED Relay UE when its service requirements are stringent otherwise it can choose the path through RRC_IDLE or RRC_INACTIVE Relay UEs.

- Solution 2 - An RRC_IDLE or RRC_INACTIVE Relay UE performs the discovery procedure only until the hop count between the relay UE and the network reaches (maximum supported hop count − 1).

- Solution 3 - Certain Relay UE(s) can be authorised to remain in RRC Connected State for longer duration after establishing the connection with the network thereby improving their availability for low-latency relay service

## 2.2 Open issue RRC-4: Extending T300, T301 and T319

Another issue for the connection establishment is the timer for the connection establishment with multi-hop since the legacy timer is only for the single-hop. If we reuse the legacy timer value, Remote UE may fail to set up its RRC connection, due to its parent relay UE(s) taking too long time to prepare the RRC configuration for the remote UE’s initial access. We believe such failures can be resolved with extended timers for multi-hop. T300, T301 and T319 timer could be increased for multi-hop U2N relay operation in accordance with the hop count.

Proposal 2: Extend the T300, T301 and T319 timer for multi-hop U2N relay operation in accordance with the hop count for connection establishment/ re-establishment and resume procedure.

## 2.3 QoS split

Based on approach 1, the gNB determines the configurations of all the remote UEs and relay UEs, the same as legacy in rel-17, thus QoS can be guaranteed by the gNB by implementation.

Proposal 3. RAN2 confirm that the QoS split is determined by gNB and guaranteed by gNB by implementation.

## SRAP configuration

In this section, we discuss the FFS that “…if this explicit mapping functionality is needed. FFS whether to introduce reflective bearer mapping as an optional feature.”

In RAN2 #129 meeting, we agreed that the remote UE ID and bearer ID can be included in the SRAP PDU header for data forwarding. For a relay UE, it will receive the SRB/DRB data from the ingress relay RLC channel of the ingress link, and forward the data to the egress relay RLC channel of the egress link.

In the UL, when the relay UE receives the data from its child UE (a relay UE or a remote UE), it will forward the data to its parent relay UE according to the SRAP PDU header and the bearer mapping configuration. The bearer mapping configuration need to contain the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel. Since SRB and DRB can share same index, ingress PC5 relay RLC channel between the relay UE and its child UE can be used to identify the SRB index and the DRB index.

In the DL, the relay UE can also forward the data based on the mapping between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel of the egress link, and ingress PC5 relay RLC channel between the relay UE and its parent UE can be used to identify the SRB index and the DRB index.

From the discussion above, we can have the following proposals for the SRAP PDU format and SRAP configuration design for the multi-hop relay.

Proposal 4: The SRAP configuration contains the mapping configuration between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel.

- For the first relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the egress PC5 relay RLC channel between the first relay UE and remote UE, the egress PC5 relay RLC channel between the first relay UE and its parent relay UE.

- For intermediate relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its parent relay UE.

- For the last relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress Uu relay RLC channel on Uu, the egress PC5 relay RLC channel between the last relay UE and its child relay UE.

Besides, since the relay UE can connect to several child UEs if it is not the first relay UE, the path information corresponding to the egress link is needed. For handling the path information, there are two options that can be considered.

Option 1:  Derivation of implicit path information by the relay UEs - The last or the intermediate relay UE may implicitly derive the path information by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission, e.g., uplink transmission of RRCSetupRequest message. This mapping can then be used to route the downlink packets for the remote UE via the child relay UE.

Option 2:  Explicit signaling of the path information to the relay UEs - The other option is to explicitly provide the path information through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

Proposal 5: For handling the Multi hop relay path information there are two options that can be considered

Option 1 - The path information can be implicitly delivered by last or the intermediate relay UE by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission of RRCSetupRequest message

Option 2 - The path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

## Paging forwarding

In RA2#129bis, some companies proposed the duplicated paging delivery because the intermediate and last relay UE may both monitor the paging for the remote UE. In our understanding, the monitoring of the last relay UE paging should be the baseline since the Uu interface of the intermediate relay UE is not stable or may be unavailable if the intermediate relay UE moves out of the cell coverage area. The intermediate relay UE can determine on its own based on its implementation whether to monitor paging for the remote UE.

Proposal 6: Paging monitoring by the last relay UE for the remote UE(s)/child UEs should be considered as the baseline approach. An intermediate relay UE, when within cell coverage, may optionally monitor paging for the connected remote/child UEs based on its implementation.

# 3	Conclusion

Alternative Solutions for Reducing Connection Establishment Latency

Observation 1: The scenario where all the relay UEs involved in the multi hop connection establishment are in the RRC IDLE/INACTIVE state when the remote UE triggers the connection establishment is unlikely to occur frequently in reality. A more common scenario is that the last and intermediate relay UEs (including the first relay UE) are already in connected state and have established the connection to the network for their own traffic before they start serving the remote UEs.

Proposal 1: Considering the benefits and minimal specification impact of the three candidate solutions, one of the following candidate solutions can be adopted to reduce the latency associated with the connection establishment procedure of a remote UE:

- Solution 1 - The Relay UE includes its RRC state in the discovery message, allowing the remote UE to prioritize selection of an RRC_CONNECTED Relay UE when its service requirements are stringent otherwise it can choose the path through RRC_IDLE or RRC_INACTIVE Relay UEs.

- Solution 2 - An RRC_IDLE or RRC_INACTIVE Relay UE performs the discovery procedure only until the hop count between the relay UE and the network reaches (maximum supported hop count − 1).

- Solution 3 - Certain Relay UE(s) can be authorised to remain in RRC Connected State for longer duration after establishing the connection with the network thereby improving their availability for low-latency relay service

Open issue RRC-4: Extending T300, T301 and T319

Proposal 2: Extend the T300, T301 and T319 timer for multi-hop U2N relay operation in accordance with the hop count for connection establishment/ re-establishment and resume procedure.

QoS split

Proposal 3. RAN2 confirm that the QoS split is determined by gNB and guaranteed by gNB by implementation.

SRAP configuration

Proposal 4: The SRAP configuration contains the mapping configuration between the remote UE’s SRB/DRB and the egress Uu/PC5 Relay RLC channel.

- For the first relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the egress PC5 relay RLC channel between the first relay UE and remote UE, the egress PC5 relay RLC channel between the first relay UE and its parent relay UE.

- For intermediate relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its child relay UE, the egress PC5 relay RLC channel between the intermediate relay UE and its parent relay UE.

- For the last relay UE, the SRAP configuration includes the remote UE ID (L2 ID and local ID), SRB/DRB ID, the identity (L2 ID) of its child relay UE, the egress Uu relay RLC channel on Uu, the egress PC5 relay RLC channel between the last relay UE and its child relay UE.

Proposal 5: For handling the Multi hop relay path information there are two options that can be considered

Option 1 - The path information can be implicitly delivered by last or the intermediate relay UE by mapping local ID of the remote UE (carried in the SRAP header) to the L2 ID of the child relay UE during the uplink transmission of RRCSetupRequest message

Option 2 - The path information can be explicitly provided through the identity (L2 ID) of the child relay UE in the SRAP configuration for the intermediate and last relay UE.

Paging forwarding

Proposal 6: Paging monitoring by the last relay UE for the remote UE(s)/child UEs should be considered as the baseline approach. An intermediate relay UE, when within cell coverage, may optionally monitor paging for the connected remote/child UEs based on its implementation.

# 4	Reference

- Session Chair Notes on Relay and Positioning RAN2 #129bis;

- [AT129bis][409][Relay] Fast/parallel RRC forwarding for approach 1 (Apple)

- R2-2503078	Fast/Parallel RRC Setup for MH relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

- TS 23.304 V19.3.0, Proximity based Services (ProSe) in the 5G System (5GS).