3GPP TSG-RAN WG2 #130	R2-2504297

Malta, Malta, 19th – 23th May, 2025

Agenda Item:	8.3.5

Source:	Ericsson

Title:	Discussion on Performance Monitoring

Document for:	Discussion

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. One of the objectives included in the SID is the Life Cycle Management (LCM) of the AI/ML model:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

For performance monitoring in AI for mobility, the agreements below have been reached in following RAN2#129bis.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2. Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

Agreements 7. For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

This contribution shares our views on different aspects (signaling solution as well as performance metrics) of performance monitoring in AI/ML for mobility use cases.

# Discussion

## General Configuration and Report signalling framework

For performance monitoring with UE-side model in functionality-based LCM, network or UE is expected to monitor the performance of inference configuration for AI functionality at the UE side, and thus majority part of performance monitoring and report configuration would be quite similar to inference and report configuration. In RAN2#129bis meeting, it has been agreed that the inference and report configuration for AI/ML mobility can be based on RRM measurement framework. It is reasonable to use RRM measurement and reporting framework as starting point for performance monitoring in RRM measurement prediction and RRM measurement event prediction. Then the UE conducts performance monitoring according to the performance monitoring occasion in accordance with the configurations send by the network.

- UE conducts performance monitoring according to the performance monitoring configuration configured by the network.

- Performance monitoring for RRM measurement (and event) prediction with UE-side model is based on the existing RRM measurement and reporting framework.

In addition, we think the network should configure the UE with additional information to assist the UE conducting the performance monitoring in specific time windows (monitoring window) or for what percentage/portion of the predictions the UE should conduct performance monitoring. Therefore, we propose the following.

- Network configures the UE with the performance monitoring window and/or the percentage of the prediction samples for which performance monitoring is needed.

In the next section we also propose different metrics for the network side and the UE side performance monitoring of the UE sided models. For example, in case of network side performance monitoring of the UE side models the UE should be configured to send the ground truth (RRM measurements) to the network and in UE assisted performance monitoring case the UE should report some performance metrics to the network. Hence, we think the network should be able to configure the monitoring quantity based on which the UE performs monitoring and reports the configured quantity to the network. Therefore, we have the following proposal.

- Network configures the UE with the performance monitoring metric/quantity based on which the UE performs monitoring and sends the monitoring report to network.

## Triggers for performance monitoring report

Performance monitoring reports could be triggered via two different approaches. One approach for performance monitoring report is to be triggered periodically, where network configures UE to report its performance monitoring every N-th occasion of inference occasion, or every x second. However, such periodic reporting mechanisms is not efficient in capturing critical, context-sensitive events. This implies that an event-based reporting framework for performance monitoring reports is also needed to enable UE to evaluate and report predictive performance metrics only when specific conditions are met, such as statistical performance metric are below or higher than identified threshold, RSRP difference is higher than a threshold, data/prediction drift, confidence degradation. The event-based reporting enables not only signaling overhead reduction in uplink, but also enhances the responsiveness and precision of mobility management by delivering actionable insights when predictive model performance deteriorates. This in turn would enables the network to take actions to cope with the performance degradation whenever it happens.

- UE reports performance monitoring outcome in periodic or event triggered basis, according to the performance monitoring report configuration configured by the network.

    - Network-side performance monitoring of the UE sided models

Regarding network-side performance monitoring for inference configuration with UE-side model, network calculates performance metric(s) based on received prediction results and ground truth from UE if the network configures the performance monitoring configuration. Hence the network can make a decision on whether to deactivate the inference configuration and/or activate other inference configuration(s) and/or fallback to legacy measurements.

- For the network side performance monitoring, the UE reports ground truth measurements to enable the network monitoring the performance of the UE AI functionalities.

In intra-frequency temporal RRM measurement prediction case A, the UE can be configured to provide the ground truth information for a portion of the prediction instances associated to a specific inference configuration, if requested by the network. Needless to mention that the performance monitoring should be performed right at the time instances that the predictions are collected.  Therefore, we believe the performance monitoring configuration should be associated with the inference configuration for which the performance monitoring is expected by the network.

- For the temporal prediction Case A network should configures the UE with performance monitoring configuration associated/linked with the inference configuration.

However, in intra-frequency temporal RRM measurement prediction case B, UE is not able to get ground truth via this method and moreover RRM measurement as inference input and RRM measurement prediction as inference output are filtering-based results and time-domain correlated. Therefore, for the case B, RRM measurement in monitoring resources for performance monitoring should be linked with corresponding inference configuration. Additionally, in legacy RRM measurement report configuration, network can configure report periodicity (i.e. reportInterval) but can’t indicate exact time instances of RRM measurement for one RRM measurement report. However, to support performance metric calculation at the network side, the time instances for predicted RRM measurement and ground-truth RRM measurement should be aligned or very closed to each other. So, in our understanding, the report for monitoring and inference are correlated too.

- For temporal prediction case B, network receives prediction of the L3 measurements as part of measurement report, as well as L3 measurements, which act ground truth in the network’s performance monitoring.

- UE reports L3 measurements as ground truth to the network which are associated to the predictions for the temporal prediction Case B.

- For network-side performance monitoring in intra-frequency temporal RRM measurement prediction case B with UE-side model, performance monitoring configuration need to be associated with corresponding inference configuration.

In inter-frequency RRM measurement prediction, RRM measurement prediction in one carrier frequency is based on RRM measurement results in another one or more carrier frequencies. In this case, for one inference instance, it is expected that the time instances for these RRM measurements and RRM measurement prediction in different frequencies are very closed to each other as shown in Figure 1. As mentioned above, RRM measurement in one carrier frequency is filtering-based results and time-domain correlated, and moreover legacy RRM measurement and report configuration only indicates report periodicity rather than exact measurement time instances for one reported RRM measurement. Therefore, similar to intra-frequency temporal RRM measurement predication case B, RRM measurement and report of monitoring resources should be linked with corresponding inference configuration in inter-frequency RRM measurement prediction, especially when partial monitoring occasion is configured.

- For network-side performance monitoring in inter-frequency RRM measurement prediction with UE-side model, RRM measurement and report of monitoring resources should be associated with corresponding inference configuration, especially when partial monitoring occasion is configured.

Figure 1: Inter-frequency RRM measurement prediction, time relation between measurement, prediction and the ground truth information. On some occasions the UE can be configured to provide the ground truth information to the network. Hence association of the performance monitoring configuration and the inference configuration is necessary.

In spatial RRM measurement prediction based on partial beam-level RRM measurements, ground truth is cell-level RRM measurement based on all beam-level RRM measurements. In this case, for network-side performance monitoring purpose, network needs to configure UE to measure all beam-level RRM measurements for cell-level RRM measurement results in specific sample time instances (i.e. monitoring windows) aligned with the sample time instances (e.g. prediction windows) for cell-level RRM measurement prediction.

- For network-side performance monitoring in spatial RRM measurement prediction with cell-level or beam level prediction measurement via UE-side model, the performance monitoring configuration should be associated with corresponding inference configuration.

Therefore based on the above observation we propose the following.

- Performance monitoring configuration should be associated/linked with the inference configuration for which the performance monitoring is required

The following figure schematically show the required configuration as part of the performance monitoring configurations.

Figure 2: Schematic view of the content of the performance monitoring configuration. Network should configure the UE with the monitoring window/percentage, monitoring quantity, reporting configuration, and association with the inference config.

Network side performance monitoring for event prediction

For RRM measurement event prediction, in some scenarios, the network may start handover preparation for one UE after receiving RRM measurement event prediction from this UE but may send handover command to the UE for mobility purpose after receiving real measurement event report from the UE. In addition to that, if the network wants to assess the performance of the UEs event prediction functionality, it can configure the UE with mobility thresholds lower than the values that should really trigger the mobility procedures. Besides, the network can ask the UE to trigger a measurement report when the actual event is triggered. This enables the network to monitor the performance of the UE’s AI functionality without creating additional cost for the network.

- One solution for simplifying the network side performance monitoring of event prediction is to configure the UE with events with lower thresholds (not triggering HO), and requesting the UE to send a second report if the actual event occurs at the UE.

However, we are open to hear other companies view on this topic.

- Standardisation impact of the network-side performance monitoring of event predictions is FFS.

    - UE-assisted performance monitoring

In another approach, instead of reporting the ground truth information along with the prediction the UE can be configured with some performance monitoring metrics such as RSRP difference or some statistical metrics (e.g., average RSRP difference over an ensemble of predictions in a period). In such cases, based on the configuration the UE conducts the evaluation of the performance metric and report to the network according to the requested report configuration.

- For UE-assisted performance monitoring in RRM measurement prediction, UE reports performance metric information to network for performance monitoring at the network side.

- For the UE-assisted performance monitoring, study how/when the UE reports the performance monitoring metrics to the network.

#### Performance metrics for the UE-assisted performance monitoring

In this sub-section we propose some performance metrics for the UE-assisted performance monitoring.

In RRM measurement prediction in temporal, frequency and spatial domain except spatial beam-level RRM measurement prediction, inference output is predicted RSRP and thus the performance metric can be short-term RSRP prediction accuracy (e.g. RSRP difference = predicted RSRP –ground truth RSRP) or long-term statistical RSRP prediction accuracy ( e.g. MSE of RSRP difference, mean of RSRP difference, [90%] CDF of RSRP difference) within the monitoring window indicated by performance monitoring configuration.

- Performance metrics for temporal RRM measurement prediction, inter-frequency RRM measurement prediction and spatial RRM measurement prediction with cell-level output via partial beam-level input can be

- Alt 1: RSRP difference

- Alt 2: statistical information of performance metric (e.g. MSE, mean, [90%] CDF of RSRP difference)

For spatial beam-level RRM measurement prediction in AI mobility, the UE may be configured to predict Top K strongest beams in Set A via measuring the beams in Set B for one cell. So the performance metric in this case can include beam prediction accuracy and RSRP prediction accuracy. For example, beam prediction accuracy can be one indication that whether the prediction is an accurate beam prediction or not, or statistical accurate beam prediction information. RSRP prediction accuracy can be the RSRP prediction accuracy of the overlapped beams in the Top M beams of monitoring and the Top K beams of inference.

- Performance metrics for spatial RRM measurement prediction with beam-level output via partial beam-level input can be

- Beam prediction accuracy

  - If at least one of Top M beam(s) of resource set(s) for monitoring is among Top K predicted beam(s) of Set A, where M = (1, 2), K = (1, [2]), the prediction is counted as an accurate beam prediction

- RSRP prediction accuracy of beams which are within both the Top M beams and the Top K beams (FFS)

  - Alt 1: RSRP difference

  - Alt 2: statistical information of performance metric (e.g. MSE, mean, [90%] CDF of RSRP difference)

Regarding indirect RRM measurement event prediction, the predicted RRM measurement event is derived based on predicted RRM measurement results and/or real RRM measurement results according to inference configuration. So RSRP prediction accuracy for measurement event predication can be used as the performance metric for the network to decide whether the related event prediction is reliable. When the UE detects real measurement event for mobility, the UE is expected to report this real measurement event to the network and thus the network knows whether the UE accurately predicts this event or not. Therefore, whether additional performance metric(s) should be introduced for RRM measurement event prediction can be FFS.

- Performance metrics for RRM measurement event prediction can be

- RSRP prediction accuracy for measurement event prediction

  - Alt 1: RSRP difference

  - Alt 2: statistical information of performance metric (e.g. MSE, mean, [90%] CDF of RSRP difference)

where RSRP difference = predicted RSRP – ground truth RSRP

Other performance metrics can be FFS.

    - UE side performance monitoring

During the meeting RAN2#129bis some companies propose to introduce a UE side performance monitoring, wherein the UE can autonomously decide about the performance outcome and may deactivate the model. We think this could be beneficial for the event prediction in particular if the UE realizes the performance of the inference configuration is very bad, instead of performing event prediction and sending the report to the network it can send inapplicability indication to the network.

- For UE-side performance monitoring in RRM measurement/event prediction with UE-side model, UE reports (in)applicability information (based on outcome of performance metric(s)) to network, if configured by the network.

## Toggling between inference configuration and legacy RRM configuration

Another aspect that might require further study is the UE actions upon poor performance by the UE or upon inapplicability of the AIML functionalities. In our understanding if the performance of the AIML functionality is not acceptable or if the model becomes inapplicable there should be a mechanism at the UE to ensure smooth mobility decisions at the network. For example, if the UE is configured with the inference configuration to perform prediction in a target frequency, if the performance of the prediction becomes unacceptable or if the model becomes inapplicable, the UE may need to perform the measurements in that target frequency instead of the measurements.

- UE toggles between inference configuration and RRM measurement configuration if inference shows poor performance or if the functionality becomes inapplicable and vice versa.

## Increasing robustness of event prediction

During the study item we agreed to use indirect event prediction meaning that we reuse the existing RRM measurement predictions in the evaluation of the mobility events e.g., A1-A6 events.

With the introduction of RRM measurement prediction into the mobility event evaluation the event becomes more uncertain (less reliable), particular if the performance of the model is not good enough. Hence it may not be trusted in all scenarios and performing mobility decisions based on such reports may destabilize the network load and leads to additional failures in scenarios such as e.g., handover, DC or CA setup.

Therefore, we think in order to enhance the reliability of the mobility event predictions, running predictions for multiple times before triggering event is necessary. In other words, the UE should be able to repeat the predictions and trigger the prediction-based report only if the predicted event is fulfilled for multiple times. This has been exemplified in the following figure.

Figure 3: UE predicts the mobility event multiple times before reporting to the network. Repetitive prediction enhances the robustness of the prediction and hence increases the reliability of network decisions based on the UE event predictions.

- UE performs event prediction multiple time before triggering report to the network based on the predicted event, if configured by the network.

# Conclusion

Based on the discussion in the previous sections we have the following observations:

- For the temporal prediction Case A network should configures the UE with performance monitoring configuration associated/linked with the inference configuration.

- For temporal prediction case B, network receives prediction of the L3 measurements as part of measurement report, as well as L3 measurements, which act ground truth in the network’s performance monitoring.

- For network-side performance monitoring in intra-frequency temporal RRM measurement prediction case B with UE-side model, performance monitoring configuration need to be associated with corresponding inference configuration.

- For network-side performance monitoring in inter-frequency RRM measurement prediction with UE-side model, RRM measurement and report of monitoring resources should be associated with corresponding inference configuration, especially when partial monitoring occasion is configured.

- For network-side performance monitoring in spatial RRM measurement prediction with cell-level or beam level prediction measurement via UE-side model, the performance monitoring configuration should be associated with corresponding inference configuration.

- One solution for simplifying the network side performance monitoring of event prediction is to configure the UE with events with lower thresholds (not triggering HO), and requesting the UE to send a second report if the actual event occurs at the UE.

Based on the above observations we have the following proposals.

- UE conduct performance monitoring according to the performance monitoring configuration configured by the network.

- Performance monitoring for RRM measurement (and event) prediction with UE-side model is based on the existing RRM measurement and reporting framework.

- Network configures the UE with the performance monitoring window and/or the percentage of the prediction samples for which performance monitoring is needed.

- Network configures the UE with the performance monitoring metric/quantity based on which the UE performs monitoring and sends the monitoring report to network.

- UE reports performance monitoring outcome in periodic or event triggered basis, according to the performance monitoring report configuration configured by the network.

- For the network side performance monitoring, the UE reports ground truth measurements to enable the network monitoring the performance of the UE AI functionalities.

- UE reports L3 measurements as ground truth to the network which are associated to the predictions for the temporal prediction Case B.

- Performance monitoring configuration should be associated/linked with the inference configuration for which the performance monitoring is required

- Standardisation impact of the network-side performance monitoring of event predictions is FFS.

- For UE-assisted performance monitoring in RRM measurement prediction with UE-side model, UE reports performance metric information to network for performance monitoring at the network side.

- For the UE-assisted performance monitoring, study how/when the UE reports the performance monitoring metrics to the network.

- Performance metrics for temporal RRM measurement prediction, inter-frequency RRM measurement prediction and spatial RRM measurement prediction with cell-level output via partial beam-level input can be

Alt 1: RSRP difference

Alt 2: statistical information of performance metric (e.g. MSE, mean, [90%] CDF of RSRP difference)

- Performance metrics for spatial RRM measurement prediction with beam-level output via partial beam-level input can be

Beam prediction accuracy

If at least one of Top M beam(s) of resource set(s) for monitoring is among Top K predicted beam(s) of Set A, where M = (1, 2), K = (1, [2]), the prediction is counted as an accurate beam prediction

RSRP prediction accuracy of beams which are within both the Top M beams and the Top K beams (FFS)

Alt 1: RSRP difference

Alt 2: statistical information of performance metric (e.g. MSE, mean, [90%] CDF of RSRP difference)

- Performance metrics for RRM measurement event prediction can be

RSRP prediction accuracy for measurement event prediction

Alt 1: RSRP difference

Alt 2: statistical information of performance metric (e.g. MSE, mean, [90%] CDF of RSRP difference)

Other performance metrics can be FFS.

- For UE-side performance monitoring in RRM measurement/event prediction with UE-side model, UE reports (in)applicability information (based on outcome of performance metric(s)) to network, if configured by the network.

- UE toggles between inference configuration and RRM measurement configuration if inference shows poor performance or if the functionality becomes inapplicable and vice versa.

- UE performs event prediction multiple time before triggering report to the network based on the predicted event, if configured by the network.