3GPP TSG-RAN WG2 Meeting #130                                    	R2-2504311

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	8.8.4

Source:	Qualcomm Incorporated

Title:	MBS broadcast service continuity

Document for:	Discussion and Decision

# Introduction

In RAN#129bis, following agreements were made on MBS broadcast service.

Agreements:

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

In this document, we provide details for MBS broadcast service continuity.

# Discussion

RAN2 has agreed to broadcast list of intended service area (ISA) in the form of reference location and radius in new SIBxx. In our view, it can be cell specific or may be area specific based on network’s choice or based on availability of information. In moving cell scenario, the cell may broadcast the list of areas where the cell may provide MBS serving in future. The cell may also include list of frequencies where the MBS service is being broadcast. The frequency may be associated with the same satellite or different satellite. Neighbor cell’s information can be obtained during Xn set up procedure.

In our view, UE can already map the MBS broadcast frequency to the associated MBS service through USD (User Service Description) or SIB21. It is to note that the UE needs to be aware of the frequency of the service it is interested in before it can receive the MBS broadcast service. The UE knows the interested frequency either from SIB21 or from USD. Therefore, it may be simple just to map the provided geographical area to one or more MBS broadcast service frequency.

The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

... ,

[[

mbs-CoverageAreaInfoList-r19             MBS-CoverageAreaInfoList-r19       OPTIONAL,  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

MBS-CoverageAreaInfoList-r19 ::=   SEQUENCE (SIZE (1..maxMBS-AreaInfo-r19)) OF MBS-CoverageAreaInfo-r19

MBS-CoverageAreaInfo-r19 ::=       SEQUENCE {

mbs-SessionAreaId-r19		MBS-SessionAreaId-r19,

mbs-FSAI-List-r19           MBS-FSAI-List-r17, 	OPTIONAL,  -- Need R

}

-- TAG-SIB21-STOP

-- ASN1STOP

MBS FSAI (Frequency Selection Area Identity) is another way how the intended service area can be associated with MBS session. SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI. The UE can obtain the FSAI of the MBS service from USD. A frequency can cover multiple MBS service Areas and the MBS FSAI could be different for the different service area. In this case, the service area can be associated with the MBS session simply by FSAI.

Mapping of Area ID and FSAI list

When NG-RAN broadcasts SIB21, it is assumed that NG-RAN has information on mapping each frequency with the FSAI. This is configured by OAM.

Again, when the NG-RAN broadcasts new SIBxx where the list of ISA is broadcast, it can know what are the MBS ISAs where MBS services are intended. RAN3 has agreed that this is configured by AMF during MBS broadcast session setup procedure.

Also during Xn setup, neighbor cells can also exchange the information of FSAIs. This means that the serving cell also has the information of neighbor cell’s broadcast service.

Now a serving cell has all information for one or more MBS broadcast sessions (set up by AMF), i.e., the FSAIs, frequencies, ISAs and neighbor cells who are also serving the same MBS broadcast session (from FSAI).

Serving cell can also provide the neighbor cell information where same MBS sessions may be provided. Therefore, the ISA configured by AFM should include the list of areas applicable also to neighbor cells in same or different frequency and perhaps in different satellite.

As it is clear currently gNBs exchange information on FSAIs for service continuity. However, ISA is new Rel-19 information and it can be discussed whether ISA should also be exchanged between gNBs for service continuity. This may also be configured by OAM.

In new SIBxx, serving cell broadcasts the list of ISAs where neighbour cells in same or different frequency are also providing the same MBS service(s) and neighbor cell may belong to different satellite.

Same as FSAIs, discuss whether ISA should also be exchanged between gNBs for service continuity.

# Conclusion

Following proposals are made.

Observation 1.	The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

Proposal 1	In new SIBxx, serving cell broadcasts the list of ISAs where neighbour cells in same or different frequency are also providing the same MBS service(s) and neighbor cell may belong to different satellite.

Proposal 2	Same as FSAIs, discuss whether ISA should also be exchanged between gNBs for service continuity.