3GPP TSG-RAN WG2 Meeting #130                                    	R2-2504317

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	8.9.2

Source:	Qualcomm Incorporated

Title:	Switching of S&F mode

Document for:	Discussion and Decision

- Introduction

In RAN2#129 meeting, following agreements were made.

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In RAN2#129bis, following agreements were agreed.

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In this document, we provide further details for solution of switching S&F mode and normal mode.

- Discussion

It is already agreed that switching indication in both direction between S&F mode and normal mode will be provided in SIB, for example UE’s application can make better decision on the use of traffic types, e.g., time sensing traffic can wait until the switch time to normal mode.

Currently the t-Service (cell stop time) is provided in SIB3. The switching time of S&F mode to normal mode can be exactly same as t-Service time, for example, MME over satellite changes or feeder link info (e.g., common TA, Kmac) resulting in cell change. A Signalling optimization can be done by just providing an indication (i.e., 1 bit) instead of broadcasting TimeOffsetUTC switch time in SIB31 (i.e., 20 bits).

If t-Service is equal to switching time of S&F mode to normal mode, broadcast 1 bit indication instead of 20bits of TimeOffsetUTC.

Whenever the satellite loses feeder link connection, cell supporting S&F mode should make sure that S&F indication is included in SIB. In this case, the cell can release all UEs that do not support S&F mode operation in RRC_CONNECTED to IDLE mode. In this case, the cell may also be releasing the legacy UEs.

The SIB may need to be updated to provide updated information of feeder link or S&F mode indication and next switch time. We think it is very important to avoid paging notification to eMTC and NB-IoT UEs. The existing cell stop time mechanism can be used to inform the legacy UEs that they need to find other cells. This can be useful for both cases, i.e., when feeder link will be lost or when feeder link will be available.

For new UEs, a new indication (probably in SIB31) can be introduced to indicate that the cell stop time is associated with the update of S&F mode operation, i.e., at cell stop time, the cell loses feeder link and operates in S&F mode.

For switching time from normal mode to S&F mode, the value of existing cell stop time (t-Service) is configured to the switching time.

For legacy UEs, the existing behavior of t-Service applies when normal mode is switched to S&F mode at t-Service.

For new UEs, S&F indication (1 bit indication as in Proposal 2) is used to associate the existing cell stop time with the switching time from normal mode to S&F mode.

Upon mode switch, discuss how to update the system information without paging if barring bit or S&F mode indication or presence of common TA is changed.

Cell selection/reselection

SA2 has sent response LS in [1] and has specified following in TS 23.401.

NOTE 6:	The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.

NOTE 7:	How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

In our understanding, even if UE has a list of satellites provided, the UE can still choose to go to other satellites. This confirms the RAN2 understanding:

The UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and

communicate with a satellite which is not included in the MME-configured satellite list.

For NB-IoT, cell reselection is based on the ranking of neighbor cells (i.e., frequency priorities are not considered). When the UE is in IDLE mode, it may find both S&F mode cells and normal mode cells. The UE may want to connect to the normal mode cell even if the ranking of S&F mode cell is better. When the UE is waiting for the satellite included in list, it may have UL data to transmit and there can be other satellites or TN cells available.

Therefore, we think if the UE has information on the normal mode cells, UE should be allowed to prioritize normal mode cell over S&F mode cell.

When connected to S&F mode cell, it can be left to UE in RRC_IDLE to periodically search for better normal mode cell (i.e., cells with feeder link connection available).

When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

Paging enhancement

There is also a discussion whether S&F mode cell has to explicitly indicate to UE not to monitor paging. In our understanding, the MME on-board can provide the UE with the list of satellites the UE can connected to and also the wait timer, i.e., how long to wait to connect to those satellites. If UE wants, it can go to any other satellite. But if it wants to stick to the same list of satellites then it can also go to sleep until time expires and save power. In addition, any change in paging enhancements requires involvement of other working groups. Therefore, we suggest not to do any enhancement on paging monitoring.

Wait timer configured by MME is sufficient for UE power saving and there is no need to add enhancements for paging monitoring.

UE capability

In store and forward case, the UE capability is reported to MME on board via NAS message. The UE capability may be needed for eNB on-board to determine few things such as triggering release procedure, neighbor cell configuration for connected mode measurement for eMTC, carrier redirection with release, logged measurement configuration etc. However, the eNB and MME are collocated on satellite, it may be sufficient that MME triggers release procedure of UEs not supporting S&F when feeder link is going to be lost. For this purpose, eNB does not need to know UE’s capability.

If eNB needs to know the UE capability for other purposes, the collocated MME could also inform eNB (e.g., with proper S1 interface or via implementation). We think first we need to decide whether eNB needs to know UE’s capability and check with RAN3 if this can be done via S1 interface.

Decide whether eNB needs to know the UE’s store and forward capability.

- Conclusion

Following observation and proposals are made:

Proposal 1	If t-Service is equal to switching time of S&F mode to normal mode, broadcast 1 bit indication instead of 20bits of TimeOffsetUTC.

Proposal 2	For switching time from normal mode to S&F mode, the value of existing cell stop time (t-Service) is configured to the switching time.

Proposal 3	For legacy UEs, the existing behavior of t-Service applies when normal mode is switched to S&F mode at t-Service.

Proposal 4	For new UEs, S&F indication (1 bit indication as in Proposal 2) is used to associate the existing cell stop time with the switching time from normal mode to S&F mode.

Proposal 5	Upon mode switch, discuss how to update the system information without paging if barring bit or S&F mode indication or presence of common TA is changed.

Proposal 6	When connected to S&F mode cell, it can be left to UE in RRC_IDLE to periodically search for better normal mode cell (i.e., cells with feeder link connection available).

Proposal 7	When in RRC_IDLE and waiting to connect to a S&F mode satellite, it can be left to UE to prioritize the search for better and normal mode cell (i.e., cells with feeder link connection available).

Proposal 8	Wait timer configured by MME is sufficient for UE power saving and there is no need to add enhancements for paging monitoring.

Proposal 9	Decide whether eNB needs to know the UE’s store and forward capability.

- Reference

[1] R2-2501753, “Reply LS on Satellite IDs for store-and-forward operation”, From SA2, To RAN2