3GPP TSG-RAN WG2 Meeting #130                                    	R2-2504318

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	CB-Msg3-EDT and Msg4 multicast

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis, following agreements were made.

Agreements;

1.	Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

2.	We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

3.	For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same).

4.	We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case)

5.	We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation.

6.	If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

7.	The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

8.	The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

9.	RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

(CB-Msg3-EDT configuration for eMTC)

10.	For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2.

11.	For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed).

12.	We will not support TDD related parameters.

13.	For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources).

14.	For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration

15.	For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters.

(CB-Msg3 configuration for NB-IoT)

16.	For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier.

17.	For NB-IoT, introduce below physical layer parameters in shared resources configuration as below:

-	Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16)

-	Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16)

-	Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set.

-	MCS configuration for NPUSCH (as in npusch-MCS-r16).

-	PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13)

-	The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used.

NOTE: confirm with RAN1 is needed

18.	For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

Agreements (part 2):

1. For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

2.	The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

3.	The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

4.	FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

In this document, we provide details for DSA solution.

- Discussion

## OCC

Following is agreed in RAN2#129 meeting.

- If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

We think OCC is important to improve further by multiplexing two users. There are some concerns raised regarding the impact of power imbalance on OCC in context of using OCC with CB-msg3. The issue is that if 2 UEs doing OCC have severe power imbalance – the performance of OCC will be impacted. We believe that is not true since the UEs will still be orthogonal. For example, let’s say UE 1 uses codeword [1 1] and let’s say UE 2 has a power imbalance of 10 dB, which can be considered as that UE 2 is using codeword [√10 -√10] with respect to UE 1. However, the codewords [1 1] and codeword [10  -√10]are still orthogonal. To verify this assumption, we perform some link level simulations with power imbalance.

We assume a power imbalance of 10 dB between 2 UEs i.e., UE 2 has a power boost of 10 dB more power than UE1. We assume TDM DMRS for both UEs, since it has been agreed as a working assumption already. Also note that TDM DMRS won’t be impacted by power imbalance. We concentrate on the performance of UE1 (with no power boost). The performance of UE1 with and without power imbalance in UE2 has been shown in Fig. Y.

Figure 1Comparison between no power imbalance and power imbalance on UE 1 for OCC2 with TDM DMRS and 3.75 kHz SCS, 4 RUs, 2 Repetitions, TBSize = 104 bits, Channel = NTN-TDL-C Rural 10 degree elevation with 5 Hz doppler

It can be observed from Fig. 1 that the loss due to power imbalance is minimal. If we observe the SNRs required to achieve 10% BLER, we see that there is only a 0.2 dB loss between the no power imbalance case and the 10 dB power imbalance case.

Even a power imbalance of 10 dB results in only a performance degradation that is minimal i.e., 0.2 dB for NPUSCH with OCC.

Based on the above discussion, we see that power imbalance is not a problem when it comes to NPUSCH with OCC. Therefore, power imbalance will not be impacting system performance if it uses OCC in NPUSCH CB-msg3 and OCC can be supported as an important feature for UL capacity enhancement.

Confirm CB-Msg3 transmission using OCC is supported.

## Transmission window configuration

As explained in [1], for DSA, the network should configure a number of replicas (i.e., K) allowed for transmission. There could be N EDT occasions configured for UE to choose randomly, and UE selects randomly K occasions (i.e., total number of Msg3 and its replica transmissions) out of N occasions. If K = 1, then UE will have only one PUSCH resource to select. If K = 2, then UE will choose randomly 2 PUSCH resources in different time occasions and transmit one original msg3 transmission and one replica of the Msg3 transmission.

Following is agreed.

- Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT

- We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure

In our view, this would mean when the network configures, K = 1 and N = 1, the size of window is same as on time domain transmission occasion duration, i.e., it can be assumed there is no transmission window. We also have following agreements:

- The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource.

- The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same.

PUSCH configuration

Defining periodic PUSCH occasions and then grouping consecutive PUSCH occasions into a CB-Msg3-EDT transmission window will lead to long latency for the UE to complete the CB-Msg3-EDT procedure.

For example in figure 2, if the network can allocate only 4 PUSCH occasions per second, having periodic PUSCH occasions will result in 4 PUSCH occasions starting at 0ms, 250ms, 500ms and 750ms. Assuming N=2 occasions within a window, the first two occasions belong to window 1 while the next two belong to window 2. Let us also assume K=2 is configured.

Figure 2 An example of CB Msg3 (DSA) window with larger PUSCH periodicity

A UE transmitting in Window 1 will transmit in TO#1, wait until start of TO#2, transmit in TO#2 and start the response window after the end of TO#2 plus RTT. So the earliest time the UE can receive success/failure response in this case is 250ms + PUSCH duration + RTT.

To reduce this latency the PUSCH peridiocity can be reduced. However this leads to increase amount of resources allocated for PUSCH occasions.

Instead, if the PUSCH occasions within a window are packed closer together (optimally back-to-back) the latency can be decreased without increased resource utilization.

Figure 3 An example of CB Msg3 (DSA) window with shorter PUSCH periodicity

The earliest time the UE can receive a success/failure response in this example in figure 3 would be 80ms + PUSCH duration + RTT.

We think network should have option to configure the CB-Msg3-EDT transmission window efficiently optimizing the resource, latency and probability of success. For example, there can be gap between two transmission windows and there should be no PUSCH resources configured in the gap. In our view, the cleaner configuration is to introduce following parameters:

- Transmission window start time

- PUSCH time occasion periodicity within transmission window or PUSCH occasion offsets from the start of the window or PUSCH occasion offsets from end of the previous PUSCH occasion within the window

- Number of replicas within transmission window

- Number of PUSCH occasions within transmission window

- Transmission window periodicity (e.g., values X SFN)

The transmission window configuration includes start time, periodicity and within transmission window includes number of replicas, number of PUSCH occasions and periodicity of PUSCH occasions.

Msg4 monitoring window start

Followings are agreement from the RAN2#129bis.

- The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

- FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

In our view, there is no need for defining two behaviors on monitoring Msg4, i.e., sometimes at the end of window and sometimes at the end of first replica depending on network configuration. With monitoring window starting after the first replica transmission, there is a chance that monitoring window overlaps with the replica transmission. This is not efficient for half duplex UEs such as NB-IoT.

However, for full duplex UEs such as in eMTC, it may be ok to start the Msg4 monitoring window after the first replica transmission. It may be cleaner to define the two different behavior for NB-IoT and eMTC. For eMTC half-duplex UEs, however, the transmission of replica should have higher priority than monitoring Msg4.

For NB-IoT, the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT.

For eMTC, the Msg4 monitoring starts at the end of first replica of CB-Msg3-EDT transmission plus UE-eNB RTT.

For eMTC half-duplex, the transmission of replica has higher priority than monitoring Msg4 in case of overlapping.

RNTI calculation

The set of RA-RNTI used is already defined in 36.321. The RNTI values used for reception of PDCCH of CB-Msg4 should be defined such that it does not overlap with the RA-RNTI values.

eMTC UEs

For BL UEs and UEs in enhanced coverage, RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI = 1+t_id+10*f_id+60*(SFN_id mod (Wmax/10))

where t_id is the index of the first subframe of the specified PRACH (0≤ t_id <10), f_id is the index of the specified PRACH within that subframe, in ascending order of frequency domain (0≤ f_id< 6), SFN_id is the index of the first radio frame of the specified PRACH, and Wmax is 400, maximum possible RAR window size in subframes for BL UEs or UEs in enhanced coverage.

However, for CB-msg3-EDT, we agreed that we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). The new CB RNTI can be derived from the SFN index of the start of the Msg3 transmission window. In addition, to offset the RA-RNTI range to avoid potential RNTI collision, we think a RA-RNTI offset should also be included. For example,

CB-RNTI = 1+ SFN_id mod (Cmax/10)+offset

where SFN_id is the index of the first radio frame of the specified CB-Msg3 window, and Cmax is the maximum possible CB-Msg4 Contention resolution window size in subframes for BL UEs or UEs in enhanced coverage. Offset is defined as the largest value that RA-RNTI can take i.e. 1+9+10*5+60*39.

NB-IoT UEs

For NB-IoT UEs, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI = 1+floor(SFN_id/4) + 256*carrier_id

where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0.

Similarly, we can define the CB-Msg4-RNTI associated with a CB-Msg3 window in which the CB-Msg3 is transmitted as

CB-RNTI = 1 + floor(SFN_id/M) + (1024/M)*carrier_id + offset

where SFN_id is the index of the first radio frame of the specified CB-Msg3 window and carrier_id is the index of the UL carrier associated with the specified CB-Msg3. The carrier_id of the anchor carrier is 0. M can be set to the minimum periodicity in radio frames allowed for CB-Msg3 window. Offset is defined as the largest value that RA-RNTI can take i.e. 1+255+256*MaxNumberOfCarriers.

For eMTC, the CB-RNTI is derived as SFN_id mod (Cmax/10)+offset, where SFN_id is the index of the first radio frame of the specified CB-Msg3 window, Cmax is the maximum possible CB-Msg4 Contention resolution window size in subframes and Offset is defined as the largest value that RA-RNTI can take.

For NB-IoT, the CB-RNTI is derived as 1 + floor(SFN_id/M) + (1024/M)*carrier_id + offset where SFN_id is the index of the first radio frame of the specified CB-Msg3 window and carrier_id is the index of the UL carrier associated with the specified CB-Msg3, M is the periodicity in radio frames allowed for CB-Msg3 window and Offset is defined as the largest value that RA-RNTI can take.

## Msg4 Response formats

It is agreed that we adopt a Single Msg4 monitoring window and Single RNTI. The Msg4 response with single RNTI should be able to address multiple UEs. This needs to be discussed how to multiplex the multiple UEs and what are the response types supported, for example, response to send the UE to RRC_IDLE, response to keep UE monitoring further DL message (or moving to RRC_CONNECTED) or providing retransmission grant.

RAN2 has also agreed that HARQ operation for Msg4 response is supported. The network may need to be aware that all UEs successfully received the response, i.e., only the positive HARQ feedback may be sufficient. The negative HARQ feedback can be implicit upon expiry of the window. Mostly we believe the multiplexing techniques, UE specific HARQ feedback resource and response formats defined for NR 2 step RACH can reused. The MsgB formats include followings:

- a MAC subheader with Backoff Indicator only;

- a MAC subheader and fallbackRAR;

- a MAC subheader and successRAR;

- a MAC subheader and MAC SDU for CCCH, DCCH or DTCH;

- a MAC subheader and padding.

Figure 4 An example of using 2step RACH formats and multiplexing techniques.

Success response:

The success response should include the contention resolution ID of the successfully decoded Msg3 indicating UE that CB-Msg3 EDT procedure is successful. It should also include the HARQ feedback resource and timing advance when HARQ feedback is enabled. There should also be an option to include the RRC message (additionally DL data for UP solution).

When RRC message is not present, there should be additional indication whether UE is allowed to move to RRC_IDLE or keep monitoring further DL RRC message. If the UE is kept to further monitor DL message, a new temporary C-RNTI should also be provided.

Msg3 retransmission grant

We believe the retransmission of Msg3 EDT can be supported but it is not necessary to support the retransmission of all replicas. Since the retransmission grant can be specific to the Msg3 transmission location (time and frequency), just single replica retransmission can be supported.

In this case, same as in fallbackRAR in 2 step RACH, the response can include the UL grant field, the Msg3 transmission location (time and frequency indicator) as in RAPID, timing advance and a temporary C-RNTI.

Therefore, we think first NR 2 step RACH should be reused for Msg4 response formats and multiplexing response of multiple UEs. For CB-Msg3 EDT, at least the success response sending UE to RRC_IDLE (with or without DL data), success response waiting further DL data, success response sending UE to RRC_CONNECTED, response with retry and backoff indication should be supported.

For CB-Msg3 EDT, NR 2 step RACH mechanisms should be reused for Msg4 response formats and multiplexing responses of multiple UEs.

For CB-Msg3 EDT, support the formats for (1) the success response sending UE to RRC_IDLE (with or without DL data), success response waiting further DL data, and success response sending UE to RRC_CONNECTED, (2) response with retry and (3) backoff indication.

It can also be discussed whether the retransmission TBS size is allowed to smaller than the first/original transmission TBS size. It can be same as EDT where the grant size can be small enough to fit the RRC request message without UL data. This can be considered as non-EDT resource. This means for CP solution, the CB-EDT is considered failure and the UE starts with normal RRC reestablishment procedure. For UP solution, the UE just updates the Msg3, i.e., demultiplexing the UL data logical channel and sending only RRC message.

However, this can be discussed whether fallback to non EDT grant should be supported for CB-Msg3 EDT. In EDT, the first UL grant is dynamic and can be non-EDT grant in RAR. However, the first UL grant for CB-Msg3 EDT is preconfigured resource and providing non-EDT grant for retransmission may not need to be considered.

Same as in EDT, discuss whether to allow network sending UE non-EDT UL grant (i.e., asking just to send RRC establishment request message without UL data) while providing CB-Msg3 EDT retransmission UL grant.

The Msg4 enhancements can be on the top of existing 4 steps EDT procedure as shown in figure 2, or applicable for the case of CB-msg3 / DSA.

The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

- Conclusion

Following proposals are made:

Proposal 1	Confirm CB-Msg3 transmission using OCC is supported.

Proposal 2	The transmission window configuration includes start time, periodicity and within transmission window includes number of replicas, number of PUSCH occasions and periodicity of PUSCH occasions.

Proposal 3	For NB-IoT, the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT.

Proposal 4	For eMTC, the Msg4 monitoring starts at the end of first replica of CB-Msg3-EDT transmission plus UE-eNB RTT.

Proposal 5	For eMTC half-duplex, the transmission of replica has higher priority than monitoring Msg4 in case of overlapping.

Proposal 6	For eMTC, the CB-RNTI is derived as SFN_id mod (Cmax/10)+offset, where SFN_id is the index of the first radio frame of the specified CB-Msg3 window, Cmax is the maximum possible CB-Msg4 Contention resolution window size in subframes and Offset is defined as the largest value that RA-RNTI can take.

Proposal 7	For NB-IoT, the CB-RNTI is derived as 1 + floor(SFN_id/M) + (1024/M)*carrier_id + offset where SFN_id is the index of the first radio frame of the specified CB-Msg3 window and carrier_id is the index of the UL carrier associated with the specified CB-Msg3, M is the periodicity in radio frames allowed for CB-Msg3 window and Offset is defined as the largest value that RA-RNTI can take.

Proposal 8	For CB-Msg3 EDT, NR 2 step RACH mechanisms should be reused for Msg4 response formats and multiplexing responses of multiple UEs.

Proposal 9	For CB-Msg3 EDT, support the formats for (1) the success response sending UE to RRC_IDLE (with or without DL data), success response waiting further DL data, and success response sending UE to RRC_CONNECTED, (2) response with retry and (3) backoff indication.

Proposal 10	Same as in EDT, discuss whether to allow network sending UE non-EDT UL grant (i.e., asking just to send RRC establishment request message without UL data) while providing CB-Msg3 EDT retransmission UL grant.

Proposal 11	The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

- References

[1] R2-2500534, “CB-Msg3 and Msg4 enhancements”, Athens, Greece, February 17th – 21st, 2025.