3GPP TSG-RAN WG2 Meeting #R2-130		        R2-2504329      St Julians, Malta, May 19-23, 2025

Agenda Item:	8.6.4

Source: 	Rakuten

Title:  	Remaining issues of Conditional LTM

WID/SID:           NR_Mob_Ph4-Core- Release 19

Document for:	Discussion, Decision

# Introduction

In RAN #105, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1], where the objectives relevant to conditional LTM (CLTM) are described below:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the LTM cell-switch command from the serving gNB-DU, which may lead to radio link failure. The conditional LTM is expected to bring the benefits of robustness while retaining the interruption time reduction benefits of LTM.

In this contribution, we discuss remaining issues of CLTM.

# Discussion

## RACH-based TA acquisition during CLTM

One of the important advantages of LTM in R18 is the reduction of service interruption during HO, due to RACH-less cell switch. It was agreed in one of the previous meetings to support RACH-less CLTM, and at least PDCCH ordered early TA acquisition.

A UE configured with CLTM performs a cell switch without receiving a cell switch command, when the CLTM execution condition of a candidate cell is satisfied. Therefore, the existing mechanism of TA information transfer to the UE in the MAC CE cell switch command is not feasible in CLTM and an alternate mechanism is required for delivering TA for RACH-less CLTM cell switches. TA delivery using random-access response (RAR) to the UE was was ruled out for non-conditional intra- and inter-CU LTM and left as FFS for conditional LTM.

In the case of PDCCH ordered RACH based TA acquisition, one solution is that the TA could be provided by the source gNB-DU in a MAC CE, similar to R18.

While this is some improvement compared to explicitly triggered LTM cell switch, there is still some dependency on the source gNB-DU to assist the cell switch i.e an event triggered UL MAC CE to report L1 measurements, and a DL MAC CE to provide TA, before the CLTM.

To overcome this signaling and latency with the source gNB-DU, a conditional early UL sync could be employed, where the TA value can be obtained based on a new execution condition (ahead of cell switching), therefore reducing the amount of time needed during cell switch itself. It also ensures that the RACH for TA acquisition is performed without the event triggered L1 measurement report to the source gNB-DU in UL and PDCCH order in DL. The early UL sync and the cell switch could therefore be performed completely based on execution conditions being satisfied at the UE.

For this conditional early UL sync, a dedicated RACH preamble may have to be allocated at the time of CLTM target cell preparation, which enables the target gNB-DU to identify the UE’s objective of performing RACH.

Together with this, the conditional trigger for performing RACH would need to be set such that the condition is met earlier than the condition for performing CLTM cell switch.

A conditional uplink sync could be perfectly complimented by delivering the TA directly to the UE using RAR.

RAR-based TA acquisition directly from target gNB-DU is supported for conditional LTM.

Direct RAR based delivery of TA was also discussed in Rel 18 LTM and the main drawback identified was that, depending on UE capability and cell frequency, it may lead to increased interruption time at the source cell, since the UE must monitor the PDCCH of the candidate cell and may not be able to receive data from the source while monitoring for RAR.

In addition, delivery of target gNB-DU RAR via the source gNB-DU is fully compatible with the Rel-18 early UL synchronization procedure and very minimal changes are required to support the inclusion of RAR over F1.

Therefore, to address the requirements of UEs which don’t have the capability of monitor PDCCH of the target cell without its serving cell data transmission getting impacted, we propose delivery of RAR via the serving gNB-DU. This could be delivered by having RAR as a payload in a MAC CE.

Delivery of target gNB-DU RAR to the UE via the serving gNB-DU is supported.

## Conditional TA acquisition during CLTM

In one of the previous meetings, it was agreed to support explicit triggering of early UL sync (using PDCCH order for TA acquisition) and early DL sync (Rel-18 Early candidate TCI State activation/deactivation).

As explained above, to reduce the signaling and latency overhead with the source gNB-DU, a conditional early UL sync could be configured at the UE, where the RACH based TA acquisition could be performed based on a new execution condition.

The CLTM execution condition could be a trigger to execute LTM cell switch or it could be a trigger for performing early DL/UL sync (e.g. activate TCI state on candidate cell, perform a RACH procedure for TA acquisition).

In summary, it should be possible to configure an execution condition (e.g. beam-based, cell-based threshold) to perform an action (cell switch, UL sync, DL sync).

In L3 measurements based CLTM, the decision to trigger early DL and UL sync are triggered based on L3 measurements and this involves additional signaling from gNB-CU to gNB-DU, as the early UL/DL sync is initiated by the gNB-DU. Conditional execution based early UL/DL sync avoids this overhead.

Conditional execution i.e triggered based on an execution condition, of the following is supported for both L1 measurements based CLTM and L3 measurements based CLTM

- CLTM cell switch

- Early DL sync (TCI state activation)

- Early UL sync (RACH based for TA acquisition).

## CLTM cell switch notification to the gNB-CU

In Rel 18 LTM, the gNB-DU notifies the gNB-CU after the LTM cell switch command is delivered to the UE. This ensures that the gNB-CU is always aware of the UE’s current serving cell.

In CLTM, the gNB-DU doesn’t deliver a cell switch command, instead the CLTM cell switch is based on CLTM execution condition. Hence, we there is a need to address this issue using an alternative mechanism.

The UE may inform the source gNB-DU of the selected CLTM target cell right when the CLTM execution condition is satisfied, but this may not be reliable as there could be a too-late HO and the message may not successfully reach the source gNB-DU.

Hence, we propose that the UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch. The target gNB-DU can send the same to the gNB-CU, using which the gNB-CU can identify the UE’s current serving cell.

UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch.

## CLTM for a MIMO 2TA UE

In RAN2#129bis made the below agreements for MIMO 2 TA.

RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

When a UE is configured with mTRP and with C-LTM, we have the following.

- There are at least two TRPs configured for the UE, which will have 2TAs corresponding to each TRP.

- The TRPs could belong to different cells, which could belong to same or different gNB-DU, but the same gNB.

- The UE can be configured with one or more C-LTM candidate cells coupled with a CLTM execution condition.

- The C-LTM candidate cells and the execution condition are TRP specific i.e candidate cells and execution condition of one TRP aren’t applicable to the other.

In CLTM, since there is no gNB control over the cell switch execution, there is a possibility where only one of the TRPs satisfy the CLTM execution condition, while the other does not. There are the following feasible alternatives when only one of the TRPs satisfy the CLTM execution condition.

- The UE performs RACH-less C-LTM cell switch for only that TRP. The other TRPs are retained with their serving cell, until either of the following occurs

  - TRP execution condition(s) is satisfied, and C-LTM cell switch can be executed.

  - UE detects RLF for the TRP and the TRP is autonomously discarded by the UE. No RRC re-establishment is performed after RLF detection as there is another TRP active. TRP recovery can be attempted, if configured and feasible.

- One of the TRPs is designated as the master TRP by the gNB and the UE performs C-LTM cell switch only if the execution condition(s) of the master TRP is satisfied.

- C-LTM cell switch is performed only when both TRPs have the execution condition(s) are satisfied.

- C-LTM candidates are never prepared with mTRP (only prepared with a single TRP). If C-LTM execution condition is satisfied for one TRP at the source, the UE performs C-LTM cell switch. The network may configure mTRP again after a successful C-LTM cell switch.

RAN2 discusses and agrees a way forward CLTM handling of a mTRP UE.

# Conclusion

We have the following proposals:

Proposal 1	RAR-based TA acquisition directly from target gNB-DU is supported for conditional LTM.RAR-based TA acquisition directly from target gNB-DU is supported for conditional LTM.

Proposal 2	Delivery of target gNB-DU RAR to the UE via the serving gNB-DU is supported.

Proposal 3	Conditional execution i.e triggered based on an execution condition, of the following is supported for both L1 measurements based CLTM and L3 measurements based CLTM

Proposal 4	UE informs the target gNB-DU of the target cell configuration index sent over RRC, after a successful CLTM cell switch.

Proposal 5	RAN2 discusses and agrees a way forward CLTM handling of a mTRP UE.

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes