3GPP TSG-RAN WG2 Meeting #130	R2-2504343

# St. Julians, Malta, 19th – 23rd May 2025

Agenda Item:	8.7.6

Source:	Ericsson

Title:	More Views on XR Rate Control

Document for:	Discussion, Decision

# 1	Introduction

On the issue of per QoS flow vs per DRB level granularity for the MAC CE, RAN3 will continue discussing the two options and evaluate their corresponding F1 impacts. Although in the last meeting it was captured that the QoS flow in MAC CE is preferred, upon further evaluation we see that this solution is complex and is not applicable to all configurations, architectures/network deployments. In comparison, the RRC-based solution with DRB-level granularity for the MAC CE is a simpler, cleaner solution and works in all configurations, architectures/network deployments.

In UE capabilities, we believe that the current radio access capability for support of XR rate control is not sufficient in view of the CN providing information of which QoS flows are rate adaptable.

Thus, in this contribution, we provide our analysis of the two discussed solutions, discuss UE capabilities and address the FFS from the last meeting.

# 2	Discussion

## 2.1 	QoS flow in MAC CE vs RRC-based Solution

### 2.1.1	Bit Rate Monitoring (per-QoS flow vs per-DRB level)

From discussions in the last RAN3 meeting in Wuhan, there was consensus among companies in RAN3 that the per-QoS flow data bit rate monitoring is only possible at the CU. Further, this is under the assumption that the CU can use the information in the SDAP-header to perform the bit rate monitoring. Hence, it is now a common understanding that only the CU can perform a per-QoS flow monitoring.

- Only the gNB-CU can perform per-QoS flow monitoring under the assumption of using the information in the SDAP header.

However, it should be noted that the SDAP header is configurable by the network and it not always present. The network configures the SDAP based on the SDAP-Config and the corresponding field description are as shown below:

As shown in the highlighted part, the SDAP header in the UL is only present if the defaultDRB field is set to true. So at least from a specification perspective, if the defaultDRB is not established, then the gNB-CU cannot perform per-QoS flow monitoring.

- The gNB-CU cannot perform per-QoS flow monitoring if the SDAP header is not present.

In addition, although such per-QoS flow monitoring is up to network implementation putting this additional requirement on the CU has the risk that not all implementations can support it.

- Putting additional requirements (i.e., per-QoS flow bit rate monitoring) on the CU has the risk that not all implementations can or will support it.

In contrast, the per-DRB level monitoring is feasible and already possible in current implementation independent of the configuration or the network architecture or deployment.

- DRB-level bit rate monitoring is feasible with all possible configurations and is architecture and deployment independent.

### 2.1.2	F1 impacts for CU-DU architecture

The end-to-end signaling of the QoS flow in MAC CE is illustrated in our companion RAN3 contribution [2]. We can see that there is much more impact on the F1 for the QoS flow in MAC CE solution. This is because the CU would need to measure the per-QoS flow data rate and inform the DU about the QoS flows that need to be rate adapted (Step-7/8 as shown in [2]). It is also unclear whether this information needs to be provided periodically or is a one-shot configuration.

In comparison, for the RRC-based solution, the CU would provide a configuration to the DU, a configuration to the UE and use the DRB-level MAC CE to provide the recommended bit rate.

- There is much more F1 signaling load for the QoS flow in MAC CE solution when comparing the RRC-based solution.

### 2.1.3	Flexibility

As mentioned, for the QoS flow in MAC CE solution, it is unclear whether the indication carrying the per-QoS flow data rate to the DU needs to be provided periodically or on a one-shot basis. If on a one-shot basis, then there is no difference to the RRC-based solution with DRB-level MAC CE in terms of flexibility.

- If the QoS flow data rate indication from the CU to the DU is to be provided on a one-shot basis, then this is no different from the RRC-based solution with DRB-level MAC CE.

But if periodic, as explained in [2], the F1 delays can be quite large resulting in stale data rate information being available at the DU thereby rendering the solution less flexible than expected. There could be further F1 delays when considering that the DU would need to inform the CU of its congestion level to receive the QoS flow data rate.

- Due to the F1 delays, the QoS flow data rate information (including the QoS flows that need to be adapted) provided to the DU can be stale, rendering the solution less flexible. If the DU needs to inform the CU of its congestion level to receive the QoS flow data rate information, this further exacerbates the F1 delays.

In addition, the intention to trigger this MAC CE (in DL to the UE) is to relieve congestion at the network as soon as possible by perform rate adaptation in the UL. Using a MAC CE is advantageous and as they originate and terminate at the MAC-layer (with no cross-layer interactions) and the signaling is fast. However, to include the QoS flow information in the MAC CE requires much more cross-layer interactions leading to significant delays in triggering the MAC CE, thereby losing the speed advantage.

- The speed advantage of a MAC CE is diminished in the QoS flow in MAC CE solution considering cross-layer interactions or F1 delays for higher layers to provide additional information, such as the QFI.

On the aspect of flexibility, the RRC-based solution does indeed require an RRCReconfiguration message to update the QoS information or the ratios in the UE. However, given that the RAN is aware of the QoS flow to DRB mapping and the related QoS information, the RAN can make a good “estimation” of the data based on this mapping and provide a reliable configuration to the UE which need not be updated during the length of a PDU session.

- By utilizing the QoS flow to DRB mapping and the associated QoS information, the RAN can accurately estimate the per-QoS flow data rate and deliver a reliable configuration to the UE that remains valid throughout the entire duration of a PDU session.

We can also consider making the RRC-based solution more flexible by including the ratio of the data rate to be split among the configured QoS flows as part of the MAC CE.

### 2.1.4	Summary

The two solutions (QoS flow in MAC CE and RRC-based with DRB-level MAC CE) were analyzed in different aspects and the following table provides the summary:

|  | RRC-based with DRB-level MAC CE | QoS flow in MAC CE |
| --- | --- | --- |
| Bit rate monitoring | DRB-level monitoring is feasible with all possible configurations and is architecture independent. | QoS flow based bit rate monitoring can only be performed at the CU and relies on the presence of the SDAP header, which is configured by the network and is not always present. |
| Additional Requirements in CU/Higher layers | No such requirements. | Additional requirement to perform per-QoS flow bit rate monitoring in the CU, there is a risk that not all implementations can support it. |
| F1 Impact | Smaller F1 signaling load, only requires the indication of the rate adaptive DRB ID. | Large F1 signaling load due to periodic updates of the per-QoS flow data rate from CU to DU. |
| Flexibility | Less flexible to begin with as changes would require RRCReconfiguration. Can be made more flexible by indication the ratio of data rate split in the MAC CE. | More flexible, but F1 delays or implementation of cross layer interactions can make it less flexible. |
| MAC CE trigger/transmission delays | Small, all information originates in the DU/MAC. Can be transmitted asap. | Large, as the DU/MAC would need to wait for information from the higher layers through F1 delays/implementation of cross-layer interactions. |
| Specification Impact | Apart from the MAC CE itself, only requires configuration of the QoS flow information and bit rate ration in the RRC. | Apart from the MAC CE itself, much more spec impact especially in RAN3 in terms of F1 and the indication of QoS flow data rate information. Additionally requiring the introduction of QoS flow information in the MAC changing clause 12 in 38.300 |
| Overall Solution | Much cleaner and simpler solution with no dependencies!!! | Too many moving parts to the solution and is complex. |

Overall, we believe that the RRC-based solution with DRB-level MAC CE is a cleaner and simpler solution both from a specification perspective and from a system/implementation perspective. Thus, RAN2 should also pursue the same.

The gNB is allowed to configure one or more QoS flows (mapped to a DRB) to be rate adapted.

For rate adaptation of more than one QoS flow within a DRB, consider three options:

  - Up to UE implementation how to perform rate adaption between the configured QoS flows.

  - gNB indicates the ratio of rate adaption between the QoS flows as part of the configuration providing rate adaptability of one or more QoS flows.

  - gNB indicates the ratio of rate adaption between the QoS flows in the MAC CE itself.

## 2.2	MAC CE Design

For the design of the MAC CE, independent of the granularity discussion from above, we believe that the LCID should be included. This is because the QFI (if agreed) is unique only within a PDU session and the LCID is required to perform the differentiation across the PDU sessions. Thus, we have the following proposal:

XR rate control MAC CE shall at least consist of the LCID.

## 2.3	UE Capabilities

### 2.2.1	CN Awareness of UE Capability

From SA2’s LS reply [1], they have confirmed the CN can indicate to the gNB that a QoS flow is subject to uplink rate control and will discuss further on the whether to reuse existing mechanisms or come up with a new mechanism. It was also agreed in the last meeting that a new radio access UE capability is introduced to indicate the support for the XR rate control MAC CE. Then, the functional representation of the end-to-end XR rate control solution is as follows:

Figure 1: Functional Representation of the end-to-end XR rate control solution

In Figure 1, we see that the CN determines the rate adaptability of a QoS flow with the input from the UE and corresponding AF, then provides the information to the RAN. The RAN then based on the UE capability information can determine whether it can signal the XR rate control MAC CE. However, even if the UE does not support this capability, the CN still performs the determination of QoS flow rate adaptability and provides the information to the RAN, only to be ignored i.e., the CN is unaware of the contents of the UE radio access capabilities.

- CN is unaware of the contents of the UE radio access capabilities and whether it can support the XR rate control radio access capability, so will always perform the determination of QoS flow rate adaptability.

This rate adaptability is also used to setup a dedicated QoS flow to prevent multiplexing of non-rate adaptable flows, which could be unnecessary if the UE does not support the capability. Thus, RAN2 should discuss a solution to overcome this issue.

Discuss the issue on the CN being unaware of UE’s XR rate control capability when determining the rate adaptability of the QoS flow.

We believe that introducing a new NAS UE capability for UEs supporting XR rate control would be the simplest solution to make the CN aware. The details should be decided by SA2, so RAN2 should send an LS to SA2 about the introduction of a new NAS UE capability.

Introduce a new NAS UE capability to inform the CN about UE’s support for XR rate control. Send an LS to SA2.

### 2.2.2	UL Query Message

It was already agreed that the reception of the UL rate control MAC CE is a per-UE optional capability with signaling. Then, the following were the list of UE capabilities for the recommended bit rate MAC CE feature:

| recommendedBitRate Indicates whether the UE supports the bit rate recommendation message from the gNB to the UE as specified in TS 38.321 [8]. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| recommendedBitRateMultiplier-r16 Indicates whether the UE supports the bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [8], clause 6.1.3.20. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |
| recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the gNB as specified in TS 38.321 [8]. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |

As we can see the different aspects (query and multiplier) of the feature are covered by different UE capabilities conditional on the recommendedBitRate capability. Thus, the same should be applicable to the XR rate control feature that if the rate query MAC CE is confirmed, it should be tied to a new UE capability conditional on the XR rate control UE capability.

Define a new (optional) per-UE capability with signaling for the XR rate query MAC CE. A UE supporting this capability shall also indicate support for the UL (XR) rate control MAC CE.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	Only the gNB-CU can perform per-QoS flow monitoring under the assumption of using the information in the SDAP header.

Observation 2	The gNB-CU cannot perform per-QoS flow monitoring if the SDAP header is not present.

Observation 3	Putting additional requirements (i.e., per-QoS flow bit rate monitoring) on the CU has the risk that not all implementations can or will support it.

Observation 4	DRB-level bit rate monitoring is feasible with all possible configurations and is architecture and deployment independent.

Observation 5	There is much more F1 signaling load for the QoS flow in MAC CE solution when comparing the RRC-based solution.

Observation 6	If the QoS flow data rate indication from the CU to the DU is to be provided on a one-shot basis, then this is no different from the RRC-based solution with DRB-level MAC CE.

Observation 7	Due to the F1 delays, the QoS flow data rate information (including the QoS flows that need to be adapted) provided to the DU can be stale, rendering the solution less flexible. If the DU needs to inform the CU of its congestion level to receive the QoS flow data rate information, this further exacerbates the F1 delays.

Observation 8	The speed advantage of a MAC CE is diminished in the QoS flow in MAC CE solution considering cross-layer interactions or F1 delays for higher layers to provide additional information, such as the QFI.

Observation 9	By utilizing the QoS flow to DRB mapping and the associated QoS information, the RAN can accurately estimate the per-QoS flow data rate and deliver a reliable configuration to the UE that remains valid throughout the entire duration of a PDU session.

Observation 10	CN is unaware of the contents of the UE radio access capabilities and whether it can support the XR rate control radio access capability, so will always perform the determination of QoS flow rate adaptability.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The gNB is allowed to configure one or more QoS flows (mapped to a DRB) to be rate adapted.

Proposal 2	For rate adaptation of more than one QoS flow within a DRB, consider three options:

a.	 Up to UE implementation how to perform rate adaption between the configured QoS flows.

b.	 gNB indicates the ratio of rate adaption between the QoS flows as part of the configuration providing rate adaptability of one or more QoS flows.

c. 	gNB indicates the ratio of rate adaption between the QoS flows in the MAC CE itself.

Proposal 3	XR rate control MAC CE shall at least consist of the LCID.

Proposal 4	Discuss the issue on the CN being unaware of UE’s XR rate control capability when determining the rate adaptability of the QoS flow.

Proposal 5	Introduce a new NAS UE capability to inform the CN about UE’s support for XR rate control. Send an LS to SA2.

Proposal 6	Define a new (optional) per-UE capability with signaling for the XR rate query MAC CE. A UE supporting this capability shall also indicate support for the UL (XR) rate control MAC CE.

# 4 	References

- S2-2504252, LS reply on uplink rate control, 3GPP SA WG2 #168, Goteborg, Sweden.

- R3-253464, Discussion on XR UL bit rate control, 3GPP RAN WG3 #128, Athens, Greece.

# 5	Annex

The following are the agreements from RAN2#128 meeting on the XR rate control objective:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

The following agreement were made in RAN2#129 on the XR rate control objective:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this

The following agreements were made in RAN2#129bis on the XR rate control objective:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.