3GPP TSG-RAN WG2 #130	R2-2504344

St. Julians, Malta, 19th – 23rd May 2025

Agenda Item:	8.13.4

Source:	Ericsson

Title:	Service Continuity for Multi-Hop Relays

Document for:	Discussion, Decision

# 1 	Introduction

In the last meeting, with the baseline assumption that the intermediate relays are all transitioned to RRC_CONNECTED (if not already there), it was agreed that both the remote UE and first relay UE can evaluate the Uu-link and relay link based on the measurement configuration. Then, if the remote UE sends a measurement report, the gNB can trigger the path switch procedure. However, it was unclear how a path switch is triggered if any of the other nodes (apart from the remote UE) in the multi-hop link sends a measurement report.

In our understanding, only the measurement report sent by the remote UE can trigger the multi-hop path switch procedure whereas, the measurement report sent by the other nodes along the multi-hop link can only trigger the path switch/handover for the corresponding node for e.g., measurement report sent by the first relay UE can result in only its own path switch. Thus, the path switch/handover of any of the nodes in the multi-hop link apart from the remote UE results in a failure of the (multi-hop) link.

In this contribution, we elaborate on the multi-hop path switch procedure and how to deal with failures caused by path switch/handover of the intermediate nodes of the multi-hop link.

# 2 	Discussion

Based on the agreements made in the previous meetings, the views appear to be divergent on the triggering of the path switch procedure for the remote UE in a multi-hop scenario. Thus, in the following sections we provide our understanding for the two service continuity scenarios.

- Intra-gNB multi-hop indirect to direct path switch (i2d)

- Intra-gNB multi-hop indirect to single hop indirect path switch (i2i)

Based on the agreements made in the last meeting, the following block diagram summarizes our current understanding of the measurement event(s) and link evaluation for the different nodes in the multi-hop i2d scenario. The assumption is that all the nodes are in the RRC_CONNECTED state via the indirect path to the gNB, except for the second relay UE.

All nodes expect the second relay UE are in the RRC_CONNECTED state via the indirect path to the gNB.

- Multi-hop indirect to direct path switch

- Multi-hop indirect to single hop indirect path switch

Figure 1: Service continuity for multi-hop scenarios

Considering O1, although the first relay UE and intermediate relay UE can be configured with events X1/X2, the triggering of the path switch procedure (to a direct/indirect path) from either of the two nodes should not result in a multi-hop path switch procedure for the remote UE. Only the measurement report sent by the remote UE should be considered as a trigger for a path switch procedure.

Only the measurement report sent by the remote UE should be considered for triggering the multi-hop path switch procedure.

For the measurement reports sent by the other nodes i.e., the first relay UE, intermediate relay UE or second relay UE, these reports are only used by the network to trigger the path switch procedures for those corresponding nodes, and this does not affect the multi-hop path switch procedure for the remote UE as group mobility is not supported.

Confirm that the measurement report sent by the first relay UE, intermediate relay UE or the second relay UE does not affect remote UE’s multi-hop path switch procedure as group mobility is not supported.

Thus, with P2, the intermediate relay UE can also be configured to evaluate either the relay link or a Uu-link.

For multi-hop i2d and i2i, the intermediate relay UE evaluates adjacent upstream relay link, candidate relay link and Uu-link, where the sidelink measurement report includes at least the information as source L2 ID, serving cell ID, an a sidelink measurement quantity information.

Again, considering P2, it raises the question about whether a path switch/handover that results in a failure of multi-hop should be notified downstream to the remote UE to triggering relay reselection. We believe this depends on the node which performs the path switch/handover or declares RLF as follows:

- First Relay UE: If the first relay UE performs a path switch, it can inform the remote UE via the NotificationMessageSidelink as defined in Rel-17, thereby triggering the relay reselection at the remote UE.

- Intermediate Relay UE: If this node performs a path switch, at some point the first relay UE will declare an RLF and inform the remote UE via a NotificationMessageSidelink as defined in Rel-18 to trigger relay reselection.

- Second Relay UE: If the second relay UE performs a handover, then at some point the intermediate relay UE will declare RLF. Subsequently, the upper layers of the remote UE can trigger the relay reselection procedure based on the non-fulfilment of the QoS requirements based on the Rel-17 behaviour that the relay reselection can be triggered by upper layers.

In essence, the path switch/handover of any of the nodes along the multi-hop link should trigger the relay reselection procedure at the remote UE. And based on the above description, we see that this is already covered by legacy specifications and anything more is only an optimization.

Triggering the relay reselection procedure at the remote UE due to path switch/handover of any of the nodes along the multi-hop link is already covered by Rel-17/18 specifications and any enhancement is only an optimization.

Thus, we believe it is not required to notify downstream of the path switch/handover procedure that results in failure of the multi-hop link.

Do not pursue multi-hop downstream notification of the path switch/handover to trigger relay reselection at the remote UE, reuse Rel-17/18 mechanisms to inform child or parent node.

# 3 	Conclusion

Based on the discussion in the previous sections we draw the following observations:

Observation 1	All nodes expect the second relay UE are in the RRC_CONNECTED state via the indirect path to the gNB.

Observation 2	Triggering the relay reselection procedure at the remote UE due to path switch/handover of any of the nodes along the multi-hop link is already covered by Rel-17/18 specifications and any enhancement is only an optimization.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Only the measurement report sent by the remote UE should be considered for triggering the multi-hop path switch procedure.

Proposal 2	Confirm that the measurement report sent by the first relay UE, intermediate relay UE or the second relay UE does not affect remote UE’s multi-hop path switch procedure as group mobility is not supported.

Proposal 3	For multi-hop i2d and i2i, the intermediate relay UE evaluates adjacent upstream relay link, candidate relay link and Uu-link, where the sidelink measurement report includes at least the information as source L2 ID, serving cell ID, an a sidelink measurement quantity information.

Proposal 4	Do not pursue multi-hop downstream notification of the path switch/handover to trigger relay reselection at the remote UE, reuse Rel-17/18 mechanisms to inform child or parent node.

# 4	Annex

The following are the agreements from RAN2#128 on the aspect of service continuity:

Agreements:

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:

- Event Y2

- Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

In addition, the following agreements were made in the CP procedure regarding the RRC state of the intermediate relay UEs:

Agreement:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.

The following are the agreements from RAN2#129 on the aspect of service continuity:

Agreements:

For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

The following are the agreement from RAN2#129bis on service continuity:

Agreements:

In scenarios A/B, the Intermediate Relay UE could be configured to perform the measurement on adjacent upstream relay link, Uu link and candidate relay link.

In scenarios A/B, the measurement report from Intermediate relay UE will also include at least serving intermediate/last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Upon Intermediate Relay UE (including first relay UE) receiving reconfigurationWithSync, it either indicates to upper layers (to trigger PC5 unicast link release) or sends NotificationMessageSidelink message to the downstream intermediate Relay/Remote UE.  The choice is implementation-based.

Measurement event X2 could be applied to scenario B with multi-hop indirect to single-hop indirect path switching.

reconfigurationWithSync without sl-PathSwitchConfig can be reused to trigger Case A (Intra-gNB multi-hop indirect to direct path switching).

reconfigurationWithSync with sl-PathSwitchConfig can be reused to trigger Case B (Intra-gNB multi-hop indirect to single-hop indirect path switching).

RAN2 will not address cases of service continuity towards a relay path with intermediate relays in idle/inactive under the current WID.