3GPP TSG-RAN WG2 Meeting #130	R2-2504356

St. Julian's, Malta, May 19th – 23rd, 2025

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	Remaining issues on intended service area

Document for	:	Discussion and Decision

Introduction

In the last meeting, RAN2 reached some agreements/decisions as follows [1];

Agreements: - RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) - If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH - If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. - The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

In addition, at the post129bis email discussion, it was proposed the followings by rapporteur.

[Proposals for easy agreement] Proposal 1 If a new SMTC periodicity is finally adopted and included in system information broadcast, RAN2 introduces the support in a backwards compatible manner. FFS signalling details (e.g., new field or extension of SMTC4). Proposal 2 RAN2 includes the mapping between a MBS broadcast session (TMGI) and ISA ID(s) extending MBS-SessionInfoList to avoid signaling overhead (duplication of TMGIs). Proposal 3 For the service continuity discussion in MBS NR NTN, RAN2 takes the following scenarios as a baseline: - Multiple frequencies, served by the same satellite, illuminating the same geographical area. Each frequency provides a different MBS service. These services are not being broadcast in the serving cell. - Multiple frequencies, served by the same or different satellites, but not covering the same geographical area (they are neighbour cells). Each frequency provides a different MBS service. These services are not being broadcast in the serving cell. [Proposals for discussion] Proposal 4 Companies are encouraged to address the following topics in their contributions: - Maximum number of SMTCs that need to be supported (e.g., 6). - Association of SMTC with location/beam information (e.g. serving cell SSB index, reference location). - Nature of SIBXX defining the intended service area: cell or area specific.

In this paper, we’d like to discuss UE behaviours for MBS related procedures.

Discussion

MCCH re-acquisition

Based on the agreement of the RAN2#129bis meeting [1], the new SIB contains a list of ISAs and a list of NW-indicated IDs associated with each ISA entry of the list. The following ASN.1 is the text proposal about new SIB suggested in the current draft CR [2] under discussion.

SIBXX information element

-- ASN1START

-- TAG-SIBXX-START

SIBXX-r19 ::=                     SEQUENCE {

intendedServiceAreaList-r19     IntendedServiceAreaList-r19      OPTIONAL,  -- Need R

lateNonCriticalExtension        OCTET STRING                     OPTIONAL,

...

}

IntendedServiceAreaList-r19 ::=   SEQUENCE (SIZE (1.. maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19

IntendedServiceAreaInfo-r19 ::=   SEQUENCE {

intendedServiceAreaId-r19       MBS-IntendedAreaID-r19,

areaCoordinates-r19             CHOICE {

polygonArea                   OCTET STRING,

circleArea                    SEQUENCE {

referenceLocation-r19       ReferenceLocation-r17,

distanceRadius-r19          INTEGER(0..65535)

}

}

}

MBS-IntendedAreaID-r19  ::= INTEGER (0.. maxNrofMBS-Area-r19)

-- TAG-SIBXX-STOP

-- ASN1STOP

This draft CR suggested that network-indicated area ID included in new SIB is not represented by a pointer aggregating multiple intended service areas, but is an ID associated with one ISA, and that each ID is also included in the MCCH message in such a way that it is associated with the session.

Therefore, if there are some changes in the contents of the new SIB for some reason, it is likely that a change in ID assignment is occurring at the same time. For example, the contents of the list of ISAs may change, or the mapping between ISAs and IDs may change, and so on. This case is indicated following modification:

| Case 1 : Current specified ISAs in new SIB - Pairs of ISA and ID - {ISA#1, ID#1} - {ISA#2, ID#2} - {ISA#3, ID#3}… | Fig2.1.1. ISAs indicated by new SIB |
| --- | --- |
| Case 2 : Modified ISAs in new SIB - Pairs of ISA and ID - {ISA#1, ID#1} - {ISA#2+ISA#3 = ISA#4, ID#XX} | Fig2.1.2 Merged ISAs indicated by modified new SIB |

In this case, the UE which knows the association between the session and ISAs by MCCH messages needs to re-acquire MCCH messages to update them with the new information. Because the UE might be inside any ISA when the SIB includes ISA information is modified or the ISA ID included in the MCCH might not be consistent with the modified ISA and ISA ID.

Observation 1: As a result of ISA information modification in the new SIB, MCCH content may also be modified. In this case, UE needs to re-acquire MCCH.

If a new SIB is updated in the above Case 2 situation, the ISA ID assignment status may be changed. It is preferable to be able to obtain the updated MCCH at the timing when the change of new SIB is notified by SIB1. Here, the UE can follow the legacy modification procedure in order to know that there has been a change in the new SIB. However, the timing of MCCH update, and the timing of UE MCCH acquisition are not synchronized as shown in Fig.2.1.3. Therefore, if the MCCH is reacquired immediately after the new SIB is received, the UE may receive an old MCCH which has not been updated yet.

Observation 2: UE cannot get the latest MCCH information in some cases if the UE re-acquire MCCH immediately after new SIB is changed.

Fig.2.1.3 new SIB and MCCH modification timeline

In the current specification, when the network changes (some of) the MCCH information, it notifies the UEs about the change starting from the beginning of the MCCH modification period via PDCCH in that modification period. Based on this notification, the UE initiates the MCCH re-acquisition procedure. If information is added to the new SIB before the MCCH modification period, and/or if information is removed from the new SIB after the MCCH modification period, the UE can follow the legacy MCCH reacquisition procedure without causing any discrepancy between the MCCH and new SIB.

Observation 3: However, based on the current mechanism, network can control the timing of the new SIB change and MCCH modification without causing any discrepancy between the MCCH and new SIB.

Proposal 1 ： No change is needed in the procedure for re-acquisition of MCCH for the introducing the new SIB.

Open Issues : Mapping between ISA and TMGI

The Open issue list [3] addressed the issue of mapping ISAs to TMGI. About this issue, we think that duplicated TMGI field should be avoided as other companies mentioned. As a result, the draft CR is updated as MBS-SessionInfoExt-r19 includes only Area list as follows.

MBSBroadcastConfiguration message

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                         OPTIONAL,

nonCriticalExtension                  MBSBroadcastConfiguration-v19xy-IEs  OPTIONAL

}

MBSBroadcastConfiguration-v19xy-IEs ::= SEQUENCE {

mbs-SessionInfoListExt-r19            MBS-SessionInfoListExt-r19           OPTIONAL,   -- Need R

nonCriticalExtension                  SEQUENCE {}                          OPTIONAL

}

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

MBS-SessionInfoList information element

-- ASN1START

-- TAG-MBS-SESSIONINFOLIST-START

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfoListExt-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfoExt-r19

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

MBS-SessionInfoExt-r19 ::=       SEQUENCE {

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-SessionPerArea-r19)) OF MBS-IntendedAreaID-r19

}

...<omit>...

-- TAG-MBS-SESSIONINFOLIST-STOP

-- ASN1STOP

Based on the consensus of following agreement from last meeting, we think sessions that are not explicitly associated with the ISA should also be supported.

- If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

Observation 3 : Based on the agreement “If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.” from last meeting, sessions that are not explicitly associated with the ISA should also be supported.

However, current CR could not support the sessions that are not explicitly associated with the ISA(s).

In addition, if all session information, i.e. both ISA related session information and no ISA related session information, are included in the MBS-SessionInfo-r17 IE and mappings are indicated in extended IE (i.e. MBS-SessionInfoExt-r19) as the current CR description, the legacy UE cannot distinguish sessions between which are associated with ISA and which are not associated with ISA. In other words, legacy UEs consider all sessions as receivable. Therefore, IEs for ISA capable UEs and legacy UEs should be clearly separated in the MCCH.

Therefore, we propose the solution which both MBS-SessionInfo-r17 IE and MBS-SessionInfo(Ext)-r19 IE are used to indicate session information. In detail, to avoid duplicate TMGI fields and to support the backward compatible behaviour (for not ISA capable UEs), we think MBS sessions indicated by MBS-SessionInfo-r17 IE should not be associated with any ISAs (i.e. whole cell is a service area), and MBS-SessionInfo(Ext)-r19 IE includes session information which are associated with ISA, and ISA mapping information. There are advantages to splitting IE between r17 and r19 besides avoiding duplication of TMGI and compatibility with legacy.

Observation 4 : The Legacy UE interprets the sessions included in MBS-SessionInfo-r17 IE are receivable regardless sessions are associated with any ISA or not.

Observation 5 : MBSBroadcastConfiguration containing session information associated with ISAs can be placed in r19 IE to avoid duplicate TMGI (i.e. reducing signalling overhead) and to support the backward compatible behaviour.

Proposal 2 : To avoid duplicate TMGI and to support the legacy UEs, MBS-SessionInfo(Ext)-r19 IE includes the ISA ID together with session information which is associated with ISA, and MBS-SessionInfo-r17 IE includes session information which is not associated with ISA .

Alternatively, if any, we propose another solution based on the following rapporteur’s proposal in current CR on TS38.331[2].

MBS-SessionInfoList information element

-- ASN1START

-- TAG-MBS-SESSIONINFOLIST-START

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfoListExt-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfoExt-r19

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

MBS-SessionInfoExt-r19 ::=       SEQUENCE {

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-SessionPerArea-r19)) OF MBS-IntendedAreaID-r19

}

...<omit>...

-- TAG-MBS-SESSIONINFOLIST-STOP

-- ASN1STOP

This MBS-SessionInfoListExt-r19 field has one or more entries equal to the number of ISAs. This indicates that when the ISA(s) is/are present, there is a one-on-one correspondence between the session and the ISA. In other words, all ISAs are explicitly indicated in the new SIB, and all MBS sessions are associated with some ISAs in MCCH. If this CR is adopted, for example, to support session(s) that are not associated with the ISA(s), it is possible to make areaCoordinates-r19 as an optional field in the ISA information of new SIB, so that UE can consider a service bound to IDs that have no areaCoordinates-r19 as a service which is not bound to an ISA.

Proposal 3 : If all sessions are associated with any ISA IDs as a current CR description, to support sessions which are not explicitly associated with ISA, new SIB contain entries which consists of only the ISA ID.

new SIB signalling

It was agreed in the last meeting that if the ISA will not be explicitly stated in the new SIB, it could be a use case where the ISA covers the entire serving cell. There are some cases in which ISAs are not explicitly indicated are when current cell does not provide services associated with ISAs, and when services associated with ISAs are provided to the whole cell. This agreement is related in latter case and the direction of new SIB signalling could be assumed some following situations.

- The list of ISA information is included in new SIB, and does not include this entry of the ISAs concerned which is larger than whole cell.

In this case, when a UE receives a new SIB without only the entry in question, it is impossible to determine whether what should have been there originally is missing if no action is taken, or whether the ISA information is all that is contained in the current contents. In order to determine this, information on the frequencies of the serving cell and its own frequencies that it is interested in receiving should be included in the SIB21, and the information that can identify the ISA to the FSAI, i.e., the ID assigned to the ISA that encompasses the whole cell, should be associated with the ISA. Furthermore, as the rare case, the agreement of “no intended service area is explicitly indicated” could be interpreted that the intendedServiceAreaList-r19 has no entry in new SIB. This case is the same situation of only broadcasting SIB20. However, this operation can only the case of “There is an ISA larger than the cell, and there is no smaller ISA inside this cell, so just scheduling MCCH with the existing SIB20 is enough.”.

- The list of ISA information is included in the new SIB and includes an entry for the ISA which is larger than whole cell, but include only the ISA ID in that entry.

If a particular entry contains only an ID and the shape of the ISA is not explicitly stated, the UE may interpret the ISA with that ID as covering the whole serving cell.  In this case, it is only necessary to specify the contents of the NEW SIB, and there is no need to extend the contents of the other SIBs involved (SIB 20 and SIB 21). Therefore, we think this direction is more prefer solution. If Proposal 3 is adopted by RAN2, RAN2 should discuss the applicability of this solution furthermore.

Proposal 4 : RAN2 discuss that among the entries in the list of ISA information contained in the new SIB, the entry for the ISA covering the whole cell includes the ISA ID.

MBS interest indication transmission by UE within ISA

In RAN2#127 meeting, RAN2 agreed UE behaviours for MRB establishment/release [4].

Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

There are some other remaining MBS related procedures (e.g. MII (MBS interest indication), cell reselection priorities handling). If these procedures are performed by UEs which are not in intended service area, there are negative impacts for NW operation, especially if UL resources are used by the procedure. For example, if UE is allowed to send MII from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service. To prevent such negative impacts, the specification should also limit UE behaviours for these MBS related procedures. Therefore, if the MBS service is associated to the intended service area(s), MII should not be sent by UE located outside of the intended service area.

Observation 6 : If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 5 : MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Conclusion

In this paper, we made the following observations and proposals;

[MCCH re-acquisition]

Observation 1: As a result of ISA information modification in the new SIB, MCCH content may also be modified. In this case, UE needs to re-acquire MCCH.

Observation 2: UE cannot get the latest MCCH information in some cases if the UE re-acquire MCCH immediately after new SIB is changed.

Observation 3: However, based on the current mechanism, network can control the timing of the new SIB change and MCCH modification without causing any discrepancy between the MCCH and new SIB.

Proposal 1 ： No change is needed in the procedure for re-acquisition of MCCH for the introducing the new SIB.

[Open Issues : Mapping between ISA and TMGI]

Observation 3 : Based on the agreement “If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.” from last meeting, sessions that are not explicitly associated with the ISA should also be supported.

Observation 4 : The Legacy UE interprets the sessions included in MBS-SessionInfo-r17 IE are receivable regardless sessions are associated with any ISA or not.

Observation 5 : MBSBroadcastConfiguration containing session information associated with ISAs can be placed in r19 IE to avoid duplicate TMGI (i.e. reducing signalling overhead) and to support the backward compatible behaviour.

Proposal 2 : To avoid duplicate TMGI and to support the legacy UEs, MBS-SessionInfo(Ext)-r19 IE includes the ISA ID together with session information which is associated with ISA, and MBS-SessionInfo-r17 IE includes session information which is not associated with ISA .

Proposal 3 : If all sessions are associated with any ISA IDs as a current CR description, to support sessions which are not explicitly associated with ISA, new SIB contain entries which consists of only the ISA ID.

[new SIB signalling]

Proposal 4 : RAN2 discuss that among the entries in the list of ISA information contained in the new SIB, the entry for the ISA covering the whole cell includes the ISA ID.

[MBS interest indication transmission by UE within ISA]
Observation 6 : If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 5 : MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Reference

- Draft_RAN2_129b_Meeting_Report_v3.zip

- R2-25xxxxx – Draft_Introduction of Rel-19 NR NTN phase 3_v05_rapp.docx

- R2-25xxxx [draft] Open_issue_list_RRC_NTN_v12_Ericsson.docx

- R2-2409210 Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands