3GPP TSG-RAN-WG2 Meeting #130	R2-2504358

St.Julians, Malta, 19th– 23rd May, 2025

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	discussion on Relay discovery and (re)selection for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#129bis meeting, RAN2 reached following agreements [1]:

Agreements: - The discovery protocol stack figure in 38.300 (currently between remote and relay) should include also the relay-to-relay case for L2 multihop. - If the candidate intermediate relay UE is out of coverage without a PC5 connection to a parent relay UE, it can forward discovery messages based on preconfiguration. If it is in (direct) coverage, or out of coverage but PC5 connected to a parent relay UE (and thus indirectly to the network), it needs SIB12 or dedicated configuration to forward discovery messages. - For discovery model B when there is no established PC5 link between the remote UE and the intermediate relay UE, the intermediate Relay UE forwards the solicitation message only if the PC5 RSRP between the Remote UE (or intermediate Relay UE) and the intermediate Relay UE is above a threshold. - For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE. FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first. - For discovery model B, the quality of the PC5 links is assumed already to have been checked when the solicitation messages were forwarded, and no AS criterion is needed for the intermediate Relay UE(s) to forward the response message towards the Remote UE. - For discovery model B, upon discovery response messages reception, the Remote UE considers an intermediate Relay UE(s) as a candidate first relay UE(s) along the path to the last Relay UE if the SD-RSRP towards the first intermediate Relay UE is above a configured threshold. - Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected). - When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE). FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message. - RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item. This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI. - When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE. - FFS detailed information in the indication and child UE handling. - FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count. - FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

In this paper, we discuss how to introduce multi-hop relay.

Discussion

  - Discovery related issues

- RRC container in discovery message

RAN2 already agreed that the multi-path topology should not be supported in multi-hop relay. In that case, remote UEs and intermediate relay UEs should use the SIBs broadcasted on the last relay UE’s serving cell. In such a case, to prevent mis-configuration, the last relay UE's child UEs need to know the last relay UE's serving cell. In Rel-17 relay discovery, the RRC container is included in the discovery message, and the cell-related information is included in the RRC container. In multi-hop relay, based on the discovery message received from the last relay UE, the corresponding cell-related information needs to be included in the discovery message to be transmitted.

Proposal 1. When an intermediate relay UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

- FFS whether the case that there is no established PC5 link between the last relay UE and the intermediate relay UE before discovery model B transmission can occur and whether the intermediate relay UE always establishes itself as a remote UE first.

If the UE follows legacy behaviour, the UE establishes Uu RRC connection upon receiving RRCSetup message from a child UE. Furthermore, during discovery phase, the remote UE initiating the discovery is selecting the path. Thus, the intermediate relay UE might not be selected. So, RRC setup/resume should be initiated by an intermediate relay UE after the remote UE selects the path, i.e. the case can occur. Therefore, the intermediate relay UE always establishes RRC connection when receives RRC setup message from a child UE if there is no established PC5 link between the last relay UE and the intermediate relay UE.

Proposal 2. The intermediate relay UE always establishes RRC connection when receives RRC setup message from a child UE if there is no established PC5 link between the last relay UE and the intermediate relay UE.

  - Relay (re)selection related issues

- FFS whether the conditions for L2 multi-hop relay can be applied L3 multi-hop relay.

We don’t see any issues to apply the condition of relay reselection to L3 multi-hop relay. However, it should be made clear that these conditions do not directly trigger relay reselection but are instead a notification to higher layers.

Proposal 3. The agreed trigger conditions for relay reselection can be applied to L3 multi-hop relay but should only be an indication to higher layers.

- FFS the content in the notification

- FFS the child node behaviour after receiving this notification

- FFS other condition/timing that the intermediate relay UE notifies its child node

RAN2 agreed that an intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. But RAN2 hasn’t decided whether there is an upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. In the January meeting, i.e., SA2 Ad Hoc, SA2 agreed that intermediate UE can initiate relay reselection while keeping PC5-RRC connection with child UEs upon detecting PC5-unicast link failure [2]. In the CR, the intermediate relay UE is also acting as a remote UE. Therefore, there is no need to support a relay reselection procedure initiated by an intermediate relay UE separated from a relay reselection procedure initiated by a remote UE, but a relay reselection procedure initiated by a remote UE can be supported by an intermediate relay UE which is also acting as a remote UE from RAN2 perspective. Furthermore, in approach 1 C-plane procedure, intermediate relay UEs need to receive RRC reconfiguration messages for a remote UE configuration. In this case, intermediate relay UE has E2E RRC connection to the gNB and it should be in RRC_CONNECTED. The topology for these connections is exactly the same as for a (multi-hop) remote UE. And RAN2 agreed to only support approach 1 in Rel-19. Therefore, an intermediate relay UE should always act as a remote UE in approach 1.

Observation 1. SA2 agreed that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 4. RAN2 confirm that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 5. An intermediate relay UE always acts as a remote UE.

In case that intermediate relay UE initiated relay reselection, child UEs should be able to re-establish an RRC connection via the same intermediate relay UE. RAN2 already agreed that intermediate relay UE sends an indication if it initiates relay reselection. If the cause value of the indication is legacy PC5-RLF used for U2U relay, a child UE couldn’t recognize that the parent UE performs relay reselection. If the intermediate relay UE performs relay reselection, child UEs doesn’t need to perform relay reselection independently. However, it should rely on UE implementation whether the child UE performs relay reselection after receiving the indication. In this case, it is helpful for a UE to know what the cause of failure is. Therefore, legacy cause value and a new flag indicating a UE is performing relay reselection should be indicated in notification message.

Proposal 6. Legacy cause value and a new flag indicating a UE is performing relay reselection should be indicated in notification message.

Furthermore, since transmissions during a link failure may result in packet loss, the child UEs should suspend UL transmission when the child UEs are indicated that a link failure has occurred and that the relay reselection is occurring. Similar mechanism was introduced for the case of MP relay handover.

Proposal 7. Child UEs suspend UL transmission to avoid packet loss when it is indicated the intermediate relay UE reselection.

It is also FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count. We think that the indication should be same even if the cell of the reselected relay UE is same. Furthermore, the behaviour of an intermediate relay UE should not rely on its RRC state. It is important for the remote UE to have a usable path regardless of the RRC state of the parent UE, i.e. if the path becomes unavailable, this fact should be indicated to the remote UE as soon as possible. The subsequent behaviour of the child UE should be up to UE implementation (i.e., performing relay reselection or cell selection).

Proposal 8. Intermediate relay UE should indicate that relay reselection is ongoing to the remote UE regardless its RRC state.

Proposal 9. It is up to UE implementation (or upper layer) whether the child UE performs relay reselection or keeps current PC5 link when the child UE receives information indicating that relay reselection is ongoing from the parent UE.

Furthermore, a child node should send RRCReestablishmentRequest to NW when the relay reselection performed by the intermediate relay UE is completed. So, to initiate sending RRCReestablishmentRequest, the intermediate relay UE should indicate the relay reselection completion if the relay reselection performed by the intermediate relay UE is completed. Furthermore, if it is fails, i.e., T311 expires, the intermediate relay UE should indicate the failure of the relay reselection to a child UE.

Proposal 10. An intermediate relay UE should indicate the completion/failure of the relay reselection to a child UE.

- Applicability to other cases

Furthermore, we think these mechanisms can be adopted to intra-gNB path switching of intermediate relay UE with not much enhancement. It is reasonable that an intermediate relay UE performs path switching with child UEs as well as the case of relay reselection.

Proposal 11. A mechanism for an intermediate relay UE to initiate relay reselection while retaining a child UE can be adopted for path switching performed by the intermediate relay UE.

Conclusion

In this paper, we provided following observations and proposals:

Proposal 1. When an intermediate relay UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

Proposal 2. The intermediate relay UE always establishes RRC connection when receives RRC setup message from a child UE if there is no established PC5 link between the last relay UE and the intermediate relay UE.

Proposal 3. The agreed trigger conditions for relay reselection can be applied to L3 multi-hop relay but should only be an indication to higher layers.

Observation 1. SA2 agreed that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 4. RAN2 confirm that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 5. An intermediate relay UE always acts as a remote UE.

Proposal 6. Legacy cause value and a new flag indicating a UE is performing relay reselection should be indicated in notification message.

Proposal 7. Child UEs suspend UL transmission to avoid packet loss when it is indicated the intermediate relay UE reselection.

Proposal 8. Intermediate relay UE should indicate that relay reselection is ongoing to the remote UE regardless its RRC state.

Proposal 9. It is up to UE implementation (or upper layer) whether the child UE performs relay reselection or keeps current PC5 link when the child UE receives information indicating that relay reselection is ongoing from the parent UE.

Proposal 10. An intermediate relay UE should indicate the completion/failure of the relay reselection to a child UE.

Proposal 11. A mechanism for an intermediate relay UE to initiate relay reselection while retaining a child UE can be adopted for path switching performed by the intermediate relay UE.

Reference

- R2-2502984, Report from session on positioning and sidelink relay