3GPP TSG-RAN WG2 Meeting #130	R2-2504364

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.4.3

Source:	InterDigital, Inc.

Title:	Discussion on RRM measurement relaxation and offloading

Document for:	Discussion

# 1. Introduction

In RAN2#129bis following agreement were made [1].

RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both. It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. How to define LP-RSRP and LP-RSRQ is up to RAN1. The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review. Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

In RAN2#127bis following working assumptions were made [2].

- Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

In this contribution, we discuss RRM relaxation of UE MR (Main Radio) with offloading condition from MR to LR (Low power wake-up Receiver).

# 2. Discussion

Criterion for offloading and LP-WUS monitoring

For power saving operation with MR, an entry condition for LP-WUS monitoring is agreed. To satisfy entry condtion for LP-WUS monitoring, the serving cell quality (e.g., (LP-)RSRP/(LP-)RSRQ) measurements via MR and LR needs to be above a threshold. When the entry condition is satisfied, the LR may monitor LP-WUS and stop the legacy PO monitoring with MR.

In addition to the entry/exit conditions for LP-WUS monitoring, serving cell measurement offloading is agreed. According to the agreement, when the offloading condition is satisfied, serving cell measurement is fully performed by LR (i.e., serving cell measurement via MR is not required).

In [2] RAN2 agreed that two different measurement values (via MR and LR) considered for entry condition of seving cell measurement offloading, similar to entry condition for LP-WUS monitoring. Based on the decision, RAN2 can consider whether the same criteria (e.g., thresholds of RSRP and LP-RSRP) can be applied for entry/exit conditions of LP-WUS monitoring and offloading of the serving cell measurement.

In our view, if RAN2 defines different criteria (e.g., different thresholds) for entry/exit condtions, it may happen when the serving cell measurement is performed by LR, but monitoring the PO is still performed by MR. This operation is not desirable to achieve power saving gains with LR.

Proposal 1: Support merging criteria (e.g., thresholds) for entry/exit conditions of LP-WUS monitoring and offloading of serving cell measurement via LR.

Relaxed RRM measurement and offloding

In RAN4#112bis, RAN4 discussed RRM relaxation and serving cell measurement offloading (e.g., fully offloading and relaxed measurement) and the following agreements were made [3]

Case #1 and #3 have been agreed RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON.

According to IDLE specification [4], the UE choose to perform relaxed measurement (i.e., neighboring cell measurement), if relaxaxtion criteria or condition is satisfied as below. In our view, specifying conditions and how to determine satisfying the specified conditions for relaxed measurement is RAN2 scope (i.e., RAN2 spec impact). Hence, it is not necessary to wait RAN4 progress on FFS the conditions and how to satisfy the conditions for relaxed neighoring cell measurement.

##### 5.2.4.9.0 Relaxed measurement rules When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2: - if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and - if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP: - the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, 4.2.2.11, 4.2C.2.7 and 4.2C.2.8 in TS 38.133 [8]; - if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 and 4.2C.2.7 in TS 38.133 [8];

According to IDLE specification [4], relaxed measurement criterion for intra-frequency and inter-frequnecy measurement with MR is described. The triggering condition of RRM measurement relaxation of neighboring cell measurment is related to ‘UE with low mobility’ and/or ‘UE not at cell edge’ criteria.

##### 5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell. ##### 5.2.4.9.2 Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

Offloading case (Case #1)

RAN2 agreed that when an offloading condition is satisfied, serving cell measurement is performed by LR (fully offloaded from MR) and serving cell measurement via MR is not required. In this case, it is desired to deactivate MR for additional power saving. However, when the serving cell measurement is offloaded from MR to LR, there may be no serving cell measurement via MR.

Hence, the UE is not able to determine whether the legacy Rel-16 relaxation condition for neighboring cell measurements is satisfied due to absence of the serving cell measurement via MR (e.g., offloading case). To handle this issue, measured value (e.g., LP-SS) of the serving cell measurement via LR can be used to determine whether to satisfy the conditions of relaxed measurement for neighboring cell measurement (while the offloading condition is satisfied).

Proposal 2: RAN2 agrees Rel-19 criterion for neighboring cell measurement based on the current working assumption associated Rel-16 criterion. (WA: For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’).

Proposal 3: For offloading case, RAN2 agrees the current working assumption for neighboring cell measurement relaxation with LR measurement (WA: FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available).

Proposal 4: For offloading case, whether to satisfy MR neighbouring cell RRM measurement relaxation (i.e., R19 criteria) can be determined by serving cell measurement results via LR.

Relaxed case (Case #3)

RAN2 discussed the partially offloading case (e.g., Case #3/Relaxed case, defined by RAN4) and made agreements. But, it is still FFS on exit condition whether to include MR measurement. In our view, we prefer a unifed exit condtion with LR-based measurement. If we consider a separate exit condtion for partial offloading case with MR measurement, it may increase complexity to combine two different measurement results, e.g., LR measurement and relaxed MR measurement.

Morevoer, there is another FFS that if the entry condition for serving relaxation is the same as neighbour cell RRM measurement relaxation. In our view, each condition is designed with different scenario and criterion, For example, the entry condition is specified in Rel-19 LP-WUS for serving cell relaxation and neigboring cell measurement relaxation is specified in Rel-16 (e.g., ‘UE not at cell edge’ or ‘UE with low mobility’). Hence, there is no reason to configure as the same condition due to different objectives.

Proposal 5: For relaxed case, support the exit condition based on LR measurement.

Proposal 6. Do not support merging criteria (e.g., thresholds) for entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation.

Relaxed RRM measurement and EMR configuration

In RAN1#118 and RAN1#118bis the following agreements were made [5][6].

Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

In RAN2#129bis meeting, the following agreements were made [1].

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

The LP-WUS capable UE can support CA and NR-DC in connected state. To support the backward compatibility, the UE needs to support the legacy Early Measurement Reporting (EMR) procedure to setup CA or DC.

According to IDLE sepcification [4], neighboring cell measurement relaxation is not allowed when freuqencies (i.e., neighboring cells) are included in EMR configuration (i.e,. MeasIdleConfig) as below:

#### 5.2.4.9 Relaxed measurement ##### 5.2.4.9.0 Relaxed measurement rules When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2: - if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and - if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and - if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP: - the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, 4.2.2.11, 4.2C.2.7 and 4.2C.2.8 in TS 38.133 [8]; - if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and - if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled: - the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 and 4.2C.2.7 in TS 38.133 [8]; … NOTE 1: It is up to UE implementation when to start performing relaxed measurements in RRC Idle/Inactive if multiple methods are configured. NOTE 2: It is up to UE implementation which relaxation method to perform based on the "allowed" cases as specified in TS 38.133 [8] for RRC Idle/Inactive if multiple methods are configured. The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

However, the situation can be different when the UE is equipped with LR in addition to MR. For example, when LR is activated, peforming neighboring cell measurement under EMR configuration without RRM relaxation is not desirable to achieve more power saving gains. While serving cell measurement requires measurement of one cell, neighboring cell measurement generally requires measurents of multiple cells. Given the situation, offloading serving cell measurement to LR may require more power consumption if the UE needs to activate both MR (to measure the neighboring cells under EMR configuration) and LR (to measure the serving cell by offloading).

To avoid the situation and corresponding additional power consumption, measurement offloading from MR to LR for neighboring cell measurement under EMR configuration should be supported. The UE can perform neighboring cell measurement via LR (at least OFDM-based) when neighboring cell measurement is relaxed via MR (i.e., legacy relaxation conditions with MR are satisfied). For example, given that the performance of neighboring cell measurements via the OFDM-based LR may not much different from the measurement results via MR because the OFDM-based LR can measure the configured NR-SS/NR-SSB (i.e., similar to MR).

Proposal 7: Support neighboring cell measurement via LR (e.g., OFDM-based) if the UE is configured with legacy EMR and relaxation criteria for MR is satisfied.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Support merging criteria (e.g., thresholds) for entry/exit conditions of LP-WUS monitoring and offloading of serving cell measurement via LR.

Proposal 2: RAN2 agrees Rel-19 criterion for neighboring cell measurement based on the current working assumption associated Rel-16 criterion. (WA: For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’).

Proposal 3: For offloading case, RAN2 agrees the current working assumption for neighboring cell measurement relaxation with LR measurement (WA: FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available).

Proposal 4: For offloading case, whether to satisfy MR neighbouring cell RRM measurement relaxation (i.e., R19 criteria) can be determined by serving cell measurement results via LR.

Proposal 5: For relaxed case, support the exit condition based on LR measurement.

Proposal 6. Do not support merging criteria (e.g., thresholds) for entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation.

Proposal 7: Support neighboring cell measurement via LR (e.g., OFDM-based) if the UE is configured with legacy EMR and relaxation criteria for MR is satisfied.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #129bis.

[2] Chairman’s notes, 3GPP TSG RAN WG2 #127bis.

[3] Chairman’s notes, 3GPP TSG RAN WG4 #112bis.

[4] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.

[5] Chairman’s notes, 3GPP TSG RAN WG1 #118.

[6] Chairman’s notes, 3GPP TSG RAN WG1 #118bis.