3GPP TSG-RAN WG2 Meeting #130	R2-2504366

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.2

Source: 	CATT, Google, Sateliot, Thales

Title: 	Discussion on RAN2 impacts due to MME-configured satellite list

Document for: 	Discussion and Decision

Introduction

Based on SA2 reply LS and approved CR [1][2], the potential RAN2 impacts due to MME-configured satellite list was further discussed in RAN2 #129bis without reaching consensus: [3]

Come back in the next meeting (the proponents could show the possible spec impact if we decide to go in this direction)

Based on the latest progress in SA2 and last-meeting discussion in RAN2, this contribution further discusses necessary enhancements to support the MME-configured satellite list from RAN2 perspective, and provides corresponding TPs to justify the Spec impacts needed.

Latest SA2 progress and technical issues in AS

As illustrated by the coversheet and the changes of the agreed CR [2], SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling. According to the latest TS 23.401 [4] which already implemented the agreed SA2 CR in [2], although it says that how the UE behaves upon reception of this MME-configured satellite list is up to UE implementation, it was also captured that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Citation from coversheet part [2] Reason for change:; As described in the discussion paper (S2-2500780), this contribution removes this EN: Editor’s Note: How to handle the specification of the “List of satelliteID(s)” and “S&F Monitoring List” as per conclusions in TR 23.700-29 is FFS. The satellite list is a tool which is used by the UE assist with saving power and determining which satellites to synchronise data with, and a useful tool for the network to guide the UEs allowing the network to save resources. While how UE behaves when receiving the satellite list from MME is UE implementation. It is difficult to define the UE behaviour to only follow the received satellite list to access the network. This contribution proposes MME sends S&F Monitoring list to UE and UE behaviour is based on UE implementation and it may use it to determine which satellites to access. For example, if the UE determines a satellite is operating in S&F Mode, the UE may follow the S&F Monitoring List to ensure a successful access; if the UE determines a satellite is not operating in S&F Mode, the UE can access to this satellite even it is not in the S&F Monitoring List..

Citation from TS 23.401, 4.13.9.1 [4] […] - When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. […]

Observation 1: SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, SA2 intends to leave some room for the UE to decide which S&F satellite to access, with the consequence also specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

However, based on the current cell (re)selection mechanism in TS 36.304 [5], the UE shall select the cell relying on frequency priority (if supported) and RSRP/RSRQ, as specified in the following excerption from TS 36.304. This means that UE has to camp on/reselect to the corresponding cells meeting the RSRP/RSRQ conditions, irrespective of whether the cells are provided by satellites included in the MME-configured satellite list. That means, current cell reselection procedure leaves no room for UE implementation to further consider the MME-configured satellite list as specified in SA2 procedure. The consequence, as what is also highlighted above in TS 23.401, is the increasing chance for UE access failure, if the current cell reselection procedure is followed without considering the MME-configured satellite list.

#### 5.2.4 Cell Reselection evaluation process ##### 5.2.4.5 E-UTRAN Inter-frequency and inter-RAT Cell Reselection criteria [...] In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in clause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows: - If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6; - If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the MME-configured satellite list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempt to a satellite not in the MME-configured list).

Observation 3 above corresponds to a possible case for UE access failure due to the existing cell reselection procedure in the UE AS, and thus this is an essential technical issue that should be well settled by RAN2.

RAN2 impacts due to MME-configured satellite list

2.1	Cell reselection enhancement

To align with the design of SA2 Spec, especially to avoid the potential UE access failure as highlighted by SA2, we propose to leave some room for the UE to consider the MME-configured satellite list during cell reselection procedure.

Based on contributions from companies [6][7] and companies' views expressed during online discussion in the last meeting, we propose to enhance the definition of suitable cell and related operations by taking into account the MME configured satellite list.

Particularly, the UE may simply consider a detected cell as unsuitable and does not treat it as a candidate for reselection, if the UE is configured with a satellite list by the MME and the detected cell is not provided by the satellites included in the MME-configured satellite list. This can be simply specified in 4.3 of TS 36.304, as follows.

## 4.3 Service types in Idle Mode [...] suitable cell: A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements. - The cell is part of either: - the selected PLMN, or: - the registered PLMN, or: - a PLMN of the Equivalent PLMN list - For a CSG cell, the cell is a CSG member cell for the UE; According to the latest information provided by NAS: - The cell is not barred, see subclause 5.3.1; - The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" [4], which belongs to a PLMN that fulfils the first bullet above; - The cell selection criteria are fulfilled, see subclause 5.2.3.2; - Except for NB-IoT, if the UE supports authorization of coverage enhancements and upper layers indicated that use of coverage enhancements is not authorized for the selected PLMN: - the cell selection criterion S in normal coverage shall be fulfilled; - If the UE supports CE mode B and upper layers indicated that CE mode B is restricted: - the cell selection criterion S in normal coverage based on values Qrxlevmin and Qqualmin or in enhanced coverage based on values Qrxlevmin_CE and Qqualmin_CE shall be fulfilled. If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell. For a S&F Satellite operation capable UE, if a satellite list is configured by upper layers, the UE may not consider a cell operating in S&F mode as suitable cell if it is not a cell provided by the satellites configured in that list. [...] #### 5.2.4.4 Cells with cell reservations, access restrictions or unsuitable for normal camping For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1. If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes. If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable for a CN type due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for the CN type for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed. If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the TS 25.304 [8]. In case of NR further requirements are defined in the TS 38.304 [38]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed. If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection. For a S&F Satellite operation capable UE, if the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell operating in S&F mode which is not suitable due to not belonging to a satellite in the satellite list configured by upper layers, the UE may not consider this cell as candidate for reselection.

Note that whether/how the UE adopts above solution during cell reselection is left to UE implementation, which is in line with the SA2 Spec that leaves how the UE behaves upon reception of the MME-configured satellite list to UE implementation.

Based on the above analyses, we propose that RAN2 agrees to enhance the suitable cell definition and related operation during cell reselection procedure, in the case that MME-configured satellite list is provided to a S&F capable UE.

Proposal 1: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the MME-configured satellite list.

The complete TP to TS 36.304 for above Proposal 1 is summarized in the Annex A. We also propose that RAN2 takes the TP in Annex A as the baseline for the TS 36.304 running CR handling.

Proposal 1a: RAN2 adopts the TP provided in the Annex A as the baseline, if Proposal 1 is agreeable.

Besides the intended UE behaviors in above proposal 1, another aspect that needs to be confirmed is whether any new assistance information needs to be introduced in system information. Specifically, it can be considered whether a mapping between satellite ID and {frequency(ies), PCI(s)} can be provided by the serving cell's system information, for the UE to know the satellite actually associated with the highest ranked cell/best cell. However, one may also wonder whether it is possible to rely on the UE to read the SIB31 and acquire the satellite ID of the highest ranked cell/best cell by itself, in order to avoid introducing a mapping configuration.

To this end, we propose that RAN2 further discusses whether any assistance information needs to be introduced to support the MME-configured satellite list, pending the conclusion on Proposal 1.

Proposal 2: RAN2 discusses whether any new assistance information needs to be introduced into system information for the S&F operation (e.g. mapping between satellite ID(s) and {frequency(ies), PCI(s)}), if Proposal 1 is agreeable.

2.2	Relaxation on IDLE mode tasks

Besides reselecting/accessing to the proper satellites, another intention of SA2 to introduce this MME-configured satellite list is for UE power saving purpose, as seen above in the approved CR cover page [2]:

The satellite list is a tool which is used by the UE assist with saving power and determining which satellites to synchronise data with, and a useful tool for the network to guide the UEs allowing the network to save resources.

If there is an intention to further pursue this power-saving intention, we may take the discontinuous coverage design as a reference. Considering that the MME-configured satellites are not the satellites which provide neighbour cell coverage (i.e. not the next satellite that provides the immediate neighbour cell coverage), if the UE starts/stops idle mode tasks right upon the indicated satellite coming and leaving, it is beneficial for UE power saving. In more detail, if the UE determines that it is out of coverage of any target satellite indicated by MME, the UE operating in S&F mode may relax its IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.). How the UE determines whether it is in coverage or out of coverage of the indicated satellite(s) using satellite assistance information can be left up to UE implementation (similar to legacy discontinuous coverage mechinism).

Note that our intention is to allow UE some flexibility to relax IDLE mode tasks for power saving (i.e. using "the UE may relax…"), but not to prohibit the UE from doing these tasks when there is no coverage of satellite configured by MME. Then whether the UE really do this relaxation is eventually still left to UE implementation, and this can address companies' concern whether UE will miss the satellite outside the MME-configured list which is still allowed for UE's access: if the UE cannot accept such missing, it simply does not do the relaxation. Anyway, nothing is mandated, and everything is eventually up to UE implementation, which is in line with SA2 design. This is proposed below, and a TP to TS 36.304 is shown in Annex B.

Proposal 3: When a MME-configured satellite list is configured by upper layers, the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.) if the UE determines that it is out of coverage of all target satellite(s) indicated by MME. How the UE determines being out of coverage of all target satellites is up to UE implementation.

In case Proposal 3 is pursued, it may also be needed to discuss whether some new assistance information related to the satellites in the MME-configured list needs to be introduced into the system information specifically for the S&F use, or the existing satellite list with assistance information in SIB32 can be directly reused. We propose RAN2 to further discuss this configuration aspect, if Proposal 3 is agreed.

Proposal 3a: If Proposal 3 is agreed, RAN2 further discusses whether to introduce a separate list in system information to provide the satellite assistance information for S&F operation (i.e., used by UE to predict the coverage time of the satellites indicated by MME).

Conclusion

In this contribution, some leftover issues on support of S&F operation are discussed with corresponding proposals listed as follows:

Observation 1: SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Observation 2: As specified in the latest TS 23.401, SA2 intends to leave some room for the UE to decide which S&F satellite to access, with the consequence also specified that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

Observation 3: Following the current cell (re)selection procedure, the UE shall (re)select to/camp on a cell that meets related RSRP/RSRQ conditions and the NW configured priority. This leaves no room for UE implementation to further consider the MME-configured satellite list as specified in SA2 Spec, resulting in increasing UE access failure as highlighted by SA2 (in case of an access attempt to a satellite not in the MME-configured list).

On Cell reselection enhancements

Proposal 1: RAN2 agrees that the UE may not consider the cell operating in S&F mode as a suitable cell for reselection, if it is not a cell provided by the satellites included in the MME-configured satellite list.

Proposal 1a: RAN2 adopts the TP provided in the Annex A as the baseline, if Proposal 1 is agreeable.

Proposal 2: RAN2 discusses whether any new assistance information needs to be introduced into system information for the S&F operation (e.g. mapping between satellite ID(s) and {frequency(ies), PCI(s)}), if Proposal 1 is agreeable.

On IDLE Mode task relaxation

Proposal 3: When a MME-configured satellite list is configured by upper layers, the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.) if the UE determines that it is out of coverage of all target satellite(s) indicated by MME. How the UE determines being out of coverage of all target satellites is up to UE implementation.

Proposal 3a: If Proposal 3 is agreed, RAN2 further discusses whether to introduce a separate list in system information to provide the satellite assistance information for S&F operation (i.e., used by UE to predict the coverage time of the satellites indicated by MME).

Reference

- S2-2502450	Reply LS on Satellite IDs for store-and-forward operation

- S2-2502513	Support of S&F Monitoring List in S&F Mode

- RAN2#129bis ChairNotes.

- 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access" (V19.2.0)

- 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

- R2-2502053	Discussion on RAN2 impacts for the support of S&F operation	CATT	discussion	Rel-19

- R2-2502101	On the support for the S&F operation	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

- Annex A: TP for cell reselection enhancements

## 4.3	Service types in Idle Mode

This clause defines the level of service that may be provided by the network to a UE in Idle mode.

The action of camping on a cell is necessary to get access to some services. Three levels of services are defined for UE:

-	Limited service (emergency calls, ETWS and CMAS on an acceptable cell). It is not applicable to RRC_INACTIVE state.

-	Normal service (for public use on a suitable cell)

-	Operator service (for operators only on a reserved cell)

Furthermore, the cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications), and it is not applicable to RRC_INACTIVE state. Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in a E-UTRAN network:

-	The cell is not barred, see clause 5.3.1;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. The UE shall have a valid USIM and such a cell shall fulfil all the following requirements.

-	The cell is part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

-	For a CSG cell, the cell is a CSG member cell for the UE;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "forbidden tracking areas for roaming" TS 22.011 [4], which belongs to a PLMN that fulfils the first bullet above;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

-	Except for NB-IoT, if the UE supports authorization of coverage enhancements and upper layers indicated that use of coverage enhancements is not authorized for the selected PLMN:

-	the cell selection criterion S in normal coverage shall be fulfilled;

-	If the UE supports CE mode B and upper layers indicated that CE mode B is restricted:

-	the cell selection criterion S in normal coverage based on values Qrxlevmin and Qqualmin or in enhanced coverage based on values Qrxlevmin_CE and Qqualmin_CE shall be fulfilled.

If more than one PLMN identity is broadcast in the cell, the cell is considered to be part of all TAs with TAIs constructed from the PLMN identities and the TAC broadcast in the cell.

For a S&F Satellite operation capable UE, if a satellite list is configured by upper layers, the UE may not consider a cell operating in S&F mode as suitable cell if it is not a cell provided by the satellites configured in that list.

barred cell:

A cell is barred if it is so indicated in the system information TS 36.331 [3].

reserved cell:

A cell is reserved if it is so indicated in system information TS 36.331 [3].

Following exceptions to these definitions are applicable for UEs:

-	camped on a cell that belongs to a tracking area that is forbidden for regional provision of service; a cell that belongs to a tracking area that is forbidden for regional provision service (TS 23.122 [5], TS 24.301 [16]) is suitable but provides only limited service.

-	as an outcome of the manual CSG selection procedure the UE is allowed to access an acceptable cell which fulfils the cell selection criteria and is not barred or reserved for operator use for UEs not belonging to AC 11 or 15 and inform NAS that access is possible (for location registration procedure).

NOTE:	UE is not required to support manual search and selection of PLMN or CSGs while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected.

-	if a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for the duration of the emergency call.

-	if the UE in RRC_IDLE fulfils the conditions to support sidelink communication or PS related sidelink discovery in limited service state as specified in TS 23.303 [29], clause 4.5.6, the UE may perform sidelink communication or PS-related sidelink discovery.

-	if the UE in RRC_IDLE fulfils the conditions to support V2X sidelink communication or NR sidelink communication in limited service state as specified in TS23.285 [36], clause 4.4.8 and TS 23.287 [40], clause, 5.7, the UE may perform V2X sidelink communication or NR sidelink communication.

For E-UTRA the cell categorization defined above is per CN type. In this specification, when the term suitable/acceptable cell is used without specifying the CN type, it means the cell is suitable/acceptable for any of the CN type(s) supported by the UE.

NOTE:	The selected CN Type is not considered during cell selection and reselection procedure.

[...]

#### 5.2.4.4	Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable for a CN type due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for the CN type for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the TS 25.304 [8]. In case of NR further requirements are defined in the TS 38.304 [38]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

For a S&F Satellite operation capable UE, if the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell operating in S&F mode which is not suitable due to not belonging to a satellite in the satellite list configured by upper layers, the UE may not consider this cell as candidate for reselection.

- Annex B: TP on relaxation on IDLE mode task

## 4.1	Overview

The idle mode tasks can be subdivided into four processes:

-	PLMN selection;

-	Cell selection and reselection;

-	Location registration;

-	Support for manual CSG selection.

The relationship between these processes is illustrated in Figure 4.1-1.

Figure 4.1-1: Overall Idle Mode process

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set TS 23.122 [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

For E-UTRA a cell may be associated with more than one CN type (EPC and/or 5GC) and hence the selected cell can be suitable for more than one CN type. The CN type(s) for which the selected cell is suitable are reported to NAS which selects a CN type to be used for camping and for the NAS registration procedure (see below). Note that CN type selection is only applicabe for UE supporting E-UTRA connected to 5GC.

For E-UTRA a cell may be associated with more than one tracking area. The UE reports all the broadcasted tracking area codes in the selected cell to NAS for registration procedure.

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN TS 23.122 [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. Similar to cell selection procedure, if the reselected cell is an E-UTRA cell and the UE supports E-UTRA connected to 5GC, the CN type(s) for which the cell is suitable are reported to NAS which selects one of them. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed. In RRC_INACTIVE state, if the new cell does not belong to the configured RNA, a RNA update procedure is performed.

If necessary, the UE shall search for higher priority PLMNs at regular time intervals as described in TS 22.011 [4] and search for a suitable cell if another PLMN has been selected by NAS.

Search of available CSGs may be triggered by NAS to support manual CSG selection.

If the UE loses coverage of the registered PLMN, either a new PLMN is selected automatically (automatic mode), or an indication of which PLMNs are available is given to the user, so that a manual selection can be made (manual mode).

Registration is not performed by UEs only capable of services that need no registration.

The UE may perform sidelink communication or V2X sidelink communication or sidelink discovery or NR sidelink communication while in-coverage or out-of-coverage for sidelink, as specified in clause 11.

The purpose of camping on a cell in idle mode is fivefold:

a)	It enables the UE to receive system information from the PLMN.

b)	When registered and if the UE wishes to establish an RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped.

c)	If the PLMN receives a call for the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNAs (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in this set of tracking areas. The UE will then receive the paging message because it is tuned to the control channel of a cell in one of the registered tracking areas and the UE can respond on that control channel.

d)	It enables the UE to receive ETWS and CMAS notifications.

e)	It enables the UE to receive MBMS services.

If the UE is unable to find a suitable cell to camp on or if the location registration failed (except for LR rejected with cause #12, cause #14, cause #15 or cause #25, see TS 23.122 [5] and TS 24.301 [16]), it attempts to camp on a cell irrespective of the PLMN identity, and enters a "limited service" state.

When NAS indicates that PSM starts, the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while the UE is in PSM it is up to UE implementation whether it performs the corresponding action immediately or the latest when PSM ends. When NAS indicates that PSM ends, the UE shall perform all idle mode tasks.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.

If MME-configured satellite list is configured by upper layers and if the UE has determined that it is out of coverage of all the satellites in MME-configured satellite list using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in coverage of the satellites in MME-configured satellite list the UE shall perform all idle mode tasks related to NTN.