# 3GPP TSG-WG2 Meeting #130                 	   R2-2504367

# Malta, MT, 19th May 2025 - 23rd May 2025

# Source: 			CMCC

# Title:  Discussion on IoT NTN Store and Forward

# Agenda item:	8.9.2

# Document for: 	Discussion

Introduction

In the last RAN2#129bis meeting, the following agreements [1] were approved as below:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

Based on the discussions and conclusions in the previous meetings, we continue discussing some RAN2 related aspects in S&F operation.

Discussion

Opposite time information indication

In the last RAN2#129bis meeting, it was agreed to introduce opposite directional indication from normal to S&F. But for how to indicate this information, there was no consensus. In the after-meeting RRC open issue list for IoT NTN, the discussion was triggered again with three opinions: Option 1: Reuse legacy t-service; Option 2 Introduce a new indication; Option 3 Reuse the agreed time information for both directions and using S&F operation indication to decide direction. However, most companies focus on Option 1 and Option 3 and seems still no consensus can be made yet. The pros and cons of three opinions are summarized as below:

Figure 1 Pros and cons analysis of three options to indicate the opposite directional time information from normal mode to S&F mode

| Options | Pros | Cons |
| --- | --- | --- |
| Option 1 Reuse t-service | - Legacy UE can recognize the time information and be able to do measurement and cell reselection to other cells before the serving cell turns into S&F mode. | - May have to change the definition of legacy t-service |
| Option 2 Introduce a new indication | - Simple | - Only suitable to R19 S&F capable UE - Increase additional SIB resource overhead |
| Option 3 Reuse agreed time information and S&F operation indication | - No need to introduce new indication | - Only suitable to R19 S&F capable UE - Complicated |

With regards to the analysis above, since the legacy UE and Rel-19 UE not supporting S&F function only recognize T-service, it is reasonable for the cell to indicate the service time expired for legacy UE and Rel-19 UE not supporting S&F function when cell mode switching from normal to S&F mode. As for the R19 S&F capable UE, it can reuse the agreed time indication as the stop indication of S&F mode after UE switching to S&F mode. Thus, we think it’s better to use T-service as the time information indication from normal mode to S&F mode. Besides, the agreed time information can be used to indicate the end of S&F mode after UE switching from normal mode to S&F mode. That can be regarded as the stop indication of the serving cell.

Proposal 1: T-service can be used as the time information indication from normal mode to S&F mode.

Proposal 2: The agreed time information can be used for to indicate the end of S&F mode after UE switching from normal mode to S&F mode.

Measurement and cell reselection

Another RRC open issue is about whether to introduce additional assistance information for neighbour cells, e.g., the neighbour cell operation mode and/or mode transmission time. In our opinion, both two assistance information (neighbour cell operation mode and mode transmission time) are useful: For the operation mode, when there are multi neighbour cells for UE to reselect and t-serviceStartNeigh/t-serviceStart of neighbour cells are almost same, introducing a mode operation indication can be useful for UE to reselect to a normal cell which can provide better services and data transmission. In our opinion, compared with S&F cells, normal cells should have high priority during the cell reselection procedure; As for mode transmission time, one potential benefit is that when the serving cell is in S&F mode, and the coming neighbour cell is in S&F mode but not in the satellite list provided by MME which means the neighbour cell does not have the UE context and have not recover the feeder link with the MME-ground yet. In this case, if the operation transition time e.g. t-serviceStartSF smaller than t-service of the serving cell, that means UE can reselect to this normal cell before t-service expired (higher priority for normal cell). If the operation transition time e.g. t-serviceStartSF larger than t-service of the serving cell, UE does not need to access the neighbour cell before t-service expiring unless the neighbour cell turns into normal mode. It do have impacts on UE behaviour on measurement and cell reselection. Thus, we think introducing operation mode time is useful.

Proposal 3: It is proposed to introduce operation mode of the neighbour cells as the additional assistance information.

Proposal 4: It is proposed to introduce mode transmission time of the neighbour cells as the additional assistance information.

As for the specific spec impact for the assistance information of neighbour cell, similar to the agreed time information for transition remaining time, we can use one absolute value to express neighbour cell both operation mode and mode transmission time, e.g., t-serviceStartSF in SIB32 or t-serviceStartNeighSF in SIB33. It can work well both in two directions mode switching: 1) For S&F mode to normal mode transition, if the IE present, it means that neighbour cell is in S&F mode and the value stands for the mode switching time; If the neighbour cell is not related to mode switching (S&F mode to normal mode), the value of new IE can be set equal to the t-service of neighbour cell. 2) For normal mode to S&F transition, if the IE is not present, it means that neighbour cell is in normal mode; if there is mode switching from normal to S&F of the neighbour cell, firstly, when t-serviceStartSF/t-serviceStartNeighSF is expired, UE will do measurement and reselect to a neighbour cell in normal mode. Then if there exists mode switching which means the cell will soon change to S&F mode, UE can following the current procedure to get the time information of transition time after reselecting to the neighbour cell. No further enhancement foreseen. Thus, we think one IE using absolute value similarly to the agreed time information is enough for indicating additional assistance information for the neighbour cells.

Proposal 5: It is proposed to introduce one absolute value time information to indicate the operation mode and mode transmission time of the neighbour cells. e.g., t-serviceStartSF in SIB32 or t-serviceStartNeighSF in SIB33.

In the last RAN2#129bis meeting, we discussed about whether to prioritize the satellites within the MME-configured satellite list. We are okay with the original proposal to prioritize the frequency of satellites in the MME-configured list. And it can be implemented by defining a new frequency list in SIB message which only contains the satellites within MME-configured list. Besides, we need also to consider the impact of finding normal mode cells during the cell reselection procedure. We think normal mode cell should have higher priority than the cell in S&F mode since it can provide better services and data transmission. Thus, in our opinion, if there exist three different kinds of satellite during the cell reselection procedure, the priority rank (from high to low) in the cell reselection of different kinds of satellites should be: normal satellite, satellite in the MME-configured list and satellite not in the list.

- Proposal 6 During cell reselection procedure, UE may prioritize frequency(ies) corresponding to the satellite(s) indicated in the MME-configured satellite list

- Proposal 7 During cell reselection procedure, compared with the satellites in S&F mode, satellites in normal mode may have a higher priority.

UE capability

- In the running CR of TS36.306, there is a editor note added for whether to introduce a new AS capability for S&F capable UE since SA2 has already defined NAS level capability. In our opinion, As UE capability is needed since we have defined new S&F operation indication, new transition time information (at least for S&F to normal, FFS normal to S&F) in SIB message. All these new defined indication need the network to know that UE has such kind of S&F capability in AS layer. Thus, it is necessary to introduce a new AS UE capability.

Proposal 8: It is proposed to introduce a new AS capability for UE supporting S&F mode.

Paging enhancement

When S&F capable UE work in the cell operating in S&F mode or out of satellite coverage, UE does not need to monitor paging message all the time. That means UE can relax the paging monitoring for some scenarios shown as below:

- When UE is not within the satellite coverage area:

  - The UE can determine when the next satellite carrying the DL data it needs (paging itself) will arrive based on the waiting timer in NAS rejection message. Therefore, when the UE is not within the coverage area of the satellite that means there is no satellite carrying downlink data to wake up the UE. Thus, the UE does not need to monitor the paging message.

- When UE is within the satellite coverage area and camps on a cell in S&F operation:

- The network has already transmitted stored DL data: as for the normal procedure, the satellite sends the stored downlink messages to the UE. If there is no data communication and the DRX timer runs out, the UE goes back to idle state. Therefore, during the time information of the S&F operation, the network will not send new downlink data to the UE since feederlink does not recover. Thus the UE can have a relaxation on paging monitoring when UE goes back to idle state after receiving all stored data from the satellite.

- The satellite never recover its feederlink: That means the current satellite does not have data transmission with ground EPC. Thus, the satellite only has stored UE uplink data and no new arrived downlink data. Therefore, even UE is within the coverage area, without recovered feederlink, UE does not need to monitor paging and only switch back to RRC connected state when has uplink data to send.

Proposal 9: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data and UE goes back to idle state after receiving all stored data from the satellite

- The satellite never recover its feederlink

System information change notification

For the possible changes of time information from S&F to normal mode, there was an agreement that a change to the (S&F-> normal mode) time indication shall not result in system information change notification. As for the changes of S&F operation indication in SIB1, we think similar rules can be applied at least for S&F capable UE in RRC connected mode in the operation mode switching procedure. NB-IoT NTN UE can deduce the mode switching based on agreed time information or T-service and eMTC NTN UE can get the updated SIB from dedicated signalling. Otherwise, if S&F operation indication changes result in system information change notification which may let network release UE to idle state to monitor paging message, UE in RRC connected mode has to go back to idle mode to receive SIB1 and then enter RRC connected mode again which will greatly increase the power consumption. Besides, this kind of operation also have a potential risk that UE can not go back to RRC connected mode after receiving system information change notification due to the heavy load of network (if cell turns into normal mode, many legacy UE or R19 S&F not capable UE will try to camp on the cell simultaneously). We should avoid this kind of corner case from happening. Thus, we propose that the changes of S&F operation indication during mode switching will not lead to UE in RRC connected mode go back to idle to monitor paging which may include system information change notification.

Proposal 10: Before mode switching, eMTC NTN UE in RRC connected mode can update changed S&F operation indication through dedicated signaling, rather than going back to idle mode and receiving paging message.

Proposal 11: Before mode switching from normal to S&F, NB-IoT NTN UE in RRC connected mode can deduce the time of operation mode switching by using operation time information based on implementation, the network does not need to release UE to idle state for the reason of changing the update system information.

Conclusions

- Opposite time information indication

Proposal 1: T-service can be used as the time information indication from normal mode to S&F mode.

Proposal 2: The agreed time information can be used for to indicate the end of S&F mode after UE switching from normal mode to S&F mode.

- Measurement and cell reselection

Proposal 3: It is proposed to introduce operation mode of the neighbour cells as the additional assistance information.

Proposal 4: It is proposed to introduce mode transmission time of the neighbour cells as the additional assistance information.

Proposal 5: It is proposed to introduce one absolute value time information to indicate the operation mode and mode transmission time of the neighbour cells. e.g., t-serviceStartSF in SIB32 or t-serviceStartNeighSF in SIB33.

- Proposal 6 During cell reselection procedure, UE may prioritize frequency(ies) corresponding to the satellite(s) indicated in the MME-configured satellite list

- Proposal 7 During cell reselection procedure, compared with the satellites in S&F mode, satellites in normal mode may have a higher priority.

- UE capability

Proposal 8: It is proposed to introduce a new AS capability for UE supporting S&F mode.

- Paging enhancement

Proposal 9: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data and UE goes back to idle state after receiving all stored data from the satellite

- The satellite never recover its feederlink

- System information change notification

Proposal 10: Before mode switching, eMTC NTN UE in RRC connected mode can update changed S&F operation indication through dedicated signaling, rather than going back to idle mode and receiving paging message.

Proposal 11: Before mode switching from normal to S&F, NB-IoT NTN UE in RRC connected mode can deduce the time of operation mode switching by using operation time information based on implementation, the network does not need to release UE to idle state for the reason of changing the update system information.

References

- 3GPP Chair_Notes, R2-129 NR-NTN-IoT-NTN (Sergio) EOM