3GPP TSG-RAN WG2 Meeting #130       	                      R2-2504380

Malta, MT, 19th-23rd May 2025

Agenda item:	8.3.4

Source:	CMCC

Title:	Discussion on data collection for AI Mob

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 had the following agreements on data collection for AI Mob:

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

According to the guidance from chair lady, this contribution will focus on general data collection procedure, and specific data content for model training for RRM measurement prediction with UE-sided model and NW-sided model, and for measurement event prediction with UE-sided model.

- Discussion

- 2.1 NW-side data collection procedure

General

During the discussion on NW-side data collection in AI/ML PHY, RAN2 considers gNB-centric and OAM-centric approaches, and aims to apply the same measurement framework for them. Until now, RAN2 takes the immediate MDT as baseline for OAM-centric approach due to UE in connected mode, and has some progress on enhancements. For AI Mob, RAN2 can also consider both OAM-centric and gNB-centric approach for NW-side data collection for model training.

Proposal 1: RAN2 to consider gNB-centric and OAM-centric approaches for NW-sided model training data collection for AI Mob, and aims to apply the same measurement framework for them. For OAM-centric approach, RAN2 takes immediate MDT as baseline.

Logging

In order to avoid frequent reporting and reduce the signaling overhead, logging in RRC_CONNECTED mode is optionally supported in AI/ML PHY. As the baseline approach, the UE receives the measurement configuration from the network, and performs measurements and logging (if supported). For AI Mob, the similar enhancement is needed.

Proposal 2: Logging in RRC_CONNECTED mode is optionally supported for model training data collection for AI Mob.

RAN2 supports periodical and event-triggered logging of L1 beam measurement results in AI PHY, and also discussed the type of triggered event, such as L1 measurement based, L3 measurement based. RAN2 agreed to support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not, and L1 measurement event triggered will not be supported. We understand the similar principle can be also applied to AI Mob.

Proposal 3: Periodical and L3 measurement event triggered logging are supported for NW-side data collection for AI Mob.

Data collection configuration

In AI PHY, RAN2 agreed that the measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc). The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. For AI Mob, the data collection configuration can include the configurations for RRM measurement and/or measurement event. Furthermore, the measurement resources and logging information, e.g. periodicity, triggered event, and RS type, will be included in the data collection configuration.

Proposal 4: For AI Mob, the data collection configuration can include the configurations for RRM measurement and/or measurement event.

Proposal 5: The data collection configuration can include the measurement resources and logging information, e.g. periodicity, triggered event and RS type.

Availability indication

During the discussion on AI PHY, RAN2 agreed that when the data collection configuration is provided, the UE performs measurements and logging (if supported). And if there is available data, the UE sends availability indication to the network which is similar to MDT. In addition, RAN2 discussed the triggered event for availability indication, and agreed to use full buffer being reached, buffer threshold being reached, or low power as triggered event. When triggered event is satisfied, the UE sends the UAI including data availability indication and/or cause value to the network. For AI Mob, the same way is also applicable.

RAN2#129bis Agreements - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

Proposal 6: When availability indication is triggered due to full buffer being reached, buffer threshold being reached or low power, the UE sends the UAI including data availability indication and/or cause value (reason for trigger, low power indication) to the network.

On the other hand, RAN2 agreed that when the buffer is full, the UE can stop data collection, however, the UE will not autonomously stop when low power state is detected until the network releases the configuration. For AI Mob, the similar mechanism can be reused.

Proposal 7: When the buffer is full, the UE can stop data collection.

Proposal 8: the UE will not autonomously stop when low power state is detected until the network releases the configuration.

Data retrieval

For AI PHY, RAN2 supports on-demand reporting while periodic reporting is not supported considering the signaling overhead. For data reporting, the same procedure and principles as for logged MDT are reused, i.e., the UEInformationRequest and UEInformationResponse messages are used for on-demand reporting. Furthermore, UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. In addition, RAN2 introduced a new SRB with lower priority for NW-side data collection. we think the above progress in AI/ML PHY can be also applied in AI Mob.

Proposal 9: UEInformationRequest and UEInformationResponse messages are used for on-demand reporting of AI/ML training data collection.

Proposal 10: UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission.

Proposal 11: A new SRB with lower priority introduced in AI/ML PHY is also used for NW-side data collection in AI Mob.

Basic flow

As per the above analysis and proposals, the basic flow for NW-side data collection procedure can be illustrated as Figure 1:

Figure 1. Basic flow for NW-side data collection procedure

Proposal 12: RAN2 takes the Figure 1 as baseline for NW-side data collection procedure.

- 2.2 UE-side data collection procedure

During the discussion on UE-side data collection in AI/ML PHY, RAN2 has the following agreements:

- RAN2#129 Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

- RAN2#129bis Agreements

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In general, the network can provide or release the data collection configuration with or without UE request. The UE can send UAI message for request of data collection measurement configuration.

Figure 2. Basic flow for UE-side data collection configuration procedure

For AI Mob, the similar principle and procedure can be reused.

Proposal 13: The network can provide or release the data collection configuration with or without UE request for UE-side data collection.

Proposal 14: The UE can send UAI message to the network for request of data collection measurement configuration, including an indication on start/stop of data collection, preferred configuration from a list of candidate configurations provided by NW.

- 2.3 Data content for RRM measurement prediction

During the study on RRM measurement prediction, RAN2 discussed temporal domain case A/case B, frequency domain and spatial domain prediction, as well as cell-based and cluster-based option. And for each scenario, the similar sub-use cases are considered for L3 beam level and cell level measurement prediction:

- For L3 cell level measurement prediction:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Case 2: To directly predict cell level results based on cell level results

- Case 3: To directly predict cell level results based on beam level results

- For L3 beam level measurement prediction:

- Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results

- Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results

- Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results

For temporal domain prediction case 1, at least L1 beam level results and/or beam ID, cell ID (e.g. for cluster-based) are needed, for case 2, at least L3 cell/beam level results and cell ID (e.g. for cluster-based) are needed, and for case 3, at least L1 beam level results and/or beam ID, L3 cell/beam level results and cell ID (e.g. for cluster-based) are needed. Furthermore, time info may be needed.

Observation 1: For temporal domain prediction, L1 beam level, L3 beam/cell level results, and time info are needed for model training, while the details can be different per sub-use case.

Regarding to case A and case B, we think OW/PW length and MRRT are not necessary for data collection. We understand that these info is up to implementation during model training procedure.

- Case A (without measurement reduction)          (b) Case B (with measurement reduction)

- Figure 3. Temporal domain prediction

Observation 2: OW length, PW length and MRRT are not necessary for model training data collection, which can be up to implementation.

For frequency domain prediction, in addition to beam/cell level results, frequency info (e.g. 2GHz, 4GHz) is also needed.

Observation 3: For frequency domain prediction, in addition to beam/cell level results, frequency info is needed.

For spatial domain prediction, RAN2 agreed to pending the discussion in last meeting since it is similar to beam management case in AI/ML PHY.

Based on the simulation results, RAN2 had the following observations on UE speed for intra-frequency temporal domain and inter-frequency prediction. Since UE speed has impacts on prediction accuracy for intra-frequency temporal domain prediction, it should be included in data collection for model training. However, UE speed has minor impact on inter-frequency prediction, it is not needed to include UE speed in data collection.

RAN2#127bis Agreement 1 For intra-frequency temporal domain, higher UE speeds result in larger prediction errors 2 For co-located scenario, the UE speed in the inter-frequency case has minor impact on prediction accuracy

Observation 4: UE speed is needed for intra-frequency temporal domain prediction, while it is not necessary for inter-frequency prediction.

Regarding to UE-sided model and NW-sided model, we think the similar data content will be used for model training, from the perspective of standardized data. Of course, the proprietary information for model training can be up to implementation.

Based on the above analysis, we summarize the data content for model training per sub-use case as follows:

Table 1. Data content for model training for RRM measurement prediction

| LCM purpose | Prediction domain | Sub-use case | Model type | Data content |
| --- | --- | --- | --- | --- |
| Model training | Temporal domain | Case 1 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - Cell ID (e.g. for cluster-based) - UE speed - Time info |
|  |  | Case 2 | UE-sided model, NW-sided model | - L3 cell level results, or L3 beam level results - Cell ID (e.g. for cluster-based) - UE speed - Time info |
|  |  | Case 3 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - L3 cell level results, or L3 beam level results - Cell ID (e.g. for cluster-based) - UE speed - Time info |
|  | Frequency domain | Case 1 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - Cell ID (e.g. for cluster-based) - Frequency info |
|  |  | Case 2 | UE-sided model, NW-sided model | - L3 cell level results, or L3 beam level results - Cell ID (e.g. for cluster-based) - Frequency info |
|  |  | Case 3 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - L3 cell level results, or L3 beam level results - cell ID (e.g. for cluster-based) - Frequency info |

Proposal 15: RAN2 takes Table 1 as starting point for model training data collection for RRM measurement prediction.

- 2.4 Data content for measurement event prediction

RAN2 studied direct prediction and indirect prediction for measurement event prediction, and only UE-sided model is considered. And RAN2 agreed to consider all A1-6 events, unless technical problems are identified.

For indirect prediction, the same data content as temporal domain RRM measurement prediction discussed in section 2.3 will be reused for model training.

For direct prediction, temporal L3 cell level measurement results of serving cell and/or neighbor cells, measurement event configuration (e.g. offset, TTT), and occurrence time are needed for model training.

Based on the above analysis, we summarize the data content for model training for direct and indirect prediction as follows:

- Table 2. Data content for model training for measurement event prediction

| LCM purpose | Prediction option | Model type | Data content |
| --- | --- | --- | --- |
| Model training | Indirect prediction | UE-sided model | - Same as RRM measurement prediction |
|  | Direct prediction | UE-sided model | - Temporal L3 cell level measurement results of serving cell and/or neighbor cells - Measurement event configuration (e.g. offset, TTT) - Occurrence time of measurement event |

Proposal 16: RAN2 takes Table 2 as starting point for model training data collection for measurement event prediction.

# 3	Conclusion

Here are the proposals on data collection for AI Mob.

NW-side data collection procedure

- General

Proposal 1: RAN2 to consider gNB-centric and OAM-centric approaches for NW-sided model training data collection for AI Mob, and aims to apply the same measurement framework for them. For OAM-centric approach, RAN2 takes immediate MDT as baseline.

- Logging

Proposal 2: Logging in RRC_CONNECTED mode is optionally supported for model training data collection for AI Mob.

Proposal 3: Periodical and L3 measurement event triggered logging are supported for NW-side data collection for AI Mob.

- Data collection configuration

Proposal 4: For AI Mob, the data collection configuration can include the configurations for RRM measurement and/or measurement event.

Proposal 5: The data collection configuration can include the measurement resources and logging information, e.g. periodicity, triggered event and RS type.

- Availability indication

Proposal 6: When availability indication is triggered due to full buffer being reached, buffer threshold being reached or low power, the UE sends the UAI including data availability indication and/or cause value (reason for trigger, low power indication) to the network.

Proposal 7: When the buffer is full, the UE can stop data collection.

Proposal 8: the UE will not autonomously stop when low power state is detected until the network releases the configuration.

- Data retrieval

Proposal 9: UEInformationRequest and UEInformationResponse are used for on-demand reporting of AI/ML training data collection.

Proposal 10: UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission.

Proposal 11: A new SRB with lower priority introduced in AI/ML PHY is also used for NW-side data collection in AI Mob.

- Basic flow

Proposal 12: RAN2 takes the Figure.1 as baseline for NW-side data collection procedure.

UE-side data collection procedure

Proposal 13: The network can provide or release the data collection configuration with or without UE request for UE-side data collection.

Proposal 14: The UE can send UAI message to the network for request of data collection measurement configuration, including an indication on start/stop of data collection, preferred configuration from a list of candidate configurations provided by NW.

Data content for RRM measurement prediction

Observation 1: For temporal domain prediction, L1 beam level, L3 beam/cell level results, and time info are needed for model training, while the details can be different per sub-use case.

Observation 2: OW length, PW length and MRRT are not necessary for model training data collection, which can be up to implementation.

Observation 3: For frequency domain prediction, in addition to beam/cell level results, frequency info is needed.

Observation 4: UE speed is needed for intra-frequency temporal domain prediction, while it is not necessary for inter-frequency prediction.

Proposal 15: RAN2 takes Table 1 as starting point for model training data collection for RRM measurement prediction.

Table 1. Data content for model training for RRM measurement prediction

| LCM purpose | Prediction domain | Sub-use case | Model type | Data content |
| --- | --- | --- | --- | --- |
| Model training | Temporal domain | Case 1 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - Cell ID (e.g. for cluster-based) - UE speed - Time info |
|  |  | Case 2 | UE-sided model, NW-sided model | - L3 cell level results, or L3 beam level results - Cell ID (e.g. for cluster-based) - UE speed - Time info |
|  |  | Case 3 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - L3 cell level results, or L3 beam level results - Cell ID (e.g. for cluster-based) - UE speed - Time info |
|  | Frequency domain | Case 1 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - Cell ID (e.g. for cluster-based) - Frequency info |
|  |  | Case 2 | UE-sided model, NW-sided model | - L3 cell level results, or L3 beam level results - Cell ID (e.g. for cluster-based) - Frequency info |
|  |  | Case 3 | UE-sided model, NW-sided model | - L1 beam level results and/or beam ID - L3 cell level results, or L3 beam level results - Cell ID (e.g. for cluster-based) - Frequency info |

Data content for measurement event prediction

Proposal 16: RAN2 takes Table 2 as starting point for model training data collection for measurement event prediction.

- Table 2. Data content for model training for measurement event prediction

| LCM purpose | Prediction option | Model type | Data content |
| --- | --- | --- | --- |
| Model training | Indirect prediction | UE-sided model | - Same as RRM measurement prediction |
|  | Direct prediction | UE-sided model | - Temporal L3 cell level measurement results of serving cell and/or neighbor cells - Measurement event configuration (e.g. offset, TTT) - Occurrence time of measurement event |

# 4	Reference

- Draft_RAN2_129bis_Meeting_Report_v2