3GPP TSG-RAN WG2 Meeting #130	R2-2504387

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.5.3

Source:	CMCC

Title:	Discussion on On-demand SIB1

Document for:	Discussion

Introduction

In RAN#102 meeting, a new WID on enhancements of network energy saving has been approved [1], one of the objectives is on-demand SIB1 for UEs in idle/inactive mode, the scope of this objective has been discussed and revised in RAN#105 [2]. RAN2#129bis meeting is the first meeting of the normative phase, following are the related agreements [3]:

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In this contribution, we provide our views on the open issues related OD-SIB1.

Discussion

Access related information in WUS configuration

In last meeting, RAN2 discussed on access related information, including feature-specific cell barring information and cell access related information in WUS configuration, but there’s no consensus:

9. Access related information in UL WUS R2-2502541 Discussion on On-demand SIB1 Qualcomm Incorporated discussion Proposal 3: RAN2 discusses the indication of feature-specific cell barring for a NES cell with on-demand SIB1. R2-2502916 On demand SIB1 handling Nokia, Nokia Shanghai Bell discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access. [Ericsson, ZTE, NTTDCM]: Prefer not to have this optimization. WUS configuration can be more updated often than access related information in SIB1. [Huawei, Apple, Google, Vivo, Xiaomi, Samsung]: Support the proposal. [Huawei]: Assume no big additional signaling overhead to include access related information. [Lenovo]: Additional information is not small including PLMN id, TAC, and other access related information. [Nokia]: This information will be defined as optional, so if NW does not want it, it can be omitted (NW choice). [Vivo]: Wonder if we need all information. [Ericsson]: If we put this information, it will make a list of cells associated with WUS configuration more difficult. [CMCC]: RAN1 already decided not to include barring information into WUS. [Samsung]: Barring is RAN2 scope. [Huawei]: We can only consider feature-specific cell barring in WUS since it needs only a few bits. [Nokia]: It does not make sense. Should be common to all access related information. Offline among interested companies. Comeback in CB session (Nokia). [Comeback session]: [Nokia]: Understand companies are not strongly against making this information optional. However there are more details we need to consider for determination. [Lenovo]: NPN, slice, blabla.. so many IEs are related to access related information. Understand it is not so simple. [Nokia]: Understand slice does not impact on access determination. [Lenovo, Ericsson]: Have different view than Nokia. Revisit it next meeting.

This issue was also discussed in RAN1, and following are the agreements made in RAN1#120 meeting[4]:

Conclusion (amended as shown in red in Friday session) - The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for FDD system. - RA-RNTI is not included in the UL-WUS configuration for FDD system. - Cell barring information is not included in the UL-WUS configuration for FDD system. - Cell selection information is not included in the UL-WUS configuration for FDD system. - Accessible UE types is not included in the UL-WUS configuration for FDD system. - CarrierBandwidth (for MPR calculation) is not included in the UL-WUS configuration for FDD system. - InitialUplinkBWP is not included in the UL-WUS configuration for FDD system. - TotalNumberOfRA-Preambles is not included in the UL-WUS configuration for FDD system. - SIB1-RequestResourceRedCap is not included in the UL-WUS configuration for FDD system. - SupplementaryUL is not included in the UL-WUS configuration

Observation 1: RAN1 agreed to not include cell barring information and cell selection information.

In our view,  feature-specific cell barring information can be included in OD-SIB1 WUS configuration while cell access related information is not needed. As known, both feature-specific barring information and cell access related information are include in SIB1 currently, and for feature-specific barring information is used to indicated whether a kind of UE, for example Redcap UE, or Rel-18 NES UE is barred or not in this cell, while cell access related information indicates cell access related information for this cell. Since the OD-SIB1 request procedure is used to acquire SIB1 by the UE, and then the UE camps or further accesses to the cell. Therefore, we think the information that whether the UE is barred or not is needed before OD-SIB1 request procedure, while how to access is not urgent. Besides, considering the WUS payload size, cell access related information may bring bigger signalling overhead than feature-specific barring information, which is not expected.

Proposal 1:Feature-specific barring information can be included in OD-SIB1 WUS configuration.

Proposal 2: Cell access related information is not included in OD-SIB1 WUS configuration.

NES Cell information in WUS configuration

In RAN2#129 meeting, RAN2 agreed to use a new SIB for WUS configuration delivery, but it’s not clear the  maximum number of WUS configurations. Considering that Cell A is more possibility to be deployed on lower frequency for better coverage, while NES cell is deployed on higher frequency as a hotspot. Therefore, it is likely that Cell A and NES cell are on the different carrier frequency. Based on this, the maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 3: The maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

As discussed above, the Cell A may have different carrier location with NES cell. Besides, RAN2 agreed to support the case that one Cell A with multiple NES Cell, it is possible that multiple NES cells are deployed with different  carrier location. Therefore, it is preferred to indicate NES cell in WUS configuration with ARFCN&PCI.

Proposal 4: The cell list in WUS configuration indicates the ARFCN&PCI of the NES cell.

For OD-SI, the maximum number of OD-SI resources is 32, which uses the same value of the maximum number of SI. As to maximum number of the OD-SIB1 resources, since the resource is only used to indicate the request of SIB1 to the NES cell, therefore we think there’s no need to provide too much resources for this purpose. What’s more, considering that WUS configuration can be configured as area specific, that one OD-SIB1 request resource can be shared among multiple cells, hence, we propose that the maximum number of OD-SIB1 resources can be the same with the number of PCIs sharing the same OD-SIB1 resources.

Proposal 5: The maximum number of OD-SIB1 resources can be considered as the same with the number of PCIs sharing the same OD-SIB1 resources.

Co-existence of OD-SIB1 and other features

- OD-SIB1 & SDT

In current RRC Running CR[5], OD-SIB1 procedure can be triggered by the UE while SDT procedure is not ongoing, and this follows the legacy behaviour of OD-SI procedure. When discussing the co-existence of SDT and OD-SIB1,  we think there’re two cases as following:

- UE’s SDT data/signalling transmission is performed in Cell A

- UE’s SDT data/signalling transmission is performed in NES cell

For the first case, UE is transmitting SDT data/signalling in Cell A, in our view, the UE will not trigger OD-SIB1 request procedure since the channel quality is good enough for UE’s SDT transmission. Therefore, SDT and OD-SIB1 can co-exist with each other in this case.

For the second case, the UE may acquire NES cell’s  SIB1 and camp on even access to the NES cell first and then transmit SDT data/signalling in NES Cell in RRC_INACITVE, and the UE may request OD-SIB1 due to SIB1’s validity issue, and in this case, we think current Running CR’s description is reasonable, since the interruption of SDT procedure should be avoided.

Proposal 6: Support the co-existence of OD-SIB1 and SDT, and OD-SIB1 procedure can only be triggered by the UE while SDT procedure is not ongoing.

- OD-SIB1 &SBFD

Sub-Band Full Duplex (SBFD) operation is supported for a TDD carrier, enabling simultaneous downlink transmission and uplink reception at the gNB on their respective sub-bands. And the related configuration is provided in SIB1 or dedicated signalling. The additional resources provided in SBFD can be used for RACH. If the co-existence of OD-SIB1 and SBFD is supported, more workload may be introduced to indicate SBFD resources in WUS configuration if SBFD resources are configured for OD-SIB1 request. What’s more, similar to OD-SIB1, SBFD is still under discussion in Rel-19, it’s better to not support the change of OD-SIB1 related configuration (e.g., WUS configuration) due to SBFD in Rel-19. But it doesn’t mean that SBFD configuration cannot be indicated in SIB1 acquired by UE’s request, since SIB1 content is the same with different acquisition method: broadcast or on-demand.

Proposal 7: The change of OD-SIB1 related configuration (e.g., WUS configuration) due to SBFD is not supported in Rel-19, and there’s no spec impact to indicate SBFD configuration in SIB1 acquired by UE’s on-demand request.

Conclusions

In this contribution, we analyse the OD-SIB1 related open issues , following are the proposals and observations:

Proposal 1:Feature-specific barring information can be included in OD-SIB1 WUS configuration.

Proposal 2: Cell access related information is not included in OD-SIB1 WUS configuration.

Proposal 3: The maximum number of configured inter frequency cells can be considered as the maximum  number of WUS configuration, i.e., 16.

Proposal 4: The cell list in WUS configuration indicates the ARFCN&PCI of the NES cell.

Proposal 5: The maximum number of OD-SIB1 resources can be considered as the same with the number of PCIs sharing the same OD-SIB1 resources.

Proposal 6: Support the co-existence of OD-SIB1 and SDT, and OD-SIB1 procedure can only be triggered by the UE while SDT procedure is not ongoing.

Proposal 7: The change of OD-SIB1 related configuration (e.g., WUS configuration) due to SBFD is not supported in Rel-19, and there’s no spec impact to indicate SBFD configuration in SIB1 acquired by UE’s on-demand request.

References

- RP-234065, New WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #102;

- RP-242354, Revised WID: Enhancements of network energy savings for NR, 3GPP TSG RAN Meeting #105;

- Report of 3GPP TSG RAN WG2 meeting #129bis;

- Report of 3GPP TSG RAN WG1 meeting #120;

- R2-250xxxx Running RRC CR NES Post129bis.