3GPP TSG-RAN WG2 Meeting #130                          R2-2504403

St.Julians, Malta, May 19th – 23rd , 2025

Agenda item:	8.4.3

Source: 	CMCC

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:	Discussion

- Introduction

At the RAN2#129bis meeting, we had the following agreements[1]:

- RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any).

- RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both.

- It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

- The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ.

- How to define LP-RSRP and LP-RSRQ is up to RAN1.

- The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review.

- Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

At the RAN2#129 meeting, we had the following agreements[2]:

- The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements.

- If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

At the RAN2#128 meeting, we had the following agreements[3]:

- The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

- FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

- FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

At the RAN2#127bis meeting, we reached the following agreements[4]:

Working assumption

- For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

- FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

At the RAN2#127 meeting, we reached the following agreements[5]:

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

- The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on the LR measurement results.

- Discussion

2.1 Clarification of Cases

In the LP-WUS discussion of RAN4, they listed several cases for measurement offloading/relaxation. In order to avoid duplication of work and conduct efficient discussions within a limited time, RAN2 should prioritize research based on the cases approved by RAN4.

Case #1 and #3 have been agreed by RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

RAN4 to further discuss Case #2:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

Based on the definitions of the cases mentioned above, we must clarify the scope of application for these three cases, as this is critical to ensure clarity in the following discussions.

For Case #1 and Case #2, according to RAN4 definitions, MR is turned off, meaning neighbor cell measurements are not required. Therefore, the thresholds for Case #1 and Case #2 should at least be greater than SIntraSearchP. Based on this, we first discuss whether to support Case #2. In urban environments, due to the irregular shapes of cells, the UE may suddenly detect a better cell at a corner, leading to a handover and thereby requiring a conservative configuration of SIntraSearchP. In rural environments, cells are typically regular circular shapes, and the network can configure a lower intra-frequency measurement trigger threshold to help the UE save power. As shown in figure 1:

Figure 1

So in our opinion, whether Case #2 is needed depends on the SIntraSearchP configured by the network. If SIntraSearchP is configured with a lower value, such as in rural environments, which means that the UE does not need to perform neighbor measurements in a larger area, then it is reasonable to support Case #2 as a supplement to Case #1 or to use Case #2 only. In short, Case #2 can be supported, but it depends on the network configuration.

Observation 1: Case #2 is a possible case depending on network configuration for various scenarios, e.g. urban environments and rural environments.

Proposal 1: For Case #1 or #2, the entry threshold is equal to or higher than SIntraSearchP.

Further, if both Case #1 and Case #2 are allowed to be configured, then the Case #2 region should be between the Case #1 threshold and SIntraSearchP.

For Case #3, RAN4 agreed that MR RRM relaxation scaling factor for serving cell/neighbour cell is 16, which is significantly larger than that of legacy Rel-16 UE power saving WI (i.e., 3).

RAN4#114 Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

Considering that LR cannot assist MR in performing neighbor cell measurement, in our opinion, R19 Neighboring cell measurement relaxation should have stricter evaluation conditions than R16. Therefore, the MR threshold in the entry condition of R19 Neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16.

Proposal 2: The threshold in the entry condition of R19 measurement relaxation should be higher than SSearchThresholdP in R16, and lower than SIntraSearchP.

Based on the above discussion, the relationship between Cases is shown in the following figure:

Figure 2 Clarification of Cases

2.2 Case #1

In the RAN2#129bis meeting, we supported the simultaneous configuration of Case #1 and Case #3. RAN4 has also defined the MR relaxation factor as 16.

Agreement: - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

Therefore, we need to consider the conditions for transitioning from Case #3 to Case #1 when configuring at the same time, which may be different from the Case #1 entry condition defined by RAN2. In our opinion, when the MR configures the relaxation factor to 16, the MR measurement results may not be timely and have limited accuracy. Therefore, the measurement results of LR should be used simultaneously to evaluate the entry conditions to improve reliability. Therefore, the conditions for transitioning from Case #3 to Case #1 can be defined as: MR and LR is above the configured threshold, where the LR threshold is mandatory.

Proposal 3: When serving cell relaxation and serving cell offloading are configured in one cell, the LR threshold of the entry condition of Case #1 is mandatory.

Proposal 4: The conditions for transitioning from Case #3 to Case #1 can be defined as: MR and LR are above the configured threshold.

Considering that the latency of starting MR is as high as hundreds of milliseconds, we should be very cautious about offloading serving cell measurements to LR. If a UE is in a high mobility state, it is likely to reach the cell edge quickly, so the UE should use MR to perform neighbor measurements to prepare for possible handovers. If the UE performs measurement offloading and turns off MR at this time, on the one hand, due to the extremely high startup delay of the MR, the UE is likely to miss the best handover opportunity; on the other hand, it will also cause unnecessary conversion power consumption. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state.

Proposal 5: UEs with high-mobility do not perform serving cell measurement offloading.

At the last meeting, RAN4 discussed how to handle high-priority frequency measurements in case #1 and came to the following conclusions:

Agreement: - Support relaxed higher priority frequency layer search at case 1 Tentative agreement: - For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. - Note: K2 is 60

According to tentative agreement, RAN4 tends to support high-priority frequency measurement with the legacy relaxation period, i.e. K2*Thigher_priority_search. However, in the traditional measurement mechanism in TS 38.133, the UE is only allowed to relax the high-priority frequency measurement with this period when the 'low mobility' condition is met. Therefore, we believe that the 'low mobility' condition should be introduced in R19 RRM measurement offloading (Case #1) to determine the high-priority frequency measurement period.

Proposal 6: Introduced 'low mobility' condition in R19 RRM measurement offloading (Case #1) to determine the measurement period of high priority frequency.

2.3 Case #3

For Case #3, we believe there are two key issues to focus on:

- The transition from Case #1 to Case #3. The original entry/exit criteria for Case #3 are simply defined as being above or below a certain threshold, which is inaccurate.

- The MR relaxation factor of 16 is very aggressive, so when defining the entry/exit conditions for Case #3, LR should be fully considered and its measurement results should be fully utilized to increase reliability.

Case #3 entry/exit condition when both Case #3 and Case #1 are configured

We would like to revisit the entry condition for Case #3, as we believe the conclusion from the RAN2 #128 and #129 meeting was not rigorous enough.

RAN2#128 Agreements - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

RAN2#129 Agreements - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

RAN4 has agreed on an MR relaxation factor of 16 for Case #3, which will significantly reduce the reliability of MR measurements. At the same time, the MR ramp-up delay in Case #1 is quite large and results in higher power consumption. Considering these two factors, we believe that the entry/exit criteria should follow the principle of "strict entry, relaxed exit."

Specifically, the entry conditions for Case #3 should consider two aspects. First, when the UE needs to initiate neighbor cell measurements and fallback from Case #1 to Case #3, the entry condition for Case #3 is equivalent to the exit condition of Case #1. According to the "relaxed exit" principle, the entry condition for Case #3 in this scenario can be defined as: Serving cell quality measured by MR or LR is lower than SIntraSearchP but higher than the configured threshold, e.g. RSRP and/or RSRQ.

Secondly, for the scenario where the UE enters Case #3 from the cell edge, following the "strict entry" principle, the entry condition for Case #3 can be defined as: Both serving cell quality measured by MR and LR are above the configured thresholds, e.g. RSRP and/or RSRQ(LR threshold is mandatory). As shown in Fig 3.

Figure 3 Case #3 entry/exit condition

Proposal 7: For case #3, when both Case #3 and Case #1 are configured, the entry condition for serving cell RRM relaxation can be defined as:

- Serving cell quality measured by MR or LR is lower than SIntraSearchP but higher than the configured threshold, e.g. RSRP and/or RSRQ; or

- Both serving cell quality measured by MR and LR are above the configured thresholds, e.g. RSRP and/or RSRQ(If LR threshold is configured).

As for the exit conditions, there is no need to define separate conditions, just define it as not meeting the entry conditions.

Proposal 8: No separate exit condition is needed, and the exit condition can be defined as failing to meet the entry condition.

Proposal 9: For case #3, when both Case #3 and Case #1 are configured, the exit condition for serving cell RRM relaxation can be defined as:

- Both serving cell quality measured by MR and LR are higher than SIntraSearchP, e.g. RSRP and/or RSRQ; or

- Serving cell quality measured by MR or LR is lower than the configured threshold, e.g. RSRP and/or RSRQ.

neighboring cells issue

In RAN2#127bis, we reached the following agreements:

RAN2#127bis Agreements Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

For the ‘UE with low mobility’ criterion, we think it is necessary. Usually we think that a high-speed mobile UE has a higher handover probability, so the UE needs to perform accurate neighbor cell measurement. However, the relaxation factor currently discussed by RAN4 is very large. If a high-speed UE performs R19 neighbor cell measurement relaxation, it will bring risks in terms of mobility.

Proposal 10: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

- Conclusions

In this contribution, we discussed the LP-WUS measurement relaxation/offloading in RRC_IDLE/INACTIVE mode, and following are our observations and proposals.

Observations:

Observation 1: Case #2 is a possible case depending on network configuration for various scenarios, e.g. urban environments and rural environments.

Proposals:

Proposal 1: For Case #1 or #2, the entry threshold is equal to or higher than SIntraSearchP.

Proposal 2: The threshold in the entry condition of R19 neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16, and lower than SIntraSearchP.

Proposal 3: When serving cell relaxation and serving cell offloading are configured in one cell, the LR threshold of the entry condition of Case #1 is mandatory.

Proposal 4: The conditions for transitioning from Case #3 to Case #1 can be defined as: MR and LR are above the configured threshold.

Proposal 5: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 6: Introduced 'low mobility' condition in R19 RRM measurement offloading (Case #1) to determine the measurement period of high priority frequency.

Proposal 7: For case #3, when both Case #3 and Case #1 are configured, the entry condition for serving cell RRM relaxation can be defined as:

- Serving cell quality measured by MR or LR is lower than SIntraSearchP but higher than the configured threshold, e.g. RSRP and/or RSRQ; or

- Both serving cell quality measured by MR and LR are above the configured thresholds, e.g. RSRP and/or RSRQ(if LR threshold is configured).

Proposal 8: No separate exit condition is needed, and the exit condition can be defined as failing to meet the entry condition.

Proposal 9: For case #3, when both Case #3 and Case #1 are configured, the exit condition for serving cell RRM relaxation can be defined as:

- Both serving cell quality measured by MR and LR are higher than SIntraSearchP, e.g. RSRP and/or RSRQ; or

- Serving cell quality measured by MR or LR is lower than the configured threshold, e.g. RSRP and/or RSRQ.

Proposal 10: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

- References

- R2-2502982, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others.

- draft R2-2501332, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- R2-2410912, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- draft R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD.