3GPP TSG-RAN WG2 Meeting #130	R2-2504409

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM measurement gaps/restrictions related enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following is captured in the LS to RAN1 and RAN2 (R4-2504972):

## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability.

And the following was agreed in RAN4 #114bis (R4-2504911):

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

Meanwhile, RAN4 does not preclude to update the number of 1 s if any issue will be identified in the future meeting.

In the same LS, RAN4 defines the following action for RAN2:

ACTION:  RAN4 respectfully asks RAN2 to implement the signaling of the above mentioned UAI reporting for XR.

In this Tdoc, we discuss signaling of UAI and related capability.

# 2	Discussion

## 2.1	Signaling of UAI for measurement gap skipping

In UAI for measurement gap skipping, the UE indicates a ratio of occasions that can be skipped during a time period of 1 s. The following FFSs were highlighted during the [POST129bis][503][XR] RRC running CR and open issues:

- FFS when the UE should trigger UAI for assistance information for measurement occasion

- FFS what are the configurations for controlling UAI reporting assistance information for measurement occasion ratio

Before discussing configuration for controlling UAI reporting and triggering conditions, one aspect that needs clarification is whether the recommended skipping ratio reported by the UE is per UE or per MG configuration.

As mentioned in the UAI discussion in other working groups, the UE’s preference for skipping ratio may depend on several conditions. As an example, under good SNR conditions, less samples are needed for completing a measurement. However, those conditions may be different for different measurement objects. Considering that multiple measurement gaps can be configured, and that for each measurement gap there is a set of associated measurement objects, a single report in the UAI may not be enough. If a single report is applied and the UE is configured with more than one measurement gap, it becomes unclear how the gNB should interpret the recommended skipping ratio R indicated by the UE. Therefore, it is proposed that the UAI allows the UE to report individual recommendations for the configured measurement gaps. As an example, 2 values for R are reported if the UE is configured with gapFR1 and gapFR2, and if the UE is configured with GapConfig-r17, a different R value is reported for each measGapId-r17.

Proposal 1: UAI allows the UE to report individual recommendations for the configured measurement gaps.

Related to UAI configuration and triggering conditions, our view is that the UE should only be configured to report UAI for measurement occasion skipping ratio if it is also configured with at least one measurement gap configuration.

The UE should trigger UAI reporting with its preference for MG cancellation whenever configured to do so and if recommended skipping ratio R has changed as compared to a previously signalled value. Taking the example above, this could happen e.g. in case the SINR conditions on one or several cells measured using a MG configuration change, or in case a new measurement object is configured that uses the corresponding MG configuration.

Similarly to when configuring UAI reporting the UE’s preference for DRX configuration, a prohibit timer should be configured for assistance information reporting of MG cancellation preference. The prohibit timer is configured and maintained per UE. That is whenever UAI reporting including the UE’s preference for MG cancellation is transmitted, the UE includes the recommended skipping ratios for all the configured measurement gaps. Upon transmission of the UAI, the prohibit timer is started and the UE is not allowed to transmit a new UAI including the UE’s preference for MG cancellation if the corresponding timer is running.

In summary, the IE used to configure UAI reporting assistance information for measurement gap cancellation includes a prohibit timer.

Proposal 2: the configuration controlling UAI reporting assistance information for measurement occasion consists in configuring the UE to provide its preference for measurement gap skipping ratio and with a value of the corresponding prohibit timer.

Proposal 3: a UE capable of providing assistance information related to skipping of measurement occasions shall initiate the procedure when configured to do so, upon change of the recommended skipping ratio for at least one of the configured measurement gaps, and if the corresponding prohibit timer is not running.

## 2.2	UE capability for UAI

For the following open issue from the capability email discussion, as agreed in RAN4 #114bis, support of UAI is optional for a UE, with optional UE capability. Though this needs to be confirmed by RAN4, our understanding is that the capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

Open issue 3: whether RAN2 to define the UE capability for UAI for a ratio of gap occasions In RAN4 LS R4-2504972 / R2-2503328, RAN4 indicates the agreement to define UAI for a ratio of gap occasions, and “Supporting of UAI is optional for UE with optional UE capability." Typically such capability should be defined in RAN4 UE feature list. However, this feature is special that RAN4 will not specify UE behavior / requirements or gNB behavior / requirement in response to UAI. RAN2 might need discussion on whether to define this capability by RAN2.

Proposal 4: support of UAI for gap cancellation ratio is optional with UE capability and RAN2 assumes it is per UE capability unless indicated otherwise from RAN4.

# 3	Conclusion

Remaining issues on RRM measurement gaps/restrictions related enhancements are discussed in this contribution with the following observations/proposals proposed:

Proposal 1: UAI allows the UE to report individual recommendations for the configured measurement gaps.

Proposal 2: the configuration controlling UAI reporting assistance information for measurement occasion consists in configuring the UE to provide its preference for measurement gap skipping ratio and with a value of the corresponding prohibit timer.

Proposal 3: a UE capable of providing assistance information related to skipping of measurement occasions shall initiate the procedure when configured to do so, upon change of the recommended skipping ratio for at least one of the configured measurement gaps, and if the corresponding prohibit timer is not running.

Proposal 4: support of UAI for gap cancellation ratio is optional with UE capability and RAN2 assumes it is per UE capability unless indicated otherwise from RAN4.