3GPP TSG-RAN WG2 Meeting #130	R2-2504411

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	DSR enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129bis:

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

In this contribution we discuss open issues on DSR enhancements from the last meeting and those that were raised during the running CR review process.

# 2	Discussion

## 2.1	Reporting control and retransmission data

The exact UE behavior for reporting control and retransmission data is still to be decided to address the FFS from last meeting and the Editor’s note in the running CR.

During DSR data volume calculation the remaining time of retransmitted [data] is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

For the purpose of MAC delay status reporting, the transmitting PDCP entity shall evaluate the delay-reporting PDCP data volume in ascending order of dsr-ReportingThreshold, and consider the following as delay-reporting PDCP data volume associated with the i:th dsr-ReportingThreshold: - the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the delay-reporting PDCP SDUs associated with the i:th dsr-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th dsr-ReportingThreshold where k < i; - [if i = 1], the PDCP Control PDUs; - [if i = 1], for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - [if i = 1], for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. Editor’s Note: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

Control PDUs have no associated remaining time, i.e. they cannot trigger a DSR. For what concerns retransmissions, a pending DSR is cancelled when all PDCP SDUs associated with a pending DSR have been discarded or when a MAC PDU is sent which includes all the PDCP SDUs associated with a pending DSR. I.e., it cannot happen that, when the MAC PDU containing a DSR MAC CE is to be built, there is a pending DSR for an LCG, and the corresponding LCG only has control or retransmission data to report. This is the same as in legacy.

Observation 1: if an LCG is with only control PDUs and/or retransmission SDUs/PDUs, there can be no DSR pending for that LCG, hence the LCG is not reported in the DSR (as in legacy).

In case there is delay-reporting data to be reported for an LCG, the main open issue is whether control and retransmission data should be included in the DSR in the buffer size corresponding to the smallest reported or the smallest configured reporting threshold for that LCG. Using the smallest reported threshold guarantees that two additional octets do not need to be included in the DSR if there is no data to be reported for that LCG in correspondence of the smallest configured threshold. This results in reduced overhead.

Proposal 1: PDCP control PDUs and retransmission data are included in the buffer size calculation corresponding to the smallest reported threshold.

## 2.2	Rel-19 DSR capability dependency on Rel-18 DSR

On the open issue of Rel-19 DSR capability dependency on Rel-18 DSR:

Agreement => An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18. Open issue 1: the pre-requisite of enhanced delay status report

Functionalities wise, we do not see why a UE supporting Rel-19 DSR would have difficulty to support Rel-18 DSR as well. Besides, the UE should be able to handle the case of not all the LCGs being configured with separate reporting threshold.

Proposal 2: a UE supporting enhancedDelayStatusReport-r19 shall also support delayStatusReport-r18.

Proposal 3: it is possible to configure some LCGs with reporting threshold while some other LCGs without and the Rel-19 DSR format is used if any LCG is configured with reporting threshold.

The following should be added to Remaining Time and Buffer Size description in the MAC running CR:

- Remaining Time: In the Single Entry DSR MAC CE or for the Multiple Entry DSR MAC CE when reporting the Buffer Size of an LCG that is not configured with any reporting threshold, this field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. In the Multiple Entry DSR MAC CE, the field Remaining Time i,j indicates the shortest remaining time among the PDCP SDUs associated with the reporting threshold j of LCG i, as specified in clause 5.15 in TS 38.323 [4], at the time of the first symbol of the first PUSCH transmission that includes this Multiple Entry DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec; - Buffer Size: In the Single Entry DSR MAC CE or for the Multiple Entry DSR MAC CE when reporting the Buffer Size of an LCG that is not configured with any reporting threshold, the Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. In the Multiple Entry DSR MAC CE, the field Buffer Size i,j indicates the total amount of delay-reporting data associated with the reporting threshold j of LCG i, according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [4] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBS-TableAllowed and the amount of data to be reported by this field is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits.

## 2.3	DSR cancellation in DC

DSR cancellation in DC has been raised as an open issue in MAC CR review based on input in R2-2501763. In summary, the main issue is whether, in case a DSR is triggered on both MAC entities and it is cancelled because a DSR MAC CE is sent and the DSR MAC CE includes delay status information of all the PDCP SDUs associated with a pending DSR, whether the DSR should be cancelled only on the MAC entity where the DSR MAC CE is transmitted (legacy behaviour) or on both MAC entities. The proposals in the above reference Tdoc are:

- Re-19: NW configuration based on UE capability signaling whether to cancel the DRS on one or both MAC entities

- Rel-18: UE beahvior is unspecified, and left for UE implementation

First, it should be observed that in Rel-18, triggering of DSR on both MAC entities can be controlled by the configuration of ul-DataSplitThreshold. A DSR is triggered on both MAC entities only if the amount of data available for transmission is above ul-DataSplitThreshold. Therefore, we see no need to introduce a new configuration to indicate whether the UE should cancel the DSR on both MAC entities. The NW can configure ul-DataSplitThreshold accordingly if it wants to avoid that a DSR is triggered on both MAC entities in the first place.

Similar to BSR, both gNBs should be able to get the DSR and have the possibility to schedule the data. Transmission of DSR to one entity cancels DSR to another entity should never be the intention. Therefore, we do not see the logic to introduce such capability/configuration.

Leaving this to UE implementation is especially not desirable as the NW may want to receive delay status information on both SCGs.

Proposal 4: when in DC, if a DSR is triggered on both MAC entities and it is cancelled because a DSR MAC CE is sent and the DSR MAC CE includes delay status information of all the PDCP SDUs associated with a pending DSR, the DSR is cancelled only on the MAC entity where the DSR MAC CE is transmitted (legacy behaviour).

# 3	Conclusion

Remaining issues on DSR enhancements are discussed in this contribution with the following observations/proposals proposed:

Observation 1: if an LCG is with only control PDUs and/or retransmission SDUs/PDUs, there can be no DSR pending for that LCG, hence the LCG is not reported in the DSR (as in legacy).

Proposal 1: PDCP control PDUs and retransmission data are included in the buffer size calculation corresponding to the smallest reported threshold.

Proposal 2: a UE supporting enhancedDelayStatusReport-r19 shall also support delayStatusReport-r18.

Proposal 3: it is possible to configure some LCGs with reporting threshold while some other LCGs without and the Rel-19 DSR format is used if any LCG is configured with reporting threshold.

Proposal 4: when in DC, if a DSR is triggered on both MAC entities and it is cancelled because a DSR MAC CE is sent and the DSR MAC CE includes delay status information of all the PDCP SDUs associated with a pending DSR, the DSR is cancelled only on the MAC entity where the DSR MAC CE is transmitted (legacy behaviour).