3GPP TSG-RAN WG2#130	     					       R2-2504416

Malta, May 19 – 23, 2025

Agenda item:	8.1.3

Source: 	Kyocera

Title: 	Remaining Issues on NW side data collection

Document for:	Discussion

Introduction

In this contribution, the remaining issues regarding NW side data collection, Structure and Control of Availability Indication and Timestamp Structure for AI/ML-related Logging are discussed.

Discussion

Structure and Control of Availability Indication

Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases

At RAN2#129-bis, the following agreements were made [1].

RAN2#129-bis Agreements Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined Agreements on data collection configuration - The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

At the previous RAN2 meeting, agreements were reached regarding the conditions under which the UE can send an availability indication to the network. As noted in the agreements, in practical scenarios, the UE may be configured with multiple data collection sets and use cases simultaneously (e.g., Beam Management, AI/ML Mobility, etc.).
In such cases, the network may either handle logging separately for each use case or collectively regardless of the specific use case.
If the network intends to manage log collection separately for each use case, it is necessary to consider introducing a mechanism to identify and associate logs accurately with the corresponding use cases.

The following two alternatives regarding how the network should handle logging could be considered:

- Alt 1 (Recommended):
The network does not distinguish between different use cases or configurations when handling logs but rather treats all availability indications and logs collectively.
→In this alternative, the UE does not need to include any use case-specific identifier in the availability indication.

- Alt 2:
The network distinguishes between different use cases or configurations when handling logs.
→In this alternative, the UE needs to include an identifier indicating the corresponding set or use case in the availability indication. The format, structure, and assignment method of the identifier would need to be further studied in RAN2.

Since Alt 1 simplifies UE operation, reduces signaling overhead, and enables consistent management on the network side, Alt 1 is recommended.

Accordingly, we propose the following:

RAN2 should agree to handle all logs collectively without distinguishing them based on use cases or configurations.

Open issue RRC-19: Reporting assistance information related to logged measurements

In previous agreements, it was stated that the availability indication may include the following elements:

- Data is available

- Reason for trigger (full buffer, threshold)

- Low power indication

Additionally, the availability indication may include a flag explicitly indicating that data is available.

However, the logical relationship among these elements are not clearly defined.

In particular:
It is unclear whether "data is available" should be treated as a standalone flag or as something inferred from the trigger reasons.

Regarding "Low power", it should be interpreted as a separate state-related condition describing the UE’s operational status, which does not require to be associated with a reason for trigger.

Treating both "data is available" and "low power" as independent information elements allows the following benefits:

- A clear separation between availability state, its triggering reasons, and the UE's operating condition

- Unambiguous interpretation by the network

- Flexibility for future expansion (e.g., adding more state indicators beyond "low power")

If “data is available” is indicated as true, then at least one of the availability-related reasons (i.e., full buffer, threshold) shall be included.

In contrast, the “Low power” indication may be reported regardless of whether data is available.

RAN2 should agree that both “data is available” and “low power” shall be treated as semantically independent information elements in the availability indication.

Timestamp Structure for AI/ML-related Logging

Open issue RRC-21: Time related content of logged data

At RAN2#129-bis, it was agreed that the network should be made aware of whether there is a gap between two consecutive samples in the temporal domain, while the amount of the gap and whether the indication is implicit or explicit were left as FFS [1]. In this section, we discuss this FFS item.

RAN2#129-bis Agreement Content of collected data: For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit

Furthermore, the procedures for NW-side data collection include both periodic and event-based logging as agreed in the following meetings [2] [3]:

RAN2#127bis Agreement Periodic logging is supported for training data collection procedure in R19

RAN2#129 Agreement Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log

In the case of periodic logging, the start time and sampling interval can be derived from the configuration, making additional timestamp signalling unnecessary. The samples are taken at fixed intervals and the timestamp can be implicitly determined by the NW.

In contrast, for event-based logging, the timing of when the logging is triggered (hereafter referred to as the "Enter time") cannot be inferred by the network, as it depends on internal measurement conditions (e.g., L3 event detection at the UE). However, the duration or termination point of the logging session (hereafter referred to as the "Leave time") can often be derived from configuration (e.g., stop condition), or is less critical.

Therefore, only the absolute timestamp for the Enter time is needed for the network to correctly interpret the temporal position of the log. This minimizes signalling overhead while preserving the necessary time domain information.

RAN2 should agree to introduce an absolute timestamp only for the Enter time of event-based logging.

Conclusion

This contribution discusses the issues in terms of NW side data collection. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should agree to handle all logs collectively without distinguishing them based on use cases or configurations.

Proposal 2	RAN2 should agree that both “data is available” and “low power” shall be treated as semantically independent information elements in the availability indication.

Proposal 3	RAN2 should agree to introduce an absolute timestamp only for the Enter time of event-based logging.

References

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#129-bis

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#127-bis

- “RAN2 Chair’s Notes”, (InterDigital), RAN2#129