3GPP TSG RAN WG2 #130			R2-2504419

St Julians, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SIB1

Agenda Item:	8.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#105 meeting, the new work item of “Enhancement of network energy savings for NR” was agreed [1]. The following agreements have been made in RAN2 so far [2]-[6]. In this contribution, we provide our views on remaining issues of on-demand SIB1 for idle/inactive mode UEs.

Agreements on OD-SIB1@RAN2#127

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

Agreements on OD-SIB1@RAN2#127bis

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

Agreements on OD-SIB1@RAN2#128

1.	NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

2.	New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

3.	Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

4.	Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

5.	The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

6.	The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

7.	If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

8.	A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no cor-responding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

9.	UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE ac-quired WUS configuration. FFS on SI validity area rather than cell level.

10.	Working assumption: UL-WUS configuration in RRC release is not supported.

Agreements on OD-SIB1@RAN2#129

1.	There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

2.	Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

3.	The UE follows the legacy validity principle of stored SIB.

4.	For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

5.	SIB-x can be cell specific or area specific, as legacy.

6.	Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

Agreements on OD-SIB1@RAN2#129bis

1.	NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

2.	When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

3.	Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

4.	SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

5.	Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

6.	The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

7.	Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

8.	Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

9.	If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

10.	WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

11.	Area id will be used as legacy.

12.	We do not need a separate new triggering condition of OD-SIB1 acquisition.

13.	When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

- Discussion

2.1	Issue of 38.304-1

In the TS38.304 CR rapporteur’s summary, the following remaining issues were identified [7].

Issue 1: FFS whether the UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided. [Issue description] Some companies proposed a new UE behavior on handling excluded cell list. Their view is that NW may not configure Rel-19 excluded cell list if there is no excluded cell for NES UE in real deployment. In this case, they think that the NES UE also needs to ignore the legacy excluded cell list because NES Cell is included in legacy IE to bar the legacy UE. Thus, they propose that the NES UE always ignores the legacy excluded cell lists received from a cell in which SIBxx is provided, irrespective of whether dedicated excluded cell lists being provided.

On issue 1, we think it is worth considering the case where the legacy excluded cell list contains only NES cells, and NW does not want to include any (normal) cells in the Rel-19 excluded list.

In our understanding, the new excluded lists were introduced because NES cells should be excluded for legacy UEs being not capable of requesting OD-SIB1 to avoid unnecessary evaluation on frequency of NES cells, while the NES cells are not set to be in excluded list for new UEs with OD-SIB1 request capability. From operators’ perspective, there may be a possible scenario where NW does not want to exclude any normal cells for legacy UEs while wanting to deploy NES cells. We are open to discussing solutions to achieve the above operation including that NES UEs always ignore the legacy excluded cell list or NES UEs can refer to Rel-19 excluded cell list that is configured as empty.

Proposal 1: (38.304-1)

- It should be possible configurations that Rel-19 UEs being capable of OD-SIB1 are not provided with any excludes cell lists while legacy UEs are provided with only NES cells as excluded cell lists

  - The following solutions can be considered:

    - NES UEs always ignore the legacy excluded-cell list

    - NES UEs refers to Rel-19 excluded-cell list that is configured as empty

2.1	Issue of 38.304-2

Issue 2: FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304. [Issue description] Some companies proposed to explicitly capture the failure case of OD-SIB1 window expiry in 38.304 according to RAN2#129b agreement. And they proposed to add the failure condition in 38.331. - If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

Regarding issue 2, we do not see any need to explicitly capture that. As discussed in previous RAN2 meeting, we think that UE behavior should be consistent when the acquisition of OD-SIB1 fails regardless of failure causes. In this sense, we do not see any merit to capturing all scenarios where a UE fails to receive OD-SIB1.

Besides, the UE behavior for monitoring PDCCH during the OD-SIB1 window is expected to be performed in PHY layer, so this type of description can be captured in RAN1 spec., if needed. If RAN1 decides to explicitly capture UE behaviors when PHY layer at UE cannot receive requested OD-SIB1 during time window, and the PHY layer indicates failure to higher layer, RAN2 can consider adding subsequent UE behavior after indicated from lower layer.

Proposal 2: (38.304-2)

- There is no need to explicitly capture the failure case of OD-SIB1 window expiry in 38.304 unless RAN1 specifies that PHY layer indicates it to higher layer.

- Conclusion

In this contribution, we provided the following observations and proposals of on-demand SIB1 operation for idle/inactive mode UEs network energy saving.

Proposal 1: (38.304-1)

- It should be possible configurations that Rel-19 UEs being capable of OD-SIB1 are not provided with any excludes cell lists while legacy UEs are provided with only NES cells as excluded cell lists

  - The following solutions can be considered:

    - NES UEs always ignore the legacy excluded-cell list

    - NES UEs refers to Rel-19 excluded-cell list that is configured as empty

Proposal 2: (38.304-2)

- There is no need to explicitly capture the failure case of OD-SIB1 window expiry in 38.304 unless RAN1 specifies that PHY layer indicates it to higher layer.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#127bis meeting, Chairman’s notes

- 3GPP, RAN2#128 meeting, Chairman’s notes

- 3GPP, RAN2#129 meeting, Chairman’s notes

- 3GPP, RAN2#129bis meeting, Chairman’s notes

- R2-2503709, Summary report of [POST129b][113][NES] 38.304 CR (Apple), Apple (Rapporteur)