3GPP TSG-RAN WG2 #130	R2-2504427

St. Julian’s, Malta, May 19th– 23rd, 2025

Agenda Item:	8.5.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions for NES

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. In this paper, we address the third objective given in the revised WID as:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss the following aspects:

- adaptation of SSB signals,

- adaptation of PRACH occasions and related signaling, and

- adaptation of paging.

# 2	Discussion

### 2.1 Adaptation of SSB signals

RAN2 made the following agreements regarding this objective in the last meeting:

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

As captured in the open issue list, RAN2 needs to capture the procedural text and the corresponding RRC configuration of L3 RRM for SSB adaptation. This requires further input from RAN1/RAN4, so we suggest waiting for further input from those WGs before finalizing the stage 3 aspects.

RAN2 requires further input from RAN1/RAN4 before finalizing the stage 3 aspects for this objective.

### 2.2 Adaptation of random-access occasions

In this section we discuss the remaining aspects concerning the configuration of additional PRACH resources and related signalling.

##### 2.2.1 Configuration of additional PRACH resources

In RAN2, several options have been considered. However, no consensus was reached, and the question is captured as an open issue in [7] “FFS: How to configure additional RACH resource for RACH adaptation in RRC, with consideration of RAN1 input”. We address this open issue taking into consideration the following RAN1 agreements:

Agreement (RAN1#116bis):

- Support adaptation mechanisms of PRACH in time-domain for following:

  - UE in idle/inactive mode

  - UE in connected mode

Agreement (RAN1#118)

- For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

  - At least DCI based adaptation is supported. No introduction of new DCI format.

Agreement (RAN1#120bis):

- For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

  - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources.

    - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

According to the agreement from RAN1#120bis meeting, additional PRACH resources are to be configured within RACH-ConfigCommon in SIB1. Whether it is within the same RACH-ConfigCommon as for legacy PRACH resources remains to be decided. In our view, however, this decision falls under the expertise of RAN2.

To answer the question of where to configure the additional PRACH resources, we first look into the list of parameters provided by RAN1 so far. We provide the full list of parameters in the Appendix A, together with the ASN.1 examples of counterpart parameters that already exist in legacy.

Upon reviewing the RAN1 list of parameters for configuring additional PRACH resources, we observe that some of the so-far agreed parameters have counterpart legacy parameters that are distributed over two current IEs. For example, several parameters (e.g., PRACH-Config Index, msg1-FDM, msg1-FrequencyStart) have a counterpart legacy parameter included in RACH-ConfigGeneric, while the ssb-perRACH-OccassionAndCB-PreamblesPerSSB parameter has a counterpart legacy parameter included in RACH-ConfigCommon.

Furthermore, considering the entirely new parameters introduced for the configuration of additional PRACH resources (e.g., prach-subsetMask-Index-Adaptation) and the parameters that RAN1 can agree upon in the next meeting, we conclude that simplicity should guide us when implementing new signalling in the specifications. It would be beneficial to collect all parameters related to the configuration of additional PRACH resources into one IE. The candidate parameter to enable this is the parameter addl-RACH-Config-Adaptation, already agreed by RAN1, for the configuration of additional RACH resources for DCI-based adaptation of RA occasions in time domain. This would also enable a simpler addition of new additional PRACH resource configuration parameters in the future. Therefore, we propose the following:

Group all newly introduced parameters for configuration of additional PRACH resources in the same IE, e.g., addl-RACH-Config-Adaptation.

Next aspect that needs to be addressed concerns the placement of this new parameter addl-RACH-Config-Adaptation that collects all parameters related to the configuration of additional PRACH resources.  If included into the SIB1 directly, the adaptation of additional PRACH resources may not be available in some scenarios, where dedicated configuration is needed. On the contrary, if the adaptation of additional PRACH resources is included in the existing RACH-ConfigCommon, it will be available both in SIB1 and RRCSetup/Reconfiguration/Resume messages, and hence the network will have the flexibility to configure additional PRACH resources both for UEs in idle/inactive and connected mode. Considering this and the above agreement from RAN1#120bis, we propose to include addLRACH-Config-Adaptation in the existing RACH-ConfigCommon IE. This approach enables dynamic adaptation of additional PRACH resources in all relevant scenarios.

Include addI-RACH-Config-Adaptation IE in the existing RACH-ConfigCommon IE.

##### 2.2.2 Signalling aspects related to additional RA resources

The network can activate time domain RA adaptations for UEs in RRC connected or idle/inactive modes dynamically. As agreed in the last meeting, this would be backwards compatible as legacy UEs will use legacy RA resources per legacy configuration.

Activation of additional PRACH occasions dynamically is supported for UEs in connected and idle/inactive modes.

Dynamic activation of additional RA resources can be achieved via system information update. However, this would be a slow mechanism, and it requires to wake up UEs unnecessarily. For idle/inactive UEs, dynamic activation can be indicated at paging occasions (POs), in paging DCI, i.e., DCI 1_0 with P-RNTI as agreed in RAN1#120 and RAN1#120bis meetings, and/or occasions where SIB1 transmissions are made with the assumption that UEs that receive a paging message or acquire SIB1 prior to triggering RA can utilize those additional RA occasions.

For UEs in idle/inactive mode, additional RA occasions can be activated at paging occasions via DCI 1_0 with P-RNTI as agreed in RAN1.

For UEs in idle/inactive mode additional RA occasions can be activated at SIB1 transmission occasions.

Assuming that adaptive RA configuration includes multiple RA transmission occasions, a Rel-19 NES UE may choose any occasion to transmit a PRACH preamble. RAN2 agreed on introducing adaptive paging occasions and as discussed above we think that it would be beneficial for UEs in idle/inactive mode if additional RA occasions are activated at paging occasions. This is since it is very likely that the paged UE, if it is camping in that cell, will trigger the random-access procedure. When paging occasions are adapted and thus become more condensed, it is possible that multiple UEs are paged within a short period of time. These UEs would then trigger the random-access procedure almost simultaneously assuming that a UE typically initiates network access via the closest possible RA occasion. To avoid collusion in such cases it would be beneficial if gNB assigns a particular instance of the activated adaptive RA resources when paging a UE so that the load due to random access can be balanced. Please see the figure below.

It would be beneficial if gNB assigns a particular instance of the activated adaptive RA resources when paging a UE to balance the load.

…

Adaptive RA resources

gNB assigns the RA resources to the paged UE

Time

SSB transmission

Paging frames/occasions

Figure 1: Assignment of adaptive RA resources

Note that such assignment, i.e., regarding which additional RA resource should be used, does not impact the performance from latency standpoint considering that the UE is paged ahead of its otherwise legacy paging occasion, when configured with adaptive paging occasions.

An activated adaptive RA resource can be assigned to an adaptive paging frame/occasion.

Concerning the UEs in connected mode, one of the questions that was raised during the RAN2#129bis meeting is how often a UE should monitor for the indication in case of DCI-based adaptation indication of additional RA resources. In our opinion, this question can be addressed similar as in legacy.

If a UE is a public warning capable UE, it monitors the DCI-based adaptation indication at least once every defaultPagingCycle, otherwise at least once every SI modification period.

Finally, concerning LS that RAN1 sent to RAN2 about the use of one bit from the Bits 5-8 within the Short Message field (c.f., the RAN1#120bis agreement below), we propose to use the first available bit, i.e., bit 5.

Agreement (RAN1#120bis):

- For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

  - Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

  - Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Use bit 5 within the Short Message field for indication in DCI format 1_0 with P-RNTI.

Adopt the TP below in the running 38.331 CR.

6.5	Short Message

Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short Messages

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 | systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period. |
| 5 | adaptiveRAResources If set to 1: Indication of activation for adaptive RA resources. |
| 6 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

### 2.3 Adaptation of paging

In this section we discuss the remaining aspects concerning the signalling overhead associated with N and Ns parameters, and the aspects concerning the co-existence of PEI and paging adaptations.

### 2.3.1 Signalling overhead and extended values of N and Ns parameters

Below, we evaluate the message size when Ns for Rel-19 UEs is set to 8 and compare the outcome with the message size of legacy configuration as baseline. In the following example, for the sake of evaluation, we assume a SIB1 configuration with hardly anything but paging configuration in all scenarios.

- Legacy baseline scenario: T = 128, N = T/16, Ns = 4 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs), and four instances of firstPDCCH-MonitoringOccasionOfPO parameter is configured for SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 2.

Figure 2: Legacy baseline scenario (T = 128, N = T/16, Ns = 4, and SCS =15 kHz).

- Option b) scenario: T = 128, N = T/16, for legacy UEs Ns = 4 and for Rel-19 UEs Ns = 8 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs for legacy UEs and 8 POs for Rel-19 UEs). We consider two cases with SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO) and SCS = 480 kHz (i.e., corresponds to the case sCS480KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 3.

Figure 3:  Option b) scenario (T = 128, N = T/16, Ns for legacy = 4, Ns for Rel-19 UEs = 8 and SCS =15 kHz).

In Table 1, we provide the message sizes for the scenarios above. We assumed that the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined in the same way as in legacy (i.e., the range of firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs is given in symbols).

Table 1: Message sizes for different paging configurations (symbol level configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs)

|  | Message size [octets] |
| --- | --- |
| Legacy T = 128, N = T/16, Ns = 4, SCS = 15 kHz (baseline) | 28 |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 15 kHz | 41 |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 480 kHz | 50 |

According to the results above in Table 1, option b) with SCS = 15 kHz and option b) with SCS = 480 kHz have 13 and 22 octets larger message size than the legacy (baseline) configuration, respectively. This accounts for approximately 46% and 79% increase in message size. One can observe that option b) introduces significant signalling overhead even for the case where Ns for Rel-19 UEs is set to 8, according to the latest RAN2 agreements.

Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Based on the results in Table 1 above, we think that it would be beneficial to reduce the impact of option b) on the message size. This can be simply achieved by defining the range of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level instead of symbol level as currently specified for legacy UEs.

Note that we are not proposing to change the meaning of firstPDCCH-MonitoringOccasionOfPO, but its resolution only (i.e., the unit of firstPDCCH-MonitoringOccasionOfPO parameter would still be monitoring occasions).  This would allow us to define the range of the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs to be the same as of the firstPDCCH-MonitoringOccasionOfPO parameters for legacy UEs but with fewer bits.

In Table 2, we summarize the evaluation for message sizes for two scenarios with option b) in which firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined at the slot level.

Table 2: Message size for different paging configuration scenarios (slot level configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs)

|  | Message size [octets] |
| --- | --- |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 15 kHz | 37 |
| Option b) T = 128, N = T/16, Ns_leg = 4, Ns_r19 = 8, SCS = 480 kHz | 46 |

Based on the results in Table 2 above, the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets smaller message size than the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level, which corresponds to approximately 31% reduction of message size.

The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO is configured at slot level.

Assuming that firstPDCCH-MonitoringOccasionOfPO parameter is configured at slot level for Rel-19 NES UEs, the “FFS on firstPDCCH-MonitoringOccasionOfPO for paging adaptations” from [5], can be addressed in the ASN.1 as follows:

firstPDCCH-MonitoringOccasionOfPOExt-r19   CHOICE {

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-SCS15KHZoneThirtySecondT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..319),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT-SCS30KHZoneThirtySecondT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..639),

sCS480KHZquarterT-SCS120KHZoneSixteenthT-SCS60KHZoneThirtySecondT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1279),

sCS480KHZoneEighthT-SCS120KHZoneThirtySecondT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2559),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..5119)

sCS480KHZoneThirtySecondT  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..10239)

}                                                                                OPTIONAL  -- Need R

Regarding the “FFS on field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions” captured in [5], we observe that for all possible values of ssb-periodicityServingCell  in the field description of pagingAdaptationNAndPagingFrameOffset IE, N can also be set to oneThirtySecondT value.

For all possible values of ssb-periodicityServingCell in the field description of pagingAdaptationNAndPagingFrameOffset IE, the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) can be set to oneThirtySecondT value.

In [6], the ASN.1 change related to the “FFS on field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions” can be captured as highlighted below:

pagingAdaptationNAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT, oneThirtySecondT} - for ssb-periodicityServingCell of 160 ms, N can be set to {oneSixteenthT, oneThirtySecondT} If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT } If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT, oneThirtySecondT}

### 2.3.2 Paging adaptations and PEI

When it comes to PEI configuration parameters, let us recall the following from the TS 38.331:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

PEI-Config field descriptions

payloadSizeDCI-2-7 Payload size of PEI DCI, i.e., DCI format 2_7. The size is no larger than the payload size of paging DCI which has maximum of 41 bits and 43 bits for licensed and unlicensed spectrums, respectively.

pei-FrameOffset Offset, in number of frames from the start of a reference frame for PEI-O to the start of a first paging frame of the paging frames associated with the PEI-O, see TS 38.213 [13], clause 10.4A.

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

Furthermore, let us recall the following from Section 7.21 in TS 38.304:

“If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N.”

According to the current specifications, max # of paging occasions per PF is four, max # of PFs associated with one PEI monitoring occasions is two, and if one PEI occasion is associated with POs of two PFs, the two PFs are consecutive PFs.

Current specifications need to be updated in order to support co-existence of PEI and paging adaptations, i.e. Ns = 8.

For Rel-19 PEI, increase the maximum number of paging occasions per PF to 8.

### 2.3.3 Rel-19 PEI configuration parameters

RAN2 made the following agreements in the last meeting:

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

According to the agreements above, Rel-19 PEI independently addresses the additional Rel-19 POs. In what follows we discuss through which parameters the agreed separation of additional Rel-19 PEI transmission occasions from legacy ones can be supported.

The separation of additional Rel-19 PEI transmission occasions from legacy ones can be achieved by configuring PEI-PO-Offset and/or pei-SearchSpace differently (including firstPDCCH-MonitoringOccasionOfPEI-O) for the legacy and Rel-19 configuration, respectively.

Separation of additional Rel-19 PEI transmission occasions from legacy ones can be achieved through different PEI-PO-Offset and/or pei-SearchSpace configurations, including firstPDCCH-MonitoringOccasionOfPEI-O.

This discussion is also related to “FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?” that was raised in [5], let us recall the pei-ConfigBWP-r17 IE.

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}                                                                                           OPTIONAL     -- Cond InitialBWP-Paging

pei-ConfigBWP Provides the configuration for PEI reception in this BWP. If the field is absent, the UE does not receive PEI in this BWP. For the initialDownlinkBWP-RedCap not including CD-SSB and the entire CORESET#0, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing monitors PEI in the initialDownlinkBWP that includes CORESET#0, if the initialDownlinkBWP is configured with pei-ConfigBWP.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Paging | This field is optionally present, Need R, if this BWP is the initialDownlinkBWP or initialDownlinkBWP-RedCap including CD-SSB and the entire CORESET#0, and pei-Config is configured in DownlinkConfigCommonSIB. Otherwise, this field is absent. |

It is in this structure that pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O are configured. Without these parameters, the UE will not be able to decode the legacy PEI. For Rel-19 PEI, one way to separate the occasions for legacy PEI transmission from occasions for Rel-19 PEI transmission is exactly through different configurations of these parameters. Therefore, we think that it is also necessary to introduce a pei-ConfigBWP-r19 for configuring the corresponding parameters for Rel-19 PEI.

To support separation of additional Rel-19 PEI transmission occasions from legacy ones, introduce pei-ConfigBWP-r19 for configuration of Rel-19 pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

In summary, for Rel-19 PEI configuration, we think the following parameters are needed. Their value ranges align with those of the Rel-17 PEI counterparts, except for firstPDCCH-MonitoringOccasionOfPEI-O-R19, where we think that the appropriate range should be in line with ongoing discussions on condensed POs and firstPDCC-MonitoringOccasionOfPO.

- po-NumPerPEI-r19

- payloadSizeDCI-2-7-r19

- pei-FrameOffset-r19

- subgroupsNumPerPO-r19

- subgroupsNumForUEID-r19

- firstPDCCH-MonitoringOccasionOfPEI-O-R19 [range TBD, depending on ongoing discussions on firstPDCCH-MonitoringOccasionOfPO]

- pei-SearchSpace-R19

- nrofCandidates-PEI-R19

The value ranges of the above parameters align with those of the Rel-17 PEI counterparts, except for firstPDCCH-MonitoringOccasionOfPEI-O-R19, where we think that the appropriate range should be at the slot level, i.e., in line with ongoing discussions on condensed POs and firstPDCCH-MonitoringOccasionOfPO.

When it comes to the parameter lastUsedCellOnly-r17 we don’t think there is need for a new counterpart, and the Rel-17 parameter applies to both legacy and Rel-19 configurations.

For Rel-19 PEI configuration, introduce the following parameters:
-po-NumPerPEI-r19, 
-payloadSizeDCI-2-7-r19, 
-pei-FrameOffset-r19,
-subgroupsNumPerPO-r19,
-subgroupsNumForUEID-r19,
-pei-SearchSpace-R19,
-nrofCandidates-PEI-R19, 
-firstPDCCH-MonitoringOccasionOfPEI-O-R19

For Rel-19 PEI configuration, use the same value ranges as for legacy PEI for the following parameters:
-po-NumPerPEI-r19, 
-payloadSizeDCI-2-7-r19, 
-pei-FrameOffset-r19,
-subgroupsNumPerPO-r19,
-subgroupsNumForUEID-r19,
-pei-SearchSpace-R19,
-nrofCandidates-PEI-R19

For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPEI-O is configured at slot level with the same value range as for legacy PEI.

# 3	Conclusion

In this contribution we discuss the objective on the adaptation of common signal/channel transmissions. Based on the discussion in the previous sections we made the following observations:

Observation 1	RAN2 requires further input from RAN1/RAN4 before finalizing the stage 3 aspects for this objective.

Observation 2	Activation of additional PRACH occasions dynamically is supported for UEs in connected and idle/inactive modes.

Observation 3	For UEs in idle/inactive mode, additional RA occasions can be activated at paging occasions via DCI 1_0 with P-RNTI as agreed in RAN1.

Observation 4	It would be beneficial if gNB assigns a particular instance of the activated adaptive RA resources when paging a UE to balance the load.

Observation 5	Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Observation 6	The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

Observation 7	According to the current specifications, max # of paging occasions per PF is four, max # of PFs associated with one PEI monitoring occasions is two, and if one PEI occasion is associated with POs of two PFs, the two PFs are consecutive PFs.

Observation 8	Current specifications need to be updated in order to support co-existence of PEI and paging adaptations, i.e. Ns = 8.

Observation 9	Separation of additional Rel-19 PEI transmission occasions from legacy ones can be achieved through different PEI-PO-Offset and/or pei-SearchSpace configurations, including firstPDCCH-MonitoringOccasionOfPEI-O.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Group all newly introduced parameters for configuration of additional PRACH resources in the same IE, e.g., addl-RACH-Config-Adaptation.

Proposal 2	Include addI-RACH-Config-Adaptation IE in the existing RACH-ConfigCommon IE.

Proposal 3	For UEs in idle/inactive mode additional RA occasions can be activated at SIB1 transmission occasions.

Proposal 4	An activated adaptive RA resource can be assigned to an adaptive paging frame/occasion.

Proposal 5	If a UE is a public warning capable UE, it monitors the DCI-based adaptation indication at least once every defaultPagingCycle, otherwise at least once every SI modification period.

Proposal 6	Use bit 5 within the Short Message field for indication in DCI format 1_0 with P-RNTI.

Proposal 7	Adopt the TP below in the running 38.331 CR.

Proposal 8	For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPO is configured at slot level.

Proposal 9	For all possible values of ssb-periodicityServingCell in the field description of pagingAdaptationNAndPagingFrameOffset IE, the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) can be set to oneThirtySecondT value.

Proposal 10	For Rel-19 PEI, increase the maximum number of paging occasions per PF to 8.

Proposal 11	To support separation of additional Rel-19 PEI transmission occasions from legacy ones, introduce pei-ConfigBWP-r19 for configuration of Rel-19 pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

Proposal 12	For Rel-19 PEI configuration, introduce the following parameters: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19,  -firstPDCCH-MonitoringOccasionOfPEI-O-R19

Proposal 13	For Rel-19 PEI configuration, use the same value ranges as for legacy PEI for the following parameters: -po-NumPerPEI-r19,  -payloadSizeDCI-2-7-r19,  -pei-FrameOffset-r19, -subgroupsNumPerPO-r19, -subgroupsNumForUEID-r19, -pei-SearchSpace-R19, -nrofCandidates-PEI-R19

Proposal 14	For Rel-19 NES UEs, firstPDCCH-MonitoringOccasionOfPEI-O is configured at slot level with the same value range as for legacy PEI.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

R2-2502150, “Comments to 38.331 CR for NES”, Ericsson, 2025 April 7-11, Wuhan, China.

R2-2502173, “Running RRC CR for enhancements for network energy efficiency”, Ericsson, 2025 April 7-11, Wuhan, China.

R2-2503607, “Running RRC CR for enhancements for network energy efficiency”, Ericsson, 2025 May 19-23, St. Julian’s, Malta.

# Appendix A

Collection of Rel-19 higher layers parameters list for adaptation of PRACH resources (post RAN1#120bis):

| Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- |
| [addl-RACH-Config-Dyn] addl-RACH-Config-Adaptation | New | Configuration of additional RACH resources for DCI based adaptation of RACH in time domain. |  | FFS within [the same] RACH-ConfigCommon in SIB1 |
| PRACH-Config Index | New | RACH configuration index for additional RACH resources for DCI based adaptation of RACH in time domain. | NTEGER (0..255) FFS: Value range | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |
| msg1-FrequencyStart | New | Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0, at least for 4-step RACH. | INTEGER (0..maxNrofPhysicalResourceBlocks-1), | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |
| addl-ssb-Periodicity | New | Additional SSB burst periodicity for the Scell. | same as ssb-periodicity, i.e. 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. FFS: number of additional SSB burst peridiocity value(s) for the Scell. X≤2 additional SSB burst periodicities from SSB burst periodicities in ssb-periodicity |  |
| [prach-Mask-SubsetIdentification-Dyn] prach-SubsetMask-Index-Adaptation | New | Single PRACH mask to identify the subset of the additional PRACH resources applicable at least for adaptation for DCI 1_0 with P-RNTI. | FFS | FFS |
| msg1-FDM | New | The number of PRACH transmission occasions FDMed in one time instance, at least for 4-step RACH. | same as msg1-FDM | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |
| ssb-perRACH-Occasion ssb-perRACH-OccasionAndCB-PreamblesPerSSB | New | Configuration of number of SSB per RO and CB preambles per SSB. | same as ssb-perRACH-Occasion.ssb-perRACH-OccasionAndCB-PreamblesPerSSB | Per [addl-RACH-Config-Dyn] Per addl-RACH-Config-Adaptation |
| [validityDurationForAddlRACHAdaptation] |  |  |  |  |
| [valueKforAssociationPatternPeriodsForPRACHsubsetMask] |  |  |  |  |

Counterpart legacy parameters corresponding to some of the parameters from the list above:

RACH-ConfigGeneric information element

-- ASN1START

-- TAG-RACH-CONFIGGENERIC-START

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

…

}

-- TAG-RACH-CONFIGGENERIC-STOP

-- ASN1STOP

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMON-START

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                       OPTIONAL,   -- Need M

…

}

-- TAG-RACH-CONFIGCOMMON-STOP

-- ASN1STOP