3GPP TSG-RAN WG2 Meeting #130	R2-2504435

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	8.7.4.2

Source:	Samsung

Title:	Remaining Issues on DSR enhancements in Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2 meeting, RAN2 made the following agreements for DSR enhancements:

Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

In this paper, we further discuss the essential aspects on the remaining issues of enhanced DSR.

Discussion

Control and Retransmitted Data Reporting

One the issue that how to associate the PDCP/RLC control and retransmitted data with the reporting threshold, among the two options, i.e., 1) the smallest reporting threshold that is reported, and 2) the smallest reporting threshold that is configured, it is more flexible and simpler to directly associate them to the smallest configured reporting threshold, without considering the delay status of other PDCP SDUs. There is no need of update for such association regardless of how the delay status of other PDCP SDUs evolve.

Proposal 1. For Rel-19 DSR, all PDCP/RLC control and retransmitted data is associated to the smallest configured reporting threshold, which is the i-th dsr-ReportingThreshold with i=1.

When it comes to remaining time field setting, it is unclear how the remaining time field should be set in the DSR MAC CE for the smallest configured reporting threshold, if the corresponding buffer size only contains PDCP/RLC control and/or retransmitted data, and so none of the data is associated with PDCP discard timer. We think this should be a rare case in practice and so no optimization is needed. A simple workaround would be setting the remaining time field to zero (distinct from the case when normal delay-reporting data is reported), in order for NW to easily figure out the situation and make proper decision in scheduling the control and retransmitted data.

Proposal 2. For Rel-19 DSR, when the smallest configured reporting threshold is associated with only PDCP/RLC Control and/or retransmitted data, set the remaining time field associated to the threshold to 0.

Configuration Constraint on Reporting Threshold

In RAN2#129, we made an agreement that

If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

while, when we review the agreement, it enforces NW to configure explicitly both triggering and reporting thresholds, even in the case that there is only one reporting threshold whose value is configured to the same value as that of triggering threshold.

From the signalling overhead point of view, it is more efficient to omit reporting threshold configuration, if there is only one reporting threshold and the value is same as triggering threshold, since we can just deem there is only one reporting threshold implicitly configured the value of the triggering threshold, if reporting threshold is not configured explicitly.

Note that in Rel-18 DSR, the triggering threshold is implicitly used as “reporting” threshold. Rel-19 DSR was expected to be enhanced with more advanced features, while the agreement serves to make Rel-19 DSR more inefficient.

Observation 1: The agreement that mandates explicit reporting threshold configuration introduces unnecessary duplicated configuration/signalling overhead, when there is only one reporting threshold configured with the same value as triggering threshold.

Proposal 3. In Rel-19 DSR, if an LCG with triggering threshold is not configured with reporting threshold, the triggering threshold is used as reporting threshold for the LCG.

Modeling of Non-Delay-Reporting Data ahead of Delay-Reporting Data

The latest PDCP running CR used the expression “will be transmitted prior to”, to avoid specifying all the detailed behaviors in determining the buffering order between two PDCP SDUs.

However, the expression “will be transmitted prior to” is rather vague and incurs more questions on how to interpret it, e.g., not clear whether it should be interpreted from PUSCH transmission time or MAC PDU assembly points of view.

Also, when implementing the UE behavior, it is challenging to anticipate the transmission order between two PDCP SDUs, before receiving the UL grants.

Moreover, if the intention is to let UE implementation judge whether a PDCP SDU “will be transmitted prior to” another PDCP SDU, a reasonable doubt can be raised for whether different UE implementations may result in different criteria, and thus, different buffer size to report, even with the same/similar buffer status. Since this feature directly assists UL grant size determination at NW, UE behavior should be unified across different UE vendors for fairness purpose.

With such consideration, COUNT-based modelling is better than the other options, since 1) it clearly captures the criteria on how to determine one SDU is ahead of another one, and 2) thus saving the efforts to interpret a vague condition in UE implementation, and thus 3) achieving a unified UE behavior across different UE vendors becomes feasible.

Observation 2: COUNT-based modelling is better than the other options in modelling the “ahead of” condition, since 1) it clearly captures the criteria on how to determine one SDU is ahead of another, and thus 2) saving the efforts to interpret a condition requiring more interpretation, and 3) achieving a unified UE behavior across different UE vendors.

Proposal 4. Using COUNT-based modelling to model the “ahead of” condition between non-delay-reporting data and delay-reporting data.

Conclusion

Based on the discussion above, we have the following observation and proposals:

Proposal 1. For Rel-19 DSR, all PDCP/RLC control and retransmitted data is associated to the smallest configured reporting threshold, which is the i-th dsr-ReportingThreshold with i=1.

Proposal 2. For Rel-19 DSR, when the smallest configured reporting threshold is associated with only PDCP/RLC Control and/or retransmitted data, set the remaining time field associated to the threshold to 0.

Observation 1: The agreement that mandates explicit reporting threshold configuration introduces unnecessary duplicated configuration/signalling overhead, when there is only one reporting threshold configured with the same value as triggering threshold.

Proposal 3. In Rel-19 DSR, if an LCG with triggering threshold is not configured with reporting threshold, the triggering threshold is used as reporting threshold for the LCG.

Observation 2: COUNT-based modelling is better than the other options in modelling the “ahead of” condition, since 1) it clearly captures the criteria on how to determine one SDU is ahead of another, and thus 2) saving the efforts to interpret a condition requiring more interpretation, and 3) achieving a unified UE behavior across different UE vendors.

Proposal 4. Using COUNT-based modelling to model the “ahead of” condition between non-delay-reporting data and delay-reporting data.