3GPP TSG-RAN WG2 Meeting #130															R2-2504447

St Julian, Malta, 19th – 23rd May 2025

Agenda item:	8.11.3

Source:	Samsung

Title:	Other Aspects of SBFD

WID/SID:	NR_duplex_evo-Core

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss other aspects/objective which are not directly related to RA operation, e.g. DC support and CLI and CSI-RS measurement in SBFD.

Discussion

CLI measurement and reporting overall

It is necessary to measure and report CLI in symbol level time scale since SBFD configuration can be per symbol level. L3 time average blur the exact CLI level in smaller time scale so the L1 based CLI measurement operation for SBFD is needed. From our understanding, RAN1 considers this principle and we provide the overall procedure to support the L1 based UE-to-UE CLI measurement and reporting.

- Step 1: Exchange of resource information to measure CLI between gNBs

Victim gNB receives the SRS resource configuration that aggressor gNB will assign to the aggressor UE via Xn message which can be used for SBFD configuration delivery. Once victim gNB receives the SRS resource configuration for the aggressor gNB, it should adjust or translate the received resource from aggressor cell to the victim cell resource:

- Only overlapped resources can be used for CLI resource (inter-freq case);

- Each SRS or CLI resources should be indicated in one of serving cells for victim UE, and using DL BWP of that cell, which includes those SRS or CLI resource configuration.

- Above adjusted / translated resource information should be signalled to the victim UE by victim gNB

- Step 2: CLI measurement configuration at (victim) gNB

- Serving cell configures resources for CLI measurement to a specific UE (possible victim UE)

  - Measuring quantity: SRS-RSRP, CLI-RSSI same as legacy L3 based CLI resources.

    - SRS-RSRP: Victim UE will measure aggressor UE’s SRS transmission

    - CLI-RSSI: Victim UE will measure aggressor UE’s UL transmission plus any interference power on its DL reception resource.

      - This resource is determined by victim gNB and there is no explicit exchange of TX resource from aggressor gNB to the victim gNB.

    - Etc.

  - Each measurement resource would be either periodic resource, semi-persistent resource or aperiodic resource.

- Based on resource type above, measurement report is at least aperiodic report on PUSCH; FFS for other reporting (e.g. semi-persistent, periodic reporting)

The legacy CSI framework is consist of Report setting, Resource setting and Trigger state:

- Resource setting (CSI-ResourceConfig): what signal to use to compute CSI

- Report setting (CSI-ReportConfig): what CSI to report and when to report it

- Trigger state (CSI-AperiodicTriggerStateList): associates CSI report and CSI resource settings

Below sections provides the detail signalling aspect to support CLI measurement resource and report configuration using CSI framework.

CLI resource configuration

Based on RAN1 parameter list [4], RRC running CR [3] implements the CLI resource configuration i.e. two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI could be configured in CSI-ResourceConfig as below. For one csi-ResourceConfigId, only one type of CLI resource is configured using the CHOICE structure.

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]],

[[

cli-MeasurementResourceSetList-r19        CHOICE {

cli-RSSI-MeasurementResourceSetList    CHOICE {

CLI-RSSI-MeasurementResourceSetList-r19,

CLI-RSSI-MeasurementResourceSet-r19

},

srs-RSRP-MeasurementResourceSetList    CHOICE {

SRS-RSRP-MeasurementResourceSetList-r19,

SRS-RSRP-MeasurementResourceSet-r19

}

}

]]

}

However from our understanding, there is no reason to have CHOICE structure for cli-RSSI-MeasurementResourceSetList and srs-RSRP-MeasurementResourceSetList. We think only CLI-RSSI-MeasurementResourceSetList-r19 and SRS-RSRP-MeasurementResourceSetList-r19 are needed for cli-RSSI-MeasurementResourceSetList and srs-RSRP-MeasurementResourceSetList, respectively.

Proposal 1: For cli-RSSI-MeasurementResourceSetList and srs-RSRP-MeasurementResourceSetList, only CLI-RSSI-MeasurementResourceSetList-r19 and SRS-RSRP-MeasurementResourceSetList-r19 are needed i.e. no CHOICE structure for this fields.

In addition, there is one legacy mandatory field in (csi-RS-ResourceSetList) in CSI-ResourceConfig IE, but we think this field is not needed if the new cli-MeasurementResourceSetList is configured. So, the new restriction to ignore the legacy mandatory field (csi-RS-ResourceSetList) in CSI-ResourceConfig IE is needed if the new CLI measurement resource set is configured

Proposal 2: Additional restriction to ignore the legacy mandatory field (csi-RS-ResourceSetList) in CSI-ResourceConfig IE is needed if the new CLI measurement resource set is configured.

CLI report configuration

If we follow the legacy CSI report framework by L1 signaling, following approach could be reused for the L1 based UE-to-UE CLI measurement and reporting as well. If network determines the RS type, then it will further determine the report type available for the RS type. This measurement resource and report configuration consist of a single measurement instance, i.e., ID, which can be referred by the DL MAC CE/DCI for its corresponding operation.

Table 1. L1 based CLI measurement and report configuration

| Resource Configuration | Report Configuration |  |  |
| --- | --- | --- | --- |
| RS Configuration | Periodic report | Semi-Persistent report | Aperiodic report |
| Periodic RS | No dynamic triggering/activation | Reporting on PUCCH : Triggering by MAC CE Reporting on PUSCH : Triggering by DCI | Triggered by DCI, additionally by MAC CE |
| Semi Persistent RS | Not Supported | Reporting on PUCCH : Triggering by MAC CE Reporting on PUSCH : Triggering by DCI | Triggered by DCI, additionally by MAC CE |
| Aperiodic RS | Not Supported | Not Supported | Triggered by DCI, additionally by MAC CE |

However, only aperiodic CLI measurement report was agreed now based on the RAN1 and RAN2 agreements, and other reporting types (e.g. periodic, semi-persistent) are FFS. From our understanding, those types of CLI reporting are deprioritized in this release and they can be introduced if time allows (or find critical reason to support them).

For aperiodic CLI reporting on PUSCH, the legacy CSI-ReportConfig IE and CSI-AperiodicTriggerStateList IEs are used for CLI measurement report configuration based on RAN1 parameter list [4]

From our understading, the two new report entities for SRS-RSRP and CLI-RSSI could be added in reportQuantity, and some restrictions of usage of reportConfigType could be added in the field description, such that “for SBFD CLI measurement report configuration, only aperiodic is set”.

CSI-ReportConfig ::=                SEQUENCE {

reportConfigId                          CSI-ReportConfigId,

carrier                                 ServCellIndex                   OPTIONAL,   -- Need S

resourcesForChannelMeasurement          CSI-ResourceConfigId,

csi-IM-ResourcesForInterference         CSI-ResourceConfigId            OPTIONAL,   -- Need R

nzp-CSI-RS-ResourcesForInterference     CSI-ResourceConfigId            OPTIONAL,   -- Need R

reportConfigType                        CHOICE {

periodic                                SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUCCH                   SEQUENCE {

reportSlotConfig                        CSI-ReportPeriodicityAndOffset,

pucch-CSI-ResourceList                  SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource

},

semiPersistentOnPUSCH                   SEQUENCE {

reportSlotConfig                        ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

reportSlotOffsetList                SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32),

p0alpha                                 P0-PUSCH-AlphaSetId

},

aperiodic                               SEQUENCE {

reportSlotOffsetList                SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32)

}

},

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

Proposal 3: RAN2 agree below aspects for CSI-ReportConfig.

- New report entities for SRS-RSRP and CLI-RSSI are added in reportQuantity.

- Usage of reportConfigType could be added in the field description, such that “for SBFD CLI measurement report configuration, only aperiodic is set”.

For CSI-AperiodicTriggerStateList, the Figure 1 provides the simplified procedure for legacy aperiodic CSI report. Based on the legacy procedure, the new resource indicator for CLI measurement resource instead of resourceForChannel is needed if the legacy CSI-AperiodicTriggerStateList IE is reused for CLI aperiodic reporting configuration. In RRC running CR [3], the new resourcesForChannelCLI-r19 is added. In that case, the legacy mandatory resourceForChannel field should be ignored if the new resourcesForChannelCLI-r19 is is present.

Figure 1. CSI aperiodic triggering state

CSI-AperiodicTriggerStateList ::=   SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...,

[[

ap-CSI-MultiplexingMode-r17         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]],

[[

ltm-AssociatedReportConfigInfo-r18  LTM-CSI-ReportConfigId-r18                                    OPTIONAL  -- Need R

]]

}

CSI-AssociatedReportConfigInfo ::=  SEQUENCE {

reportConfigId                      CSI-ReportConfigId,

resourcesForChannel                 CHOICE {

nzp-CSI-RS                          SEQUENCE {

resourceSet                         INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info                            SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL  -- Cond Aperiodic

},

csi-SSB-ResourceSet                 INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfig)

},

csi-IM-ResourcesForInterference     INTEGER(1..maxNrofCSI-IM-ResourceSetsPerConfig)               OPTIONAL, -- Cond CSI-IM-ForInterference

nzp-CSI-RS-ResourcesForInterference INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)          OPTIONAL, -- Cond NZP-CSI-RS-ForInterference

...,

[[

resourcesForChannel2-r17        CHOICE {

nzp-CSI-RS2-r17                 SEQUENCE {

resourceSet2-r17                INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

qcl-info2-r17                   SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId

OPTIONAL   -- Cond Aperiodic

},

csi-SSB-ResourceSet2-r17        INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)

}                                                                                             OPTIONAL,  -- Need R

csi-SSB-ResourceSetExt          INTEGER (1..maxNrofCSI-SSB-ResourceSetsPerConfigExt)          OPTIONAL   -- Need R

]],

[[

resourcesForChannelTDCP-r18     SEQUENCE {

resourceSet2TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig),

resourceSet3TDCP-r18        INTEGER (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)              OPTIONAL  -- Need R

}                                                                                                     OPTIONAL,  -- Cond TDCP

applyIndicatedTCI-State-r18     CHOICE {

perSet-r18                      ENUMERATED {first, second},

perResource-r18                 SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF ENUMERATED {first, second}

}                                                                                                OPTIONAL, -- Cond ApplyIndicatedTCI

applyIndicatedTCI-State2-r18    CHOICE {

perSet-r18                     ENUMERATED {first, second},

perResource-r18                SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF  ENUMERATED {first, second}

}                                                                                                     OPTIONAL, -- Cond SecondCSICMR

csi-ReportSubConfigTriggerList-r18  CSI-ReportSubConfigTriggerList-r18                                OPTIONAL   -- Need R

]],

[[

resourcesForChannelCLI-r19      SEQUENCE {

resourceSetCLI-r19              INTEGER (1..maxNrofCLI-MeasurementResourceSetsPerConfig)                         OPTIONAL,  -- Need R

qcl-infoCLI-r19                 SEQUENCE (SIZE(1.. maxNrofCLI-MeasurementResourceSetsPerConfig)) OF TCI-StateId  OPTIONAL  -- Need R

}

]]

}

Proposal 4: Add the text to ignore the legacy mandatory field of resourceForChannel if the new resourcesForChannelCLI-r19 is present.

Since the new RRC configuration is added in the legacy CSI-AperiodicTriggerStateList, the legacy Aperiodic CSI Trigger State Subselection MAC CE is reused to indicates the selection status of the Aperiodic Trigger States configured for CLI measurement report, i.e. no new Aperiodic CSI Trigger State Subselection MAC CE is needed for CLI purpose.

Proposal 5: RAN2 confirm that the legacy Aperiodic CSI Trigger State Subselection MAC CE is also used to indicate the selection status of the Aperiodic Trigger States configured for aperiodic CLI measurement report.

In addition, the SRS configuration at the aggressor gNB is dynamic because this SRS configuration is dedicated configuration per UE. Plus, the SRS activation at the aggressor gNB would be dynamic because this SRS resource from other cells are not configured by periodic manner. In this case, the exchange of the information on the activation/deactivation of SRS at aggressor gNB is not easy to be exchanged with the victim gNB. For handling this, the information exchanged between gNBs can further information for this SRS configurations but this is not a scope of this release based on the WID as below.

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

Based on this information, RP meeting confirmed that the enhancement of SRS configuration coordination is not supported in Rel-19, it means that SRS CLI measurement on SP/AP resources would be only possible in intra-CU case (i.e., no Xn signaling needed) in this release.

Proposal 6: RAN2 confirm that exchange of the information on the activation/deactivation of SRS between gNBs is not supported in Rel-19.

CSI-RS measurement impact

From RAN4 RRM requiremements perstive, follow SSB based and/or CSI-RS based measurement requirements are specified as summarized in below table:

Table 1: SSB and/or CSI-RS related measurement in TS 38.133

| Serving cell RRM measurement |  |  | Neighbouring cell measurement |
| --- | --- | --- | --- |
| L3 RRM measurement | L1-BM (L1-RSRP, L1-SINR) with in CSI framework | L1 RLM L1 BFD/BFR | L3 RRM measurement |

For the RRM measurement impact, RAN1 made the below agreement for CSI reporting.

Agreement

For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options.

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

  - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only.

    - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD

Based on the above agreements, UE is configured with the separate CSI report on SBFD and non-SBFD, it means UE monitors the CSI-RS resources in SBFD and non-SBFD, separately and the results will be reported to the gNB. The reason why this independent CSI measurement/report is needed between SBFD and non-SBFD symbols is that the UE may experience different propagation characteristics between SBFD symbols and non SBFD symbols.

Then, the next question is how the RAN2 impacts if this separate CSI measurement and report for SBFD is introduced. We think following two features need to be considered:

- Issue 1: Separate BFD/BFR procedure for SBFD symbols

  - Main objective of BFD/BFR is checking the radio link monitoring and we think even the separate link quality is monitored by SBFD symbols, there will be legacy CSI monitoring for BFD/BFR

  - SBFD resources could be used for BFD/BFR configuration perspective, but we think the new procedure to handle this SBFD resources separately is not needed.

  - In [2], following proposals are provided, and it could be just NW configuration impact (i.e. no specification change is needed)

    - Keep the legacy BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger BFD and BFR for non SBFD symbols and SBFD symbols separately

    - The network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement i.e. SBFD symbols could be used for this RLM/BFD/CBD measurement

- Issue 2: Separate RRM measurement

  - Based on the above RAN1 agreements, L1 CSI-RS resources for SBFD symbols could be measured separately, then it is also possible the separate CSI-RS measurement is used for L3 measurement (i.e. RRM manner).

  - It is good for gNB to know the separate measurement report for SBFD in order to decide the mobility (i.e. HO), the how to support could be considered.

    - E.g. MeasObject and/or ReportConfig configures the separate CSI-RS resources in SBFD symbol which is different with the measurement in non-SBFD symbols.

  - In short, the measurement of L1 CSI-RS resources for SBFD symbols separately is already supported, then only the new thing is reporting the measured results for L3 RRM manner. We think this feature has no big spec. impact and RAN2 could decide it.

Proposal 7: RAN2 does not support the separate BFD/BFR procedure for SBFD symbols but below proposal in [2] could be used for enhancement of BFD/BFR for SBFD without specification impact.

- Keep the legacy BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger BFD and BFR for non SBFD symbols and SBFD symbols separately

- The network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement i.e. SBFD symbols could be used for this RLM/BFD/CBD measurement.

Proposal 8: RAN2 consider to support the separate Layer3 measurement report for CSI-RS resources in SBFD symbol.

Conclusion

Proposal 1: For cli-RSSI-MeasurementResourceSetList and srs-RSRP-MeasurementResourceSetList, only CLI-RSSI-MeasurementResourceSetList-r19 and SRS-RSRP-MeasurementResourceSetList-r19 are needed i.e. no CHOICE structure for this fields.

Proposal 2: Additional restriction to ignore the legacy mandatory field (csi-RS-ResourceSetList) in CSI-ResourceConfig IE is needed if the new CLI measurement resource set is configured.

Proposal 3: RAN2 agree below aspects for CSI-ReportConfig.

- New report entities for SRS-RSRP and CLI-RSSI are added in reportQuantity.

- Usage of reportConfigType could be added in the field description, such that “for SBFD CLI measurement report configuration, only aperiodic is set”.

Proposal 4: Add the text to ignore the legacy mandatory field of resourceForChannel if the new resourcesForChannelCLI-r19 is present.

Proposal 5: RAN2 confirm that the legacy Aperiodic CSI Trigger State Subselection MAC CE is also used to indicate the selection status of the Aperiodic Trigger States configured for aperiodic CLI measurement report.

Proposal 6: RAN2 confirm that exchange of the information on the activation/deactivation of SRS between gNBs is not supported in Rel-19.

Proposal 7: RAN2 does not support the separate BFD/BFR procedure for SBFD symbols but below proposal in [2] could be used for enhancement of BFD/BFR for SBFD without specification impact.

- Keep the legacy BFD and BFR procedure in the MAC layer, i.e., no need for the MAC entity to trigger BFD and BFR for non SBFD symbols and SBFD symbols separately

- The network configures the valid symbol type for CSI-RS based RLM/BFD/CBD measurement i.e. SBFD symbols could be used for this RLM/BFD/CBD measurement.

Proposal 8: RAN2 consider to support the separate Layer3 measurement report for CSI-RS resources in SBFD symbol.

References

- RP‑241614	Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD); RAN#104

- R2-2500885	CSI-RS measurements for RLM/BFD/CBD in SBFD		Ericsson

- R2-250xxxx	RRC running CR for Evolution of NR duplex operation: Sub-band full duplex (SBFD)		Rel-19	NR_duplex_evo-Core		Huawei.

- R1-2501646		Collection of Rel-19 higher layers parameters list		Post RAN1#120.