3GPP TSG-RAN WG2 Meeting #130	R2-2504457

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.7.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Closing the Loop on XR rate control

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Rel-19 uplink rate control aims at reusing the Recommended bit rate procedure as much as possible, but certain aspects such as table design and MAC CE design need to be modified to support QoS flow specific bit rate adjustment. RAN2 discussed such aspects and the following agreements were made in RAN2#129bis:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Moreover, in the list of Rel-19 XR open issues in MAC, the following rate control related issues have been identified:

- [MAC-04] Which type of ID should be used to identify a QoS flow, e.g. DRB ID + QFI ID or some other identifier - [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows - [MAC-06] Any additional fields that should be included in the Rate Control MAC CE - [MAC-07] Format of the Rate Control MAC CE - [MAC-08] Handling of triggered UL rate queries (e.g. multiplexing, transmission and cancelation, etc) - [MAC-09] How UE should handle the Rate Control MAC CE in DC configuration - [MAC-10] Behavior of bitRateQueryProhibitTimer - [MAC-11] Whether to apply the same design for UL rate control design to DL data transmission - [MAC-12] How to indicate in the Rate Control MAC CE that a query from UE is for available bit rate.

In this contribution, we address the open issues identified during the running CR discussion and the remaining issues from the last meeting.

# 2	Discussion

## 2.1	Rate Control MAC CE Design [MAC-04/ 05/ 06/ 07/ 11/ 12]

For the design of new MAC CE for QoS flow level rate control, we note that at least two fields are required to start with:

- ID information to identify a QoS flow;

- Bit rate index.

In RBR MAC CE, there is 6 bits of LCID field which identifies the logical channel for which the RBR or the RBR query is applicable. For uplink rate control, however, to identify QoS flow uniquely within the UE, QoS flow ID alone would not be sufficient because the QoS flow ID is uniquely assigned only within a PDU session. Moreover, from TS 38.331, the maximum number of QFI (maxNrofQFIs) is 64 and the maximum number of PDU session (maxNrofPDU-Sessions) is 256. In the extreme case, it would require total 14 bits – 6 bits for QFI and 8 bits for PDU session ID.

As an alternative, RAN2 briefly considered an approach using QFI and LCID together, which would require total 12 bits – 6 bits for QFI and 6 bits for LCID. As RAN2 has assumed to start with 8 bits of bit rate field, this approach results in 3 octets of Rate Control MAC CE for a single QoS flow:

Figure 1. Single entry Rate Control MAC CE using LCID and QFI for a QoS flow identification

Meanwhile, there were quite some interests in supporting multiple entry Rate Control MAC CE as the gNB is likely to adjust the bit rate of several QoS flows simultaneously under congestion situation. If the approach of using both LCID and QFI is applied as-is, the LCID would be repeated even for the QoS flows belonging to the same LCID, resulting in a 6-bit - or effectively almost 1-octet – additional overhead per QoS flow. Since the same MAC CE will also be transmitted over the uplink for query purpose, this overhead must be carefully considered. One possible option is to include the LCID only once for QoS flows that share the same LCID. For example, in the case of two QoS flows for rate control with the same LCID, this alternative would require 5 octets of MAC CE – 6 bits of LCID followed by two [6 bits for QFI + 8 bits for Bit Rate]. In contrast, including LCID for each QoS flow would result in 5 or 6 octets depending on whether to R bits are kept unused or not.

Since multiple entry Rate Control MAC CE already shows overhead savings even with just two QoS flows, it seems beneficial to adopt the multiple entry Rate Control MAC CE with a possibility to omit the LCID field for the QoS flows sharing the same LCID.

Proposal 1: Support Multiple entry Rate Control MAC CE using LCID and QFI for a QoS flow identification.

Proposal 2: In Multiple entry Rate Control MAC CE, the LCID field is 6 bits and is omitted for the rest of QoS flows belonging to the same LCID.

Additionally, we don’t think that all the QoS flows are applicable for rate control. In practice, only a small number of QoS flows within a logical channel are likely to require bit rate adjustment, e.g., one or two QoS flows. Thus, having 6 bits of QFI may be unnecessary and waste of resources. As an alternative, assuming that only a subset of QoS flows is subject to rate control, a bitmap-based approach could be considered. Considering practical deployment scenarios and byte alignment of MAC CE design, a 2-bit bitmap should be sufficient, enabling rate adjustment up to two QoS flows within a single logical channel.

Given that the UE and the gNB both know which QoS flows are subject to rate control, each bit of the bitmap can indicate a QoS flow in ascending order of the QoS flow ID of the QoS flows which are subject to rate control within the logical channel.

Proposal 3: In Multiple entry Rate Control MAC CE, a 2-bit bitmap is used to indicate a QoS flow to which the rate control is applied, in ascending order of the QoS flow ID of the QoS flows which are subject to rate control within the associated logical channel.

In RAN2#129bis, RAN2 agreed that, for bit rate field, the starting point is 8 bits with the understanding that overhead implications would be evaluated. One possible issue that could affect this decision is the support of DL rate control. However, since RAN2 as well as other WGs have not yet discussed in detail, it would be reasonable to exclude DL rate control in Rel-19. Therefore, we confirm the use of 8 bits of bit rate field. If DL rate control is to be supported, the bit rate field could be reduced to 7 bits since RAN2 understood that 7 bits is also sufficient, with 1 bit is used to indicate the direction, i.e., DL or UL.

It was also suggested during post email discussion on MAC running CR [Post129bis][502] that the meaning of Bit Rate should be differentiated for command from gNB and query from UE. Our understanding is that the suggestion was not to introduce a separate indication field in the MAC CE, but rather to clarify the distinction through a more detailed field description. However, the distinction between ‘recommended’ and ‘available’ may not be meaningful in practice since from UE perspective, a bit rate that is available is inherently considered recommended. Therefore, we do not see significant benefit in providing separate description for the command and query cases.

Proposal 4: In Multiple entry Rate Control MAC CE, an 8-bit bit rate filed is included unless DL rate control is supported. If DL rate control is supported, a 7-bit bit rate field and 1-bit Direction field are included for byte alignment.

With above proposals, the Multiple entry Rate Control MAC CE would look like below:

Figure 2. Multiple entry Rate Control MAC CE with LCID, bitmap of QF, and Bit Rate field

## 2.2	Query procedure

## 2.2.1	Query procedure handling [MAC-08]

During the post email discussion on MAC [Post129bis][502], one of the open issues is handling of triggered UL rate queries (e.g., multiplexing, transmission, and cancellation).

In RBR procedure:

-	an RBR MAC CE is generated for the pending query when the transmission is available and not prohibited by the bitRateQueryProhibitTimer;

-	the RBR query is cancelled when the corresponding RBR MAC CE is generated;

Such an approach is generally taken throughout MAC specification and can therefore also be applied to UL rate control query procedure:

-	an UL rate control MAC CE is generated for the pending query when the transmission is available and not prohibited by the bitRateQueryProhibitTimer;

-	the UL rate control query is cancelled when the corresponding RBR MAC CE is generated;

Proposal 5: For XR rate control, multiplexing, transmission, and cancellation are specified by taking the RBR query as a baseline.

### 2.2.1	prohibit timer behaviour [MAC-10]

In RAN2#129, it was assumed that rate query MAC CE is supported with the option to disable it via a prohibit timer, similar to legacy. The remaining question is whether the prohibit timer should be per QoS flow or per DRB. The prohibit timer serves two functions:

-	it prevents the UE from transmitting queries too frequently to the gNB by enforcing a configured prohibit interval.

-	it disables the query function when the prohibit timer is not configured since the UE cannot generate a MAC CE in that case.

During the post email discussion on RRC [Post129bis][503], companies showed a preference for aligning the granularity of the prohibit timer with that of the MAC CE. However, if the goal is to prevent the UE from sending queries too frequently, then configuring the prohibit timer at the DRB level seems to be sufficient. One primary purpose of the query was to allow the UE for XR – different from voice service – to request an increased bit rate back when congestion is alleviated. Once the gNB receives a query for any QoS flow within a DRB, the gNB can adjust the uplink bit rate of all QoS flows in accordance with the current congestion situation, similar to the initial adjustment.

Even when the purpose is to disable or enable the query function, there seems to be no clear advantage to disable/enable the query function at the QoS flow level. Therefore, it is deemed sufficient to configure the prohibit timer at the DRB level.

Proposal 6: For XR rate control, the prohibit timer for query function is defined/configured at the DRB level (as legacy).

In the Recommended Bit Rate (RBR) procedure, the prohibit timer – bitRateQueryProhibitTimer – is defined per logical channel and direction. The bitRateQueryProhibitTimer starts when the Recommended bit rate MAC CE is generated so that the MAC entity does not generate another MAC CE in a short period. While we aim to reuse the RBR procedure, there’s one issue worth considering given the role of the prohibit timer. When the gNB transmits an uplink rate control MAC CE to the UE, it would be less efficient for the UE to query soon, as it could still be a congestion situation and may only cause a ping-pong effect. Therefore, it would be beneficial to prevent the UE from querying in such case.

Proposal 7: For XR rate control, RAN2 discusses whether the prohibit timer starts or restarts when the UE receives the uplink rate control MAC CE from the gNB.

## 2.3	Table design

One FFS from the MAC running CR is whether the index 0 in the bit rate table can be used for any specific purpose, i.e., no new recommendation bit rate is given as in the legacy RBR table.

For the legacy bit rate table in Table 6.1.3.20-1, RAN2 agreed in R2-1818771 that the index 0 indicates that no new recommendation bit rate is given as in LTE. In LTE, RAN2 agreed on the bit rate table based on R2-1701652. It was explained that there may be some cases where the gNB currently has no recommended bitrate available, or just indicates that the previous bitrate value is no longer valid without any specific recommendation.

In our view, such indication is unnecessary because:

-	The UE behaviour is not defined when there is no recommended bit rate or it indicates the previous value is no longer valid. Sending a command with no expected UE behaviour could be just waste of radio resources.

-	If the gNB has not determined a proper uplink bit rate even under congestion, the best approach would be to send nothing. If the gNB must send something, e.g., to indicate continuous congestion situation, it would be sufficient to retain the previous value. For this purpose, there is no need to assign a separate index as the gNB can send the same index as the previous one.

Proposal 8: In the table of bit rate values, there is no need to assign the index 0 to indicate either no desired bit rate or invalidity of the previous value.

## 2.4	Scenarios with dual connectivity [MAC-09]

One of the identified open issues is: [MAC-09] How UE should handle the Rate Control MAC CE in DC configuration. In the scenarios with dual connectivity (DC) and split bearer, the data from a single DRB or QoS flow is sent over multiple cells e.g. MCG and SCG. When congestion occurs either in MCG or SCG or both, the UE may receive the recommended bit rate MAC CE with updated bit rate from at least one of the connected cells in case no coordination between MCG and SCG. In this case, due to different congestion situations in the connected cells, UE may or may not need to change the bit rate at application layer.

As another possibility, it is also feasible that only MCG sends the updated recommended bit rate MAC CE to cover the latest recommended bit rate from both cells. From UE point of view, it might be easier to determine whether to change the application bit rate or not based on a single MAC CE.

In our view, the first approach – each of MCG and SCG independently sends its own uplink bit rate – is recommended as it avoids complex coordination between the two gNBs. Furthermore, since the actual adjustment at the UE side is left to implementation, the UE may be able to adjust the bit rate more effectively by having detailed knowledge of which paths is more congested.

Proposal 9: For XR rate control in DC scenario, the MCG and SCG sends the available bit rate independently without any coordination. It is up to UE how to utilize such information.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Support Multiple entry Rate Control MAC CE using LCID and QFI for a QoS flow identification.

Proposal 2: In Multiple entry Rate Control MAC CE, the LCID field is 6 bits and is omitted for the rest of QoS flows belonging to the same LCID.

Proposal 3: In Multiple entry Rate Control MAC CE, a 2-bit bitmap is used to indicate a QoS flow to which the rate control is applied, in ascending order of the QoS flow ID of the QoS flows which are subject to rate control within the associated logical channel.

Proposal 4: In Multiple entry Rate Control MAC CE, an 8-bit bit rate filed is included unless DL rate control is supported. If DL rate control is supported, a 7-bit bit rate field and 1-bit Direction field are included for byte alignment.

Proposal 5: For XR rate control, multiplexing, transmission, and cancellation are specified by taking the RBR query as a baseline.

Proposal 6: For XR rate control, the prohibit timer for query function is defined/configured at the DRB level (as legacy).

Proposal 7: For XR rate control, RAN2 discusses whether the prohibit timer starts or restarts when the UE receives the uplink rate control MAC CE from the gNB.

Proposal 8: In the table of bit rate values, there is no need to assign the index 0 to indicate either no desired bit rate or invalidity of the previous value.

Proposal 9: For XR rate control in DC scenario, the MCG and SCG sends the available bit rate independently without any coordination. It is up to UE how to utilize such information.