3GPP TSG-RAN2 Meeting #130	R2-2504461

St.Julians, Malta,  May 19th – 23rd , 2025

Source: 			ZTE Corporation

Title: 	Discussion on data collection for AI mobility

Agenda item:	8.3.4

Document for: 	Discussion and Decision

- Introduction

In the last meeting, we discussed the LCM and spec impact for AI mobility and achieved the following agreements:

1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management - Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) - The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference - For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details - For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details - Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP.

In this document, we’d like to share our view on the data collection for network side model and UE side model.

- Discussion

### 2.1. Data collection for UE side model

    - Data collection for model training

Regarding the training data collection for UE-side model, some agreements were made in the rel-19 AI-phy study, as follows:

RAN2 #129: Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: a. The network can decide when to start/stop the data collection and send configuration. b. The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication). RAN2 #129bis: - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request

Generally, the AI-phy training data collection procedure for AI-phy can be taken as baseline, which mainly include:

- The UE can request measurement configuration for data collection.

- The network can provide or release the data collection configuration (at any point in time), with or without UE request.

In the AI-phy, the UE request can contain preferred configuration from a list of candidate configurations provided by the network. The similar mechanism can be applied to AI-mobility.

Observation 1: For UE side data collection in AI-PHY, the network can configure a list of candidate configurations to the UE, and the UE can indicate the preferred configuration from the list of candidate configuration provided by the NW, this mechanism can be applied to AI-Mobility.

For AI-mobility, the candidate configurations shall include the frequency used for input of the trained model and the frequency used for output of the trained model. For each frequency, the network may provide one or more measurement configuration, e.g. the cell/beam that the UE can collect the data on, measurement timing configuration.

Proposal 1: For UE side data collection in AI mobility, the network can provide a list of candidate data training configuration to the UE, and the UE reports its preferred candidate configuration from the indicated list to the network (same as in AI-phy).

Proposal 2: Each candidate data training configuration provided by the network can include:

- The frequency used for the input of the UE side AI model;

- The frequency used for the output of the UE side AI model;

- For each frequency, the measurement resource configuration, including the cell/beam information that the UE can collect the data on and the measurement timing configuration.

Upon receiving the preferred candidate configuration from the UE, the network can determine what to configure at end, same as in AI-phy. This is because different UE may have different preferrence, and the network may be not able to satisfy the request of all the UE(s).

Proposal 3: In AI mobility, it is up to network to decide the final configuration for data collection for UE sided model (same as in AI-Phy).

### 2.2. Data collection for network side model

In the last meeting, we have agreed that the AI-phy network-side data collection procedure, configuration and reporting framework can be considered as baseline for AI-mobility. The general procedure for training data collection includes:

- The UE performs measurement and logs the measurement results based on the network configuration;

- The UE reports logged data when the network requests it (via UEInformationRequest/UEInformationResponse)

Similarly, in AI-mobility, the network shall explicitly configure the UE data collection configuration, and the UE performs the measurement and stores the data based on the received network configuration. The data collection configuration can include the following configurations:

- The frequency whose measurement results needs to be logged;

- The type of logged measurement results (e,g, L1 filtered beam results, L3 filtered cell results)

- The expected L1/L3 measurement interval;

Proposal 4: For the network side model, the training data collection configuration can include:

- The frequency whose measurement results needs to be logged;

- The type of logged measurement results (e,g, L1 filtered beam results, L3 filtered cell results)

- The expected L1/L3 measurement interval;

- Conclusion and proposals

Observation 1: For UE side data collection in AI-PHY, the network can configure a list of candidate configurations to the UE, and the UE can indicate the preferred configuration from the list of candidate configuration provided by the NW, this mechanism can be applied to AI-Mobility.

Proposal 1: For UE side data collection in AI mobility, the network can provide a list of candidate data training configuration to the UE, and the UE reports its preferred candidate configuration from the indicated list to the network (same as in AI-phy).

Proposal 2: Each candidate data training configuration provided by the network can include:

- The frequency used for the input of the UE side AI model;

- The frequency used for the output of the UE side AI model;

- For each frequency, the measurement resource configuration, including the cell/beam information that the UE can collect the data on and the measurement timing configuration.

Proposal 3: In AI mobility, it is up to network to decide the final configuration for data collection for UE sided model (same as in AI-Phy).

Proposal 4: For the network side model, the training data collection configuration can include:

- The frequency whose measurement results needs to be logged;

- The type of logged measurement results (e,g, L1 filtered beam results, L3 filtered cell results)

- The expected L1/L3 measurement interval;

- Reference

- Report of RAN2 129 meeting.