3GPP RAN WG2 Meeting #130	R2-2504463

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.1.2.3

Source:	HONOR

Title:	Discussion on LCM for Positioning Use Case

Document for:	Discussion and Decision

1. Introduction

In Rel-19 AI/ML for NR air interface WID, the following objectives were achieved:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In RAN2#129bis meeting, the following agreements were made:

Agreements for positioning case 1 - LMF is responsible for functionality management - UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control. - Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation". - An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. - For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)

In this contribution, we discuss the remaining issues on AI/ML positioning.

2. Discussion

2.1 (issue LPP-16) Signalling of Monitoring Outcome

In RAN1#116-bis, the following agreements on performance monitoring were achieved:

RAN1#116-bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Regarding this issue, further discussions were held in the subsequent meetings, and the following conclusions were reached in RAN1#119 and RAN 1#120-bis meeting:

RAN1#119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: • Option A. The target UE side performs monitoring metric calculation. o The target UE may signal the monitoring outcome to the LMF. o FFS: content of monitoring outcome • FFS: Option B RAN#120bis Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: • the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

For monitoring metric calculation in UE side, the corresponding assistance information (e.g., ground truth label of the target UE, positioning assistance information) for monitoring metric calculation might be needed. The UE can request assistance information for monitoring metric calculation via RequestAssistanceData message.

Proposal 1: (issue LPP-16) For Option A (the target UE side performs monitoring metric calculation), the UE can request assistance information (e.g., ground truth label of the target UE, positioning assistance information) for monitoring metric calculation via RequestAssistanceData message.

In the previous meeting, it was agreed that LMF is responsible for functionality management. For functionality management by LMF, the monitoring metric can be calculated by both UE and LMF. If the monitoring metric is calculated by UE, the UE needs to provide the monitoring metric to LMF. Then, the LMF performs functionality management based on the received monitoring metric.

Proposal 2: (issue LPP-16) For functionality management by LMF, if UE side performs monitoring metric calculation, the monitoring metric should be sent by UE to LMF via ProvideLocationInformation message.

For monitoring metric reporting, LMF can configure the condition for triggering monitoring outcome report. If the configured condition is satisfied, the UE will report the monitoring outcome. A periodic report can also be configured for monitoring metric reporting.

Proposal 3: (issue LPP-16) The UE reports the monitoring metric periodically or triggered by the configured condition.

If option B is adopted by RAN1, one issue regarding how LMF calculates the monitoring metric is identified. LMF needs to calculate the monitoring metric at least based on the inferred results. Therefore, the LMF can request the inference results for monitoring metric calculation via RequestLocationInformation message.

Proposal 4: (issue LPP-16) If LMF performs monitoring metric calculation, the LMF requests inference results for monitoring metric calculation via RequestLocationInformation message.

2.2 Other issue

In RAN2#128, it was agreed, if the AI/ML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AI/ML based positioning, and reply with LPP ProvideLocationInformation message with error cause. If this happens, the UE needs to perform fallback operation. The issue how to perform fallback was identified, and the corresponding resolution was agreed by the previous meeting: switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in an LPP Request Location Information message.

In step 5, the LMF can provide multiple positioning methods (e.g., AI/ML positioning, non-AI/ML positioning). Since the UE does not know which positioning method to use, the priority levels of different positioning methods can be provided by LMF to assist in selection. It is generally recommended that AI/ML positioning methods have higher priority than non-AI/ML methods.

Proposal 5: If multiple positioning methods are included in an LPP RequestLocationInformation message, AI/ML positioning methods are recommended to have higher priority than non-AI/ML methods.

In the previous meeting, it was agreed that an AI/ML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. Generally, the RequestLocationInformation message will provide inferred configuration information for a specific AI/ML positioning functionality, which will implicitly activate this functionality.

For the functionality deactivation, if the LMF detects a decline in the performance of this functionality (e.g., reduced positioning accuracy), the LMF can directly deactivate the AI/ML positioning functionality with an explicit indication.

Proposal 6: LMF can deactivate an AI/ML positioning functionality explicitly if the performance reduction is detected.

In LCM for UE-sided model for Beam Management, a relevant conclusion for functionality deactivation was reached:

RAN2#128 Agreement When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Such mechanism can be reused for AI/ML positioning case 1. If an activated functionality becomes non-applicable, which means that it cannot be used at this moment, UE can report the non-applicability of this functionality and LMF is expected to deactivate it. The LMF can explicitly deactivate this functionality. Alternatively, if the inference configuration corresponding to other applicable functionality is included in RequestLocationInformation message, the non-applicable functionality is deactivated implicitly.

Proposal 7: After receiving non-applicability report of one certain functionality, LMF can deactivate it explicitly or deactivate it implicitly by providing inference configuration corresponding to other applicable functionality in RequestLocationInformation message.

During the inference period, if the functionality is not usable for location inference, the UE may report an error indication in ProvideLocationInformation. For triggered location reporting, if multiple positioning methods are configured in an LPP RequestLocationInformation message, the UE will perform positioning with a fallback positioning method. After completing positioning, the procedure will terminate, and there is no deactivation scenario during this process. In contrast, for periodical location reporting, a deactivation operation is needed, and the same method used for non-applicability reporting can be applied.

Proposal 8: For periodical location reporting, if the error indication is included in ProvideLocationInformation, LMF can deactivate it explicitly or deactivate it implicitly by providing inference configuration corresponding to other functionality in RequestLocationInformation message.

3. Conclusions

In this contribution, we discussed the issues related to LCM for Positioning Use Case. Based on the discussion, the following proposals are concluded:

Proposal 1: (issue LPP-16) For Option A (the target UE side performs monitoring metric calculation), the UE can request assistance information (e.g., ground truth label of the target UE, positioning assistance information) for monitoring metric calculation via RequestAssistanceData message.

Proposal 2: (issue LPP-16) For functionality management by LMF, if UE side performs monitoring metric calculation, the monitoring metric should be sent by UE to LMF via ProvideLocationInformation message.

Proposal 3: (issue LPP-16) The UE reports the monitoring metric periodically or triggered by the configured condition.

Proposal 4: (issue LPP-16) If LMF performs monitoring metric calculation, the LMF requests inference results for monitoring metric calculation via RequestLocationInformation message.

Proposal 5: If multiple positioning methods are included in an LPP RequestLocationInformation message, AI/ML positioning methods are recommended to have higher priority than non-AI/ML methods.

Proposal 6: LMF can deactivate an AI/ML positioning functionality explicitly if the performance reduction is detected.

Proposal 7: After receiving non-applicability report of one certain functionality, LMF can deactivate it explicitly or deactivate it implicitly by providing inference configuration corresponding to other applicable functionality in RequestLocationInformation message.

Proposal 8: For periodical location reporting, if the error indication is included in ProvideLocationInformation, LMF can deactivate it explicitly or deactivate it implicitly by providing inference configuration corresponding to other functionality in RequestLocationInformation message.

4. References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface