3GPP TSG-RAN WG2 Meeting #130	R2-2504470

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.3

Source:	HONOR

Title: 	Discussion on on-demand SIB1

Document for:	Discussion and decision

- Introduction

In RAN2#129bis, the following aspects were agreed to be specified in the on-demand SIB1 work item [1]:

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In this contribution, we will analyse the remaining issues of OD-SIB1 procedure, including contents of WUS configuration, and triggering conditions of UL-WUS.

- Discussion

2.1 Triggering conditions of UL-WUS

In the last meeting, RAN1 agreed that WUS configuration should be mandatory to carry the K_SSB, searchSpaceZero, and controlResourceSetZero of NES cells [2].

Agreement

From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and

applicability to TDD and/or FDD, adopt the followings:

- PhysCellId and ARFCN-ValueNR are mandatory

- frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification)

- K_SSB is mandatory

- searchSpaceZero and controlResourceSetZero are mandatory

- ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory

Based on the above agreement, we believe that the K_SSB indicated in the MIB of NES cells may no longer retain its original physical meaning (i.e. the subcarrier offset from subcarrier 0 in common resource block to the lowest-numbered subcarrier of the SS/PBCH block) [3].

Observation 1: The K_SSB indicated in the MIB of NES cells may no longer retain its original physical meaning, i.e. no longer indicate the subcarrier offset from subcarrier 0 in the common resource block to the lowest-numbered subcarrier of the SS/PBCH block.

In addition, the PDCCH-ConfigSIB1 field also has different indications depending on the value of K_SSB [4]. Therefore, both K_SSB and PDCCH-ConfigSIB1 fields can be reused in the NES cell to indicate new meanings.

According to agreements of RAN2#127-bis meeting, RAN2 has agreed that a NES UE needs to check if the SIB1 is currently being broadcasted or provided on demand for that NES cell before requesting the SIB1 of that cell [5].

Agreement

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided

on demand for that cell before requesting SIB1 of that cell.

However, RAN2 has not yet clearly concluded how the UE confirms the SIB1 broadcast status of the NES cell.

Proposal 1: K_SSB/PDCCH-ConfigSIB1 could be used to indicate the SIB1 broadcast status.

When the NES UE confirms that the SIB1 of the NES cell is being broadcast, it can directly acquire the SIB1 without initiating an OD-SIB1 request. However, a situation may arise where a new UE reselects to the NES cell at the end of the SIB1 time window and there will be the last or no SIB1 transmission within the time window. In this case, the new UE is unable to acquire SIB1 within this time window. Therefore, the network should provide information such as the remaining time/number of SIB1 transmissions to instruct the UE to avoid unnecessary monitoring or triggering UL-WUS for further SIB1 broadcasting.

In summary, it is suggested to redefine the PDCCH-ConfigSIB1 in the MIB of NES cells to provide more SIB1 transmission information, e.g. the remaining number of SIB1 transmissions in a SIB1 time window or the SIB1 time window end time.

Proposal 2: It is proposed to indicate the remaining number of SIB1 transmissions in the SIB1 time window or the end of the SIB1 time window if SIB1 is broadcasting, e.g. redefine the PDCCH-ConfigSIB1.

NES UEs decide whether to acquire SIB1 directly or trigger UL-WUS for further SIB1 broadcasting based on the above SIB1 transmission information. For example, when the remaining time of this SIB1 time window is short, the UE does not need to monitor and can trigger the next SIB1 broadcasting. Conversely, when there is sufficient remaining time in the SIB1 time window, the UE should begin monitoring and attempt to acquire SIB1.

In the last meeting, RAN2 reached a new agreement on cell barring mechanism: If a NES UE fails to receive SIB1 upon the SIB1 time window expires, the UE may consider the cell as being barred [1].

Agreement

If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

However, it should be noted that the agreement is intended to explain that when the NES UE initiates UL-WUS and triggers SIB1 broadcasting, if it does not receive SIB1 in the SIB1 time window, the cell should be barred. It does not consider the case where the NES UE does not initiate UL-WUS but directly tries to acquire the broadcast SIB1 triggered by other NES UEs. Therefore, it is proposed that the NES UE may not bar the cell and can trigger UL-WUS for further SIB1 broadcasting if it cannot acquire SIB1 upon the expiry of the SIB1 time window triggered by the other UEs.

Proposal 3: NES UE may trigger UL-WUS for SIB1 if it cannot acquire SIB1 upon the expiry of the SIB1 monitoring window triggered by the other NES UEs.

2.2 SI change procedure in NES Cell

In legacy spec, the network broadcasts the updated SIB1 in the next modification period (MP) after notifying UEs about changes in system information (SI). If no UEs are present in the NES cell during this broadcast, it results in wasted resources and energy. Therefore, to better align with the goal of NES, the network will not actively broadcast the updated SIB1 after sending the SI change indication. Instead, the UE should trigger WUS transmission to request SIB1 after receiving SI change indication.

Proposal 4: After the NW sends the SI change indication in the NES cell, it will not actively broadcast the updated SIB1 unless a NES UE sends an OD-SIB1 request.

2.3 OD-OSI procedure in NES Cell

The UE should acquire the SIB1 for si-RequestResources/si-BroadcastStatus before sending the request for on-demand system information. If SIB1 is provided on-demand, the UE needs to trigger a Random Access (RA) process for SIB1, followed by another RA for the invalid OSI. This may result in frequent SIB1 requests, leading to significant network energy consumption. It is beneficial if the UE could trigger the OSI request without re-acquiring SIB1 to check the broadcasting status.

Proposal 5: In NES cell, the UE could trigger the request for OSI without re-acquiring SIB1 to check OSI broadcasting status.

To avoid incorrect MSG1-based request, the si-RequestResources for specific SIB/SI message should be fixed instead of explicitly mapped based on broadcasting status. Alternatively, the network should not enable the MSG1-based OSI, which depends on network implementation.

Proposal 6: For OD-OSI in NES cell, the mapping between si-RequestResources and SI message should not be changed with broadcasting status of every OSI.

2.4 Contents of WUS configuration

In legacy spec, SIB1 contains cell access related information and cellbarred indication for UEs with different features. They will instruct UEs to determine whether the current cell allows access and whether they need to perform a cell reselection.

In NES cells, NES UEs that are barred by the networks must initiate the OD-SIB1 procedure to obtain SIB1 before confirming their access prohibition. For these UEs, the OD-SIB1 procedure is unnecessary. To prevent these UEs from attempting the OD-SIB1 procedure, the WUS configuration should provide the necessary cell access information. This allows NES UEs to directly confirm access permissions, reducing unnecessary energy consumption and resource waste while improving the efficiency of cell reselection. Considering the limited capacity of WUS configuration, it is recommended that WUS configuration does not need to include all cell access related information in SIB1 as legacy but should at least include the PLMN ID and cellbarred indication for UEs with different features.

Proposal 7: WUS configuration provides some necessary cell access information optionally, including PLMN ID, and cellbarred indication for UEs with different features as a baseline.

- Conclusions

In this contribution, we discussed several detailed on-demand SIB1 issues, and made the following observations and proposals:

Observation 1: The K_SSB indicated in the MIB of NES cells may no longer retain its original physical meaning, i.e. no longer indicate the subcarrier offset from subcarrier 0 in the common resource block to the lowest-numbered subcarrier of the SS/PBCH block.

Proposal 1: K_SSB/PDCCH-ConfigSIB1 could be used to indicate the SIB1 broadcast status.

Proposal 2: It is proposed to indicate the remaining number of SIB1 transmissions in the SIB1 time window or the end of the SIB1 time window if SIB1 is broadcasting, e.g. redefine the PDCCH-ConfigSIB1.

Proposal 3: NES UE may trigger UL-WUS for SIB1 if it cannot acquire SIB1 upon the expiry of the SIB1 monitoring window triggered by the other NES UEs.

Proposal 4: After the NW sends the SI change indication in the NES cell, it will not actively broadcast the updated SIB1 unless a NES UE sends an OD-SIB1 request.

Proposal 5: In NES cell, the UE could trigger the request for OSI without re-acquiring SIB1 to check OSI broadcasting status.

Proposal 6: For OD-OSI in NES cell, the mapping between si-RequestResources and SI message should not be changed with broadcasting status of every OSI.

Proposal 7: WUS configuration provides some necessary cell access information optionally, including PLMN ID, and cellbarred indication for UEs with different features as a baseline.

- References

- Draft_RAN2_129bis_Meeting_Report_v2.docx

- Draft_Minutes_report_RAN1#120b_v010.docx

- TS 38.211 V18.5.0, Physical channels and modulation, 2024-12

- TS 38.213 V18.4.0, Physical layer procedures for control, 2024-09

- R2-2409502 RAN2#127bis Meeting Report MCC report