3GPP TSG-RAN WG2 Meeting #130	R2-2504475

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.5

Source:	HONOR

Title:	Discussion on RLC enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#129bis, the following agreements were agreed for enhancing RLC retransmissions [1]:

Agreements on RLC enahncements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review). - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission. - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

In this contribution, we discuss the potential solutions related to RLC enhancements.

2. Discussion

2.1 (Open issue RLC-2) Update RX_Highest_Status

Open issue RLC-2 (not essential, but important): whether further changes are needed for SR triggered by t-RxDiscard expires.

On the one hand, the RX_Highest_Status updates when the SDU with SN equals to RX_Highest_Status is completely received. In the previous discussion, the abandoned RLC SDUs determined by t-RxDiscard are positively acknowledged, so those SDUs can be regarded as ACK (completely received) RLC SDUs in the RX. Therefore, the RX_Highest_Status needs to be updated when some RLC SDUs are discarded.

On the other hand, the RX_Highest_Status holds the highest possible value of the SN which can be indicated by "ACK_SN". The RX_Highest_Status needs to be updated, because the value of ACK changes when some RLC SDUs are positively acknowledged by the expiration of t-RxDiscard.

Proposal 1: (RLC-2) The RX_Highest_Status should be updated because the abandoned RLC SDUs determined by t-RxDiscard are positively acknowledged.

2.2 (Open issue RLC-9) Status report for polling enhancement

Open issue RLC-9: FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission due to polling enhancement.

The objective of polling enhancement is to receive status report timely. However, the transmission of status report is constrained by the t-StatusProhibit when a poll is received. To maximize the benefits of polling enhancement, enhanced poll should be distinguished with usual poll. Furthermore, the RX should be able to trigger an SR without the restriction by t-StatusProhibit when receiving an enhanced poll.

Proposal 2: (RLC-9) Enhanced poll distinguished with usual poll can be introduced.

Proposal 3: (RLC-9) An SR should be triggered without the restriction by t-StatusProhibit when receiving an enhanced poll.

3. Conclusions

(Open issue RLC-2) Update RX_Highest_Status

Proposal 1: (RLC-2) The RX_Highest_Status should be updated because the abandoned RLC SDUs determined by t-RxDiscard are positively acknowledged.

(Open issue RLC-9) Status report for polling enhancement

Proposal 2: (RLC-9) Enhanced poll distinguished with usual poll can be introduced.

Proposal 3: (RLC-9) An SR should be triggered without the restriction by t-StatusProhibit when receiving an enhanced poll.

4. References

[1]	RAN2 #129bis Chairman Notes