3GPP RAN WG2 Meeting #130	R2-2504479

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.9.3

Source:	HONOR

Title:	Discussion on UL capacity enhancement

Document for:	Discussion and Decision

1. Introduction

In Rel-19 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID, the following objectives were achieved:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#129bis meeting, the following agreements were made:

- For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure) - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring. - The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1 - The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. - Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached. - Backoff information could be included in CB-Msg4. - L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported. - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

In this contribution, we discuss the remaining issues on CB-Msg3 mechanism.

2. Discussion

  - 2.1 (issue MAC-12) MSG4 multiplexing

From the previous meeting discussion, we find that there are mainly two options for MSG4 multiplexing:

Option 1: MSG4 multiplexing for the UEs transmit CB-MSG3 with same frequency domain

Option 2: MSG4 multiplexing for the UEs transmit CB-MSG3 with same time domain

For option 1and option 2, the NW can receive multiple CB-MSG3 message transmitted on the same frequency domain or on same time domain, and the UE is also able to receive the corresponding multiplexed MSG4. From this perspective, both option 1 and option 2 can be supported. For the MSG4 multiplexing with the same frequency domain resource, a multiplexed MSG4 responds to the multiple CB-MSG3 transmission on different time domain occasions. This also indicates that the multiplexed MSG4 needs to be postponed for sending until receiving multiple CB-MSG3 transmission on different time domain occasions. The network can ensure that the multiplexed MSG4 can be sent before the end of response window corresponding to the first CB-MSG3. Therefore, it is feasible to support MSG4 multiplexing with the same frequency domain resource.

For MSG4 multiplexing with the same time domain resource, a multiplexed MSG4 responds to the multiple CB-MSG3 transmission on different frequency domain occasions. Since the MSG4 multiplexing transmits on one certain frequency domain resource, which might be different from the frequency domain resource used for CB-MSG3 transmission, the UE needs to know on which frequency domain resource to monitor for the multiplexed MSG4. Therefore, the information of the frequency domain resource that used for MSG4 multiplexing transmission should be indicated.

Proposal 1: (issue MAC-12) For CB-MSG3 transmission on the same time domain resource or on the same frequence domain resource, MSG4 can be multiplexed into one MAC PDU.

Proposal 2: (issue MAC-12) For MSG4 multiplexing with the same time domain resource, it needs to indicate the information of the frequency domain resource that used for MSG4 multiplexing transmission.

For CB-MSG3 transmission on the same time domain resource, the UE needs to know which MSG4 is relevant to itself. For example, in the subheader of success RAR, one filed can be introduced to indicate this success RAR is related to a certain frequence domain resource occasion, which is used for CB-MSG3 transmission.

Proposal 3: (issue MAC-12) For MSG4 multiplexing with the same time domain resource, the frequence domain resource occasion corresponding to CB-MSG3 transmission should be indicated for each MSG4.

Similar to MSG4 multiplexing with the same time domain resource, for CB-MSG3 transmission on the same frequency domain resource, in the subheader of success RAR, one filed can be introduced to indicate this success RAR is related to a certain time domain resource occasion, which is used for CB-MSG3 transmission.

Proposal 4: (issue MAC-12) For MSG4 multiplexing with the same frequency domain resource, the time domain resource occasion corresponding to CB-MSG3 transmission should be indicated for each MSG4.

If the successRAR is followed by one MAC SDU/padding, it means that the MAC SDU/padding and the success RAR are applied to the same UE. One filed in the subheader of successRAR can be used to indicate the length that the UE can skip decoding the followed MAC SDU/padding.

Proposal 5: (issue MAC-12) If the successRAR is followed by one MAC SDU/padding, one filed in the subheader for successRAR can be used to indicate the length that the UE can skip decoding the followed MAC SDU/padding.

  - 2.2 (issue MAC-13) Msg4 structure

For CB-MSG3 transmission, since the cell specific PUSCH resources are configured for Msg3 transmission, after receiving Msg3, a contention resolution mechanism should be applied. The UE contention resolution identity included in the successRAR/contention resolution MAC CE is used to determine whether the contention is successful. If the UE contention resolution identity matches the first 48bits used in CCCH SDU, that would mean success from the contention.

Proposal 6: (issue MAC-13) Reuse the legacy contention resolution mechanism, UE contention resolution identity(48bits) should be included.

If the contention resolution indication is transmitted separately and followed by subsequent RRC message or DL data, the UE who won the competition needs to know whether to detect and receive the subsequent RRC message or DL data.

Whether to detect and receive the subsequent RRC message or DL data can be indicated by successRAR/contention resolution MAC CE. For instance, if the C-RNTI is included, there is a subsequent RRC message or DL data. That is to say, if C-RNTI is present, the UE needs to monitor PDCCH for the subsequent RRC message or DL data. If C-RNTI is not present, the UE will terminate this procedure.

Proposal 7: (issue MAC-13) The C-RNTI can be included in the success RAR to indicate the UE needs to receive the subsequent RRC messages or DL data.

The issue whether a RRC message is allowed to be included in MSG4 should be taken into account. If a RRC message is included in MSG4, this will lead to an increase in the size of MSG4. For MSG4 multiplexing, since the size of MSG4 messages will increase by multiple RRC message, it will affect the performance of detection and decoding. But for non- MSG4 multiplexing, the problem will become less serious. Therefore, a RRC message is allowed to be included in MSG4, and it is up to NW implementation to decide whether a RRC message is included in MSG4.

Proposal 8: (issue MAC-13) A RRC message is allowed to be included in MSG4, and it is up to NW implementation to decide whether a RRC message is included in MSG4.

  - 2.3(other issue) CB-Msg3 Response window

In the previous meeting, it was agreed that, for CB-msg3-EDT we adopt a single Msg4 monitoring window and single RNTI (the RNTI is derived on the transmit resource for the transmission window). In general, for a single Msg4 monitoring window, there are three options for starting Msg4 monitoring window:

Option 1:one Msg4 monitoring window will be started/restarted after sending every replica.

One Msg4 monitoring window will be started/restarted after sending the next replica, and it can be regarded as a form of single Msg4 monitoring window. In detail, if the previous window has already ended before sending the next replica, the Msg4 monitoring window will be started directly after sending the next replica. Else, the Msg4 monitoring window will be restarted.

Option 2: one Msg4 monitoring window will be started after the first replica.

In RAN2#129 bis, an FFS was left:

FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring.

In this option, if the UE selects the transmitting replicas with a large interval, it may cause the response messages of subsequent replicas to fall outside the window. In order to solve this problem, a relatively long window length needs to be configured by NW. However, it may bring about unnecessary power consumption due to the monitoring behavior.

In addition, in Msg4 monitoring window, whether the UE prioritize the replica transmission or monitoring needs to be standardized, which will have impacts to UE behavior.

Option 3: the Msg4 monitoring window can be started after the end of DSA resource window.

If the UE selects the earlier occasion of DSA resource within this window, the monitoring window has to wait for the end of DSA time window, which might have a slight impact on UL capacity.

Proposal 9: The Msg4 monitoring window should be started/restarted after sending every replica.

3. Conclusions

In this contribution, we discussed the issues related to CB-Msg3 mechanism. Based on the discussion, the following proposals are concluded:

Proposal 1: (issue MAC-12) For CB-MSG3 transmission on the same time domain resource or on the same frequence domain resource, MSG4 can be multiplexed into one MAC PDU.

Proposal 2: (issue MAC-12) For MSG4 multiplexing with the same time domain resource, it needs to indicate the information of the frequency domain resource that used for MSG4 multiplexing transmission.

Proposal 3: (issue MAC-12) For MSG4 multiplexing with the same time domain resource, the frequence domain resource occasion corresponding to CB-MSG3 transmission should be indicated for each MSG4.

Proposal 4: (issue MAC-12) For MSG4 multiplexing with the same frequency domain resource, the time domain resource occasion corresponding to CB-MSG3 transmission should be indicated for each MSG4.

Proposal 5: (issue MAC-12) If the successRAR is followed by one MAC SDU/padding, one filed in the subheader for successRAR can be used to indicate the length that the UE can skip decoding the followed MAC SDU/padding.

Proposal 6: (issue MAC-13) Reuse the legacy contention resolution mechanism, UE contention resolution identity(48bits) should be included.

Proposal 7: (issue MAC-13) The C-RNTI can be included in the success RAR to indicate the UE needs to receive the subsequent RRC messages or DL data.

Proposal 8: (issue MAC-13) A RRC message is allowed to be included in MSG4, and it is up to NW implementation to decide whether a RRC message is included in MSG4.

Proposal 9: The Msg4 monitoring window should be started/restarted after sending every replica.

4. References

[1]