3GPP TSG RAN WG2 #130				R2-2504489

St. Julians, Malta, May. 19th – 23rd, 2025	Revision of R2-2502819

Agenda Item:	8.2.2

Source:		ASUSTeK

Title:				Discussion on Ambient IoT paging message design

Document for: Discussion and Decision

- Introduction

In the WID agreed in RAN#106 meeting [1], the following objectives on Ambient IoT RAN2 scope were set:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

Additionally, in RAN2#129bis meeting [2], the following agreements on paging message were made:

Agreements FFS which solution if any for device behaviour if it gets a new service request while one procedure is still ongoing or leave it to implementation. RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

In this contribution, we further discuss the detailed design of A-IoT paging message.

- Discussion

2.1  Paging ID

During the offline discussion in RAN2#129bis [3], the intention of “forward compatibility” was clarified.

- Forward compatibility … Opt 1: Rel20 Reader can transit a common paging message. Rel19 AIOT device can partially understand the (Rel19) part of the paging message and act accordingly while gracefully ignoring the parts it does not understand. Rel20 message formats are generally ‘extension’ of Rel-19 message formats with the same Message Type. (This does not exclude the possibility of new Messages only used by Rel20 devices to accommodate new Rel20 use cases.) Opt 2: Rel20 Reader transmits separate paging messages for Rel19 and Rel20 devices. Rel19 AIOT devices only understand Rel19 paging message, and responds to it accordingly if needed. Rel20 AIOT devices only understand Rel20 paging message, and respond to it accordingly if needed. Rel20 message formats will use new Message Type, and can be completely different format wrt Rel-19 message formats for even the same message. … Rapp Summary: Companies seem to be ok with following recommendation: At least for the purpose of current discussions, we consider opt1 as the intended ‘forward compatibility’. I.e., RAN2 aims to design Rel-19 AIoT messages extensible to accommodate devices and features of future release.

It has been confirmed that RAN2 aims to design Rel-19 A-IoT R2D messages extensible to accommodate devices and features of future release. To achieve the purpose for forward compatibility, one way to design a paging message format such that multiple identifiers can be contained is to include a field in a paging message to indicate a number of IDs. Each ID may have its corresponding transaction ID or all IDs share the same transaction ID. And the field is always set to 1 in Release 19.

Proposal 1:  To support multiple IDs in a paging message for forward compatibility, a paging message includes a field to indicate the number of IDs.

2.2  Device type

On the other hand, the reader may page devices separately based on their device type due to different device capabilities. Since A-IoT devices have very low capability, it is important for the network to schedule these devices to perform data transmission properly. For example, the reader could indicate access occasions in the paging message based on device type, for device 1 might not be able to apply resources with larger frequency shift.

Although only device 1 is supported in this release, device 2a and device 2b may be included in future releases. It is proposed to consider including device type information in a paging message, since the paging format should allow the devices from different releases to co-exist in the fields.

Proposal 2:  To support different device types for forward compatibility, a paging message includes a field to indicate device type information.

2.3  Information for Msg1 resource

One of the open issues of paging message content is how to indicate the number of access occasions. It has been agreed in RAN2#129 meeting that the A-IoT paging message could include a number of Msg1 resources. Additionally, it has been agreed in RAN2#129bis meeting that the R2D trigger message indicates the start of a set of Msg1 resources that were configured in the paging message. According to the Figure 1 cited from TR 38.769 [4], a set of access occasions is followed or indicated by the R2D trigger message. The set of access occasions could be referred as an access round, which is the grey block in Figure 1. That is to say, there would be several access rounds indicated by a paging message.

Figure 1 General framework of slotted-ALOHA for A-IoT random access procedure in [4]

For Msg1 access occasions, the paging message should include the number of access rounds. And as explained in [5], to attain flexible scheduling for the reader, the number of time domain resources could be included in the R2D trigger message.

Proposal 3:  For Msg1 resource selection, a paging message includes a field to indicate the number of access rounds.

- Conclusion

Proposal 1:  To support multiple IDs in a paging message for forward compatibility, a paging message includes a field to indicate the number of IDs.

Proposal 2:  To support different device types for forward compatibility, a paging message includes a field to indicate device type information.

Proposal 3:  For Msg1 resource selection, a paging message includes a field to indicate the number of access rounds.

- Reference

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”

[2] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

[3] R2-2503155, “Offline discussion report: [AT129bis][010][AIoT] Paging”

[4] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”

[5] R2-2504490, “Discussion on Ambient IoT random access”