3GPP TSG-RAN WG2 Meeting #130	R2-2504491

St. Julian’s, Malta, 19th-23rd May, 2025 	Revision of R2-2502821

Agenda Item:	8.2.4

Source:			ASUSTeK

Title:	Discussion on the assistance information from device

Document for: 	Discussion and Decision

- Introduction

In Rel-19 study item phase, assistance information from device side for A-IoT transmission has been discussed in RAN2 meetings and the following is captured in TR 38.769 [1]:

The segmentation study focuses on D2R direction (it can be further considered on R2D direction). For the candidate solution for segmentation: - The sequence number, the segment number and the number of segments are not supported; - An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details; - It is beneficial for the reader to be able to trigger a re-transmission of a segment; - It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s). The use of the following assistance information is studied (the need of each is still to be decided): - The energy status report from the A-IoT device to the reader, which indicates that the A-IoT device, if the A-IoT device can determine this, does not have sufficient energy to perform the remaining/follow-up procedure. - The A-IoT device may report a 1-bit energy status indication to the reader in a D2R message. It needs to be further discussed if and how the reader control this indication of such report. - The reader may take this indication into account for the remaining/follow-up procedure (up to the reader implementation), e.g., the reader may choose not to transmit the subsequent messages for some time, or the reader may choose to not do anything in particular, etc. - It is beneficial for the reader to know an estimate of expected D2R message size. This D2R message size information would be beneficial but it is not essential. Two options are studied: - Option 1: The estimate of expected following D2R message size from the CN to the reader; - Advantages/disadvantages of option 1: If this information is available from CN, it saves the overhead/latency caused by Option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed. - Option 2: The simple message size indication from the device to the reader. - Advantages/disadvantages of option 2: It is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

In this contribution, we would like to further discuss the assistance information provided via the D2R message.

- Discussion

The segmentation indication and data size indication has been discussed in several previous RAN2 meetings. And it was agreed in the last meeting that “1-bit indication is sufficient to indicate whether more D2R data will be sent” for segmentation. The 1-bit segmentation indication could be either optionally or always present in D2R messages. To have a general format for D2R MAC PDU, we propose to confirm that the segmentation indication is always carried in each D2R MAC PDU (e.g., D2R Upper Layer Data Transfer message), as currently captured in the running CR of TS 38.391 [2]. If the size of available D2R data is larger than the resources for a D2R transmission, the device segments the D2R data and sets the segmentation indication to 1. If the size of available D2R data is not larger than the resources for the D2R transmission or the D2R transmission is the last segmentation, the device sets the segmentation indication to 0. The 1-bit segmentation indication from the device along with estimated D2R message size form the CN could ensure efficient resources allocation on the reader.

Proposal 1: Confirm that the 1-bit segmentation indication is always included in the D2R message for upper layer data transmission, with value 1 or 0 to indicate whether there is more data or not.

For energy status indication, in RAN2 #127bis meeting, it was agreed that the device may include energy status indication in D2R messages (e.g., MSG1, MSG3 and Command Response message). As captured in TR 38.769 [1], the device could provide a 1-bit energy status indication to the reader in a D2R message, which indicates that the A-IoT device does not have sufficient energy to perform the remaining/follow-up procedure. With limited energy capacity, the A-IoT device may not be always able to perform transmissions. If an A-IoT device does not have enough energy to receive/transmit the following transmission(s), the ongoing/follow-up procedure would fail.

Similar to segmentation indication, the 1-bit energy status indication could be either optionally or always present in D2R messages. To have a general format for D2R MAC PDU, we propose that the energy status indication could be always carried in each D2R MAC PDU (e.g., D2R Upper Layer Data Transfer message). If the remaining energy of the device is sufficient for following transmission(s), the energy status indication indicates 0. If the remaining energy of the device is insufficient for following transmission(s), the energy status indication indicates 1. In addition, the device could stop the ongoing procedure or not initiate the follow-up transmission after providing the energy information indicating the remaining energy of the device being insufficient.

Proposal 2: The energy status indication could be always included in the D2R message for upper layer data transmission, with value 1 or 0 to indicate whether the remaining energy of the device is insufficient.

Proposal 3: The device stops the ongoing procedure after providing the energy information indicating the remaining energy of the device being insufficient.

On the other hand, forward compatibility for R2D message design was discussed in the last meeting. It has been agreed that RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. Similarly, we propose that the Rel-19 AIoT D2R messages could be extensible for future release. To achieve this, some reserved bits could be included in the D2R MAC PDU, along with the segmentation indication and energy status indication. Since the D2R MAC PDU and upper layer data SDU would be byte-aligned, the assistance information should also be byte-aligned.

Proposal 4: For forward compatibility, reserved bits could be included along with the indications in the D2R message for upper layer data transmission.

- Conclusion

We have the following proposals for assistance information in D2R message for upper layer data transmission:

Proposal 1: Confirm that the 1-bit segmentation indication is always included in the D2R message for upper layer data transmission, with value 1 or 0 to indicate whether there is more data or not.

Proposal 2: The energy status indication could be always included in the D2R message for upper layer data transmission, with value 1 or 0 to indicate whether the remaining energy of the device is insufficient.

Proposal 3: The device stops the ongoing procedure after providing the energy information indicating the remaining energy of the device being insufficient.

Proposal 4: For forward compatibility, reserved bits could be included along with the indications in the D2R message for upper layer data transmission.

- Reference

- 3GPP TR 38.769 V19.0.0, “Study on solutions for Ambient IoT in NR.”

- R2-2503997, “A-IoT MAC running CR.”