3GPP TSG RAN WG2 #130				R2-2504499

Malta, MT, May 19 – 23, 2025

Agenda Item:	8.12.3 Others (NR_MIMO_Ph5-Core)

Source:		ASUSTeK

Title:				Discussion on RRC impacts for UEI Beam Reporting Mode B

Document for: Discussion and Decision

- Introduction

In this document, we discuss some RRC impacts for UEI Beam Reporting Mode B.

- Discussion

Offset between the 1st Step and 2nd Step

According to the RAN1 agreements and parameter list shown below, RAN1 has included the parameter numOfSymbols-ModeB-r19 in the higher layer parameters list to configure the time offset, in number of symbols, for determining the first available transmission occasion of PUSCH in Mode-B following a PUCCH transmission.

Proposal 1:  Add numOfSymbols-ModeB-r19 in CSI-ReportUE-IBM-r19 to configure time offset, in number of symbols, for determining the first available transmission occasion of PUSCH on CG Type 1 in Mode-B, following the PUCCH transmission of the report notification.

R1#118 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. R1#120 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, the value of X symbols for determining available transmission occasion of the second UL channel is configured by RRC (FFS: subject to a corresponding UE capability). R1#120b Conclusion Regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, there is NO consensus on further supporting the case that first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms).

(from parameter list, RAN1#120b):

| NR_MIMO_Ph5 | UEIBM |  |  | CSI-ReportConfig |  | numOfSymbols-ModeB-r19 | New |  | This parameter is used to configure time offset in number of symbols for determining available transmission occasion of second PUSCH in Mode-B from the first PUCCH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

2nd Step – configuredPUSCHResourceOfModeB-r19

RAN1 provides three parameters to indicate the index of the Configured Grant configurations, which is currently used in indicating Type-1 CG resource for Mode-B PUSCH transmission:

configuredPUSCHResourceOfModeB-r19      SEQUENCE {

configuredGrantConfigIndex-r19                ConfiguredGrantConfigIndex-r16                            OPTIONAL,    -- Need R

ul-BWP-Id-r19                                BWP-Id,

servCellIndex-r19							 ServCellIndex

}                                                                                                          OPTIONAL,    -- Need R

CSI-ReportUE-IBM field descriptions

configuredPUSCHResourceOfModeB-r19 Indicates Type-1 CG PUSCH resource for the second step of PUSCH transmission in mode-B UE-initiated beam reporting.

However, from RAN2 point of view, a single parameter ConfiguredGrantConfigIndexMAC-r16 should be sufficient as it can indicate the index of the CG configurations within the MAC entity.

configuredGrantConfigIndex Indicates the index of the Configured Grant configurations within the BWP.

configuredGrantConfigIndexMAC Indicates the index of the Configured Grant configurations within the MAC entity.

Proposal 2:  Use ConfiguredGrantConfigIndexMAC to indicate Type-1 CG PUSCH resource for the second step of PUSCH transmission in mode-B UE-initiated beam reporting.

HARQ processes for 2nd Step in ConfiguredGrantConfig

RAN2 has confirmed that the CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH) , meaning that HARQ operation should not be used for transmitting the beam report. Therefore, configuring nrofHARQ-Processes for the CG is unnecessary.

Proposal 3:  The UE should ignore nrofHARQ-Processes if it is configured within ConfiguredGrantConfig for UE-initiated beam reporting Mode B.

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

...

nrofHARQ-Processes The number of HARQ processes configured. It applies for both Type 1 and Type 2. See TS 38.321 [3], clause 5.4.1. If the UE is configured with nrofHARQ-Processes-v1700, the UE shall ignore nrofHARQ-Processes (without suffix). The network sets the value of this field to 1 when cg-LTM-Configuration is configured.

- Conclusion

In this document, we have the following proposals for UEI Beam Reporting Mode B.

Proposal 1:  Add numOfSymbols-ModeB-r19 in CSI-ReportUE-IBM-r19 to configure time offset, in number of symbols, for determining the first available transmission occasion of PUSCH on CG Type 1 in Mode-B, following the PUCCH transmission of the report notification.

Proposal 2:  Use ConfiguredGrantConfigIndexMAC to indicate Type-1 CG PUSCH resource for the second step of PUSCH transmission in mode-B UE-initiated beam reporting.

Proposal 3:  The UE should ignore nrofHARQ-Processes if it is configured within ConfiguredGrantConfig for UE-initiated beam reporting Mode B.

Reference

Text proposal for proposal 2 (based on running RRC CR for MIMO ph5)

Text proposal for proposal 2:

CSI-ReportUE-IBM-r19             ::=     SEQUENCE {

...

configuredPUSCHResourceOfModeB-r19                     ConfiguredGrantConfigIndexMAC

OPTIONAL,    -- Need R

…

}