3GPP TSG-RAN WG2 Meeting #130	R2-2504503

# St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.2.4 (Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on A-IoT data transmission

Document for:	Discussion and Decision

1.	Introduction

This document discusses ambient IoT (A-IoT) data transmission. In this contribution, we discuss mainly segmentation, AS ID, and segmentation and message content for data transmission. The related agreements in R2-129bis meetings are as follows.

Agreements on AS ID 1 AS ID is applied for Inventory + command case; 2 AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed. 3 For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits. 4 Do not specify the reader behaviour on how exactly the ASID is generated. 5 The device releases the AS ID upon power off (no stage 3 specification impact); 6 The device only keeps one AS ID at a time. 7 For CFRA, command message is used for AS ID assignment 8 For CBRA, Msg 2 is used for AS ID assignment 9 The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) - FFS other cases for release ASID to avoid keeping it indefinitely. Agreements on segmentation - For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. - FFS whether the reader always includes the command for retransmission of segments. - 1-bit indication is sufficient to indicate whether more D2R data will be sent - For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption. Agreements on MAC PDU format - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message - RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1. - The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message - The MAC padding is supported at least for D2R from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS. - In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS. FFS whether we introduce D2R message type. Discuss after looking at the overall MAC header design and space before deciding whether we introduce message type or reserved bits

2.	Discussion

The A-IoT MAC supports segmentation in D2R direction. Retransmission of a segment is also supported. For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. However, whether to include the command for retransmission of the following segment and whether to include offset zero in the first segment has not been concluded yet.

If the command and offset are always included in the R2D message for scheduling segmented D2R messages, a common R2D message format can be used for the first segment and the following segments. However, the device receives unnecessary information, and it wastes radio resources in the A-IoT air interface. In addition, the device needs to decode unnecessary field and to handle the information from the field (at least, ignoring or discarding the information). It may increase the device complexity. On the other hand, there is another opinion that the use of the common R2D message format is beneficial for reducing device complexity because the device’s behavior for decoding the format is common for all segments. However, after decoding, the device needs to perform additional actions to handle the unnecessary information as mentioned before. Considering both the complexity decrease in decoding and complexity increase in handling the decoded information, the benefit of common R2D message format is not clear. On the other hand, if the command for retransmission is not included in the R2D messages for the following segments and offset zero is not included in the R2D message for the first segment, the benefit seems clear. Unnecessary information is not transmitted, and the A-IoT device does not need to decode unnecessary information.

Proposal 1. The reader does not include the command for retransmission of segments except the first segment/unsegmented D2R message.

Proposal 2. The reader does not include offset zero in the first segment/unsegmented D2R message.

AS ID is released in the device upon receiving Paging with new transaction id for that device according to the agreements in R-129bis meeting. From the device perspective, it seems to be working well. However, from the reader perspective, the reader needs to store the AS ID after sending a paging with new transaction ID if the paging is not for the device. The reader keeps the AS ID unless a new paging for that device is sent. It will increase the reader complexity for managing AS IDs. However, the AS ID is not used even after a paging with new transaction id which is not for that device because parallel service requests by the same reader are not supported according to the agreement in R2-129 meeting. In other words, if the reader sends a paging with a new transaction id for a new service request, the previous service request from the same reader is considered as completed or terminated. The reader will not send any R2D data transmissions for the devices which are triggered by the previous paging. Therefore, AS ID can be released upon receiving paging with new transaction ID even in case that the paging does not trigger the device.

Proposal 3. The device releases the AS ID upon receiving paging with new transaction id for other devices.

The MAC padding is supported for D2R transmission. A field is required to indicate how many SDU bytes are present. However, how to be provided between SDU length field or padding length field has not been concluded yet.

Generally, padding is located in the last part of the PDU, and padding follows data. It can be assumed that it is the same for A-IoT MAC PDU design. Therefore, to handle SDU and padding properly, it needs to know either end of SDU or start of padding. If SDU length field is provided, end of SDU can be obtained from start of SDU and the length field. On the other hand, if padding length field is provided instead of SDU length field, total MAC PDU size is additionally used for calculating either end position of SDU or start position of padding. Alternatively, the device needs to read the MAC PDU from the end of the MAC PDU to handle the padding bytes first. It may work. However, compared to SDU length field, padding length field seems a less familiar way and the process to obtain SDU by using padding length field does not look straightforward. Therefore, we prefer SDU length field.

In other aspects, if padding length field and padding follow SDU, it is imagined that the padding length field and padding can be contained in the MAC PDU only if necessary. If no padding is required, padding length filed is also omitted. However, it may not work. If the padding length field is located behind the SDU without SDU length field. It is very hard for the device to find the location of the padding length field because the device does not know SDU length. But, to find SDU length, the device needs to get the padding length. It seems somewhat similar with a circular problem. With this understanding, there is no clear reason to support the padding length field.

Proposal 4. SDU length field is used to indicate how many SDU bytes are present.

Regarding a ‘write’ command, an issue about how to handle ‘write’ operation response has been raised during post email discussion ([POST129bis][017][AIoT] 38.391 Running CR). The writing time in device may be a quite long duration and it may be different among A-IoT devices. Also, the reader may not know precisely the writing time of the target A-IoT device. Therefore, it is a possible scenario that a device does not have an available ‘write’ response for the D2R transmission resource scheduled by D2R scheduling information contained the R2D ‘write’ command message. Generally, it is possible for a device to have no available command response at a D2R transmission resource.

From the RAN2 perspective, if the A-IoT device has no data available for transmission, it may be a proper operation for the device to send padding only MAC PDU. Also, it seems proper for the reader to consider it as successful D2R transmission if padding only D2R MAC PDU is successfully received.

From the upper layer perspective, if the A-IoT network does not receive the expected response, it may handle the procedure, for example, by sending a query command associated to the previous write command to ask whether the writing is completed or not to the device. Then, the reader can send the command to the device with another D2R scheduling for the response.

Proposal 5. From RAN2 perspective, the device may send padding only MAC PDU if there is no available upper layer data at a D2R transmission resource. The reader considers it as the successful D2R transmission if successfully received. This assumes that upper layer message exchanges are managed by the upper layers.

3.	Conclusion

This document discusses segmentation, AS ID, and segmentation and message content for data transmission.

Proposal 1. The reader does not include the command for retransmission of segments except the first segment/unsegmented D2R message.

Proposal 2. The reader does not include offset zero in the first segment/unsegmented D2R message.

Proposal 3. The device releases the AS ID upon receiving paging with new transaction id for other devices.

Proposal 4. SDU length field is used to indicate how many SDU bytes are present.

Proposal 5. From RAN2 perspective, the device may send padding only MAC PDU if there is no available upper layer data at a D2R transmission resource. The reader considers it as the successful D2R transmission if successfully received. This assumes that upper layer message exchanges are managed by the upper layers.