3GPP TSG-RAN WG2 #130	R2-2504519

St.Julians, Malta, 19th – 23rd Apr. 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on RLC enhancements

Document for:        	Discussion and decision

Agenda Item:         	8.7.5		RLC enhancements

- Introduction

In RAN #107, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding RLC enhancements.

- Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

And the following agreements related to RLC enhancements were reached in RAN2#129bis meeting.

Agreements on RLC enhancements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

In this contribution, we provide the discussion on RLC enhancements.

- Discussion

Avoid unnecessary RLC retransmissions

RAN2 agreed that the expiration of t-RxDiscard triggers an SR, but there is FFS on whether this is just usual SR, or some changes are needed. According to the current specification, RX_Highest_Status holds the highest possible value of the SN which can be indicated by ACK_SN. It means that when the SR is triggered by the expiration of t-RxDiscard, the RLC entity would set the ACK_SN to RX_Highest_Status in the STATUS PDU. However, after the expiry of t-RxDiscard, RX_Next may be updated to a value larger than RX_Highest_Status. This is a possible scenario if the RLC entity receives an AMD PDU with SN equal to or larger than RX_Highest_Status after the t-Reassembly has expired but before the t-RxDiscard has expired. In such a scenario, it would be beneficial to set ACK_SN to the same value as updated RX_Next rather than RX_Highest_Status in order to provide positive acknowledgements of more RLC SDUs. Therefore, when the t-RxDiscard expires, the SR should be triggered after RX_Highest_Status is updated to the latest value (e.g., the same value as the updated RX_Next).

Observation 1:	RX_Next may be updated to a value larger than RX_Highest_Status after the expiry of t-RxDiscard. In such a scenario, it would be beneficial to set ACK_SN to the same value as updated RX_Next in the STATUS PDU in order to provide positive acknowledgements of more RLC SDUs.

Proposal 1:	(RLC-2) When the t-RxDiscard expires, the SR should be triggered after RX_Highest_Status is updated to the latest value (e.g., the same value as the updated RX_Next).

We also need to consider the case where an RLC SDU that triggered t-Reassembly is discarded (i.e., RX_Next_Status_Trigger falls outside of receiving window) when t-RxDiscard expires. If this t-Reassembly expires, an unnecessary SR for the discarded SDU would be triggered. Therefore, it would be preferable to stop t-Reassembly which is still running for the discarded RLC SDU when t-RxDiscard expires. Furthermore, according to the current CR [2], if there is still an SN gap even after t-RxDiscard has expired and RX_Next has been updated, t-RxDiscard will be started again. In this case, it may be necessary to start t-Reassembly as well as t-RxDiscard at the same time. This is because if t-Reassembly is not started, the RLC SDU may be discarded without a retransmission being requested by the SR based on t-Reassembly. For these reasons, the following additional changes can be considered.

R2-2503620, Running RLC CR for XR [2] 5.2.3.2.x Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if t-Reassembly is running; - if RX_Next_Status_Trigger = RX_Next; or - if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or - if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size: - stop and reset t-Reassembly. - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest. - if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above); - start t-Reassembly; - set RX_Next_Status_Trigger to RX_Next_Highest.

Proposal 2:	An RLC entity can stop t-Reassembly which is still running for the discarded RLC SDU when t-RxDiscard expires.

Observation 2:	When an SN gap is detected, if an RLC entity starts t-RxDiscard without starting t-Reassembly, there is a possibility that the RLC SDU may be discarded without a retransmission being requested by the SR based on t-Reassembly.

Proposal 3:	If there is still an SN gap even after t-RxDiscard has expired and RX_Next has been updated, an RLC entity can start t-Reassembly as well as t-RxDiscard.

- Conclusion

In this contribution, we provided the following observations and proposals.

Observation 1:	RX_Next may be updated to a value larger than RX_Highest_Status after the expiry of t-RxDiscard. In such a scenario, it would be beneficial to set ACK_SN to the same value as updated RX_Next in the STATUS PDU in order to provide positive acknowledgements of more RLC SDUs.

Proposal 1:	(RLC-2) When the t-RxDiscard expires, the SR should be triggered after RX_Highest_Status is updated to the latest value (e.g., the same value as the updated RX_Next).

Proposal 2:	An RLC entity can stop t-Reassembly which is still running for the discarded RLC SDU when t-RxDiscard expires.

Observation 2:	When an SN gap is detected, if an RLC entity starts t-RxDiscard without starting t-Reassembly, there is a possibility that the RLC SDU may be discarded without a retransmission being requested by the SR based on t-Reassembly.

Proposal 3:	If there is still an SN gap even after t-RxDiscard has expired and RX_Next has been updated, an RLC entity can start t-Reassembly as well as t-RxDiscard.

- References

[1]	RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3

[2]	R2-2503620, RLC running CR for XR, vivo