3GPP TSG-RAN WG2 Meeting #130		R2-2504523

St.Julians, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on remaining issues on paging

Agenda Item:		8.13.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #129bis meeting, multi-hop relay related paging has been discussed, and the following agreements were reached[1]:

Agreements:

Support the child UE relying on the parent relay UE for paging monitoring.

Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, while avoiding duplicated paging delivery to the remote UE due to double-monitoring by upstream UEs.

In this contribution, we would like to further discuss the remaining issues of paging, trying to identify the spec impact of above agreement, and take the post email discussion in SIB (request) forwarding in multi-hop as the reference.

- Discussion

In RAN2 #129bis meeting, it is agreed that ‘Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface’. However, it is still not clear how to avoid the duplicated paging delivery due to double-monitoring by upstream UEs and immunize the spec impact. The question would be, e.g., whether the intermediate relay UE should further forward the paging information of the Remote UE to the last Relay UE if it can already monitor paging for a child UE on Uu interface?

The similar discussion was ongoing for the SIB message in email discussion ‘[Post129bis][412]’, as in FFS-issue 4 [2]. For SIB, the following proposal was given by rapporteur after the discussion:

- For the SIB requested by child node, if the SIB has not been included in SIB request in parent link, if IDLE/INACTIVE intermediate relay UE obtain the requested SIB via Uu interface [8/11] or unsolicited SIB1 forwarding [6/8] and CAN respond SIB request of child UE, adopt legacy condition (“if the UE has SIB request information to provide”) to determine SIB request in parent link is needed or not.

In our understanding, we could mimic the discussion in SIB, therefore, to minimize the spec impact, we can also use the legacy condition to determine whether the paging request in parent link is needed or not.

Observation 1: In post email discussion about SIB request on parent link, the legacy condition is preferred by companies to adopt to determine SIB request in parent link is needed or not.

Proposal 1: To minimize the spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, adopt legacy condition (‘if the UE has paging related information to provide’) to determine paging request in parent link is needed or not.

Similarly, in RAN2 #129 meeting, there is an left FFS point:

What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)?

FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay).

There is also similar discussion in [2], on FFS issue-5.

1.5 FFS issue-5 It’s FFS that: The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay): • upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested) • FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

And the final proposal given by rapporteur is as follows:

- Legacy condition “if the UE has SIB request information to provide” is enough [6/8], no additional specification change is needed to support “when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay” [8/10].

Similarly, for paging information, we can adopt the legacy condition as well to determine whether to initiate/cancel paging request forwarding on parent link. This is also the simplest way to minimize the spec impact on this issue.

Proposal 2: Legacy condition (“if the UE has paging related information to provide”) is enough and no additional specification change is needed to support FFS ‘upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay)’.

- Conclusion

In this contribution, we discussed potential spec impact on paging request forwarding on the link between intermediate relay UE and its parent relay UE. We have the following Observations and Proposals:

Observation 1: In post email discussion about SIB request on parent link, the legacy condition is preferred by companies to adopt to determine SIB request in parent link is needed or not.

Proposal 1: To minimize the spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, adopt legacy condition (‘if the UE has paging related information to provide’) to determine paging request in parent link is needed or not.

Proposal 2: Legacy condition (“if the UE has paging related information to provide”) is enough and no additional specification change is needed to support FFS ‘upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay)’.

- References

- RAN2 #129bis meeting chair notes;

- R2-250xxxx Report of [Post129bis][412][Relay] FFS issues on system information_v18_Rapp(ZTE);

- RAN2 #129 meeting chair notes;